SA WG2 Meeting #128	S2- 186621

2 - 6 July 2018, Vilnius, Lithuania	(revision of S2-18xxxx)

Source:	Ericsson, Samsung

Title:	Update of solution #3

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_eV2XARC / Rel-16

Abstract of the contribution: This contribution proposes some updates to remove editor’s notes.

# Discussion

For solution #3 there exists two open questions as “Editor’s Notes” one relates to external data base and one relates to UE triggered provisioning.

In a case having a database external to UDR the V2X configuration parameters will be transferred via the AF and handled by PCF.

The UE provisioning is controlled from the PCF and if any parameters are changed the update is performed by the PCF. If the UE of some reason finds a need to update the parameters, the UE have to re-register in the network and the PCF will provide the parameters using existing procedures. In a failure case were the parameters may be lost by the UE, the UE will in any case re-register in the network and the PCF will provide the parameters to the UE using the existing procedures. These re-registers triggered by the UE will be very rare.

# Proposal

It is proposed to add a CP based V2X reference architecture model in TR23.786.

***** First Change *****

## 6.3	Solution #3: Service Authorization and Provisioning to UE for eV2X communications over PC5 and NG-Uu reference points

### 6.3.1	Functional Description

This solution addresses Key Issue #5 and Key Issue #11.

This solution applies to the architecture option Alternative#1 in Annex A.

In 5GS, the service authorization parameters for V2X PC5 and V2X NG-Uu communication can be made available in the UE in different ways, i.e.

-	pre-configured in the ME, or

-	configured in the UICC, or

-	preconfigured in the ME and configured in the UICC, or

-	provided/updated by the V2X AS via PCF, or

-	provided/updated by the PCF to the UE

When the service authorization parameters for V2X PC5 and NG-Uu communication are provided by the PCF, the existing procedures will be used:

-	When the UE performs initial Registration as defined in clause 4.2.2 "Registration Management procedures" in TS 23.502 [9], the PCF provisions the eV2X PC5 and eV2X NG-Uu service authorization parameters to the UE using procedures as defined clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy delivery" in TS 23.502 [9] and clause 6.1.2 "Non-session management related policy control" in TS 23.503 [10].

-	During mobility, e.g. UE moves from one PLMN or another, the UE performs Registration and the PCF may update the UE with new service authorization parameters for the new PLMN.

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform V2X communication service over PC5 and/or NG-Uu reference point, the PCF may need to update the policy/parameters to the UE. If the serving PLMN is removed from the list when the UE is authorized to perform V2X communication service over PC5 and/or NG-Uu, it means the service authorization is revoked in the UE.

-	When the user is roaming, the subscription change resulting in updates of V2X policy/parameters will be transferred by H-PCF via V-PCF.

-	If the V2X configuration parameters is located in a database external to the UDR the V2X configuration parameters will be transferred to the PCF via the AF

NOTE 1:	Update of provisioning information directly to the UE (i.e. V1 reference point) is available in this option as well.

NOTE 2:	The UE provisioning is controlled by the PCF and no UE triggered provisioning update is needed. If the UE, due to a failure case or of some other reason, see a need to update the provisioning, the UE have to re-register in the network. This re-registration is foreseen as very rare.

### 6.3.2	Procedures

Procedure for UE policy update can be reused:

-	Clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy delivery" in TS 23.502 [9].

-	Clause 6.1.2 "Non-session management related policy control" in TS 23.503 [10].

### 6.3.3	Impact on existing entities and interfaces

Impacts on existing entities, interfaces and procedures are limited to adding new policies/parameters, no new functionality is introduced. Below are the impacted entities and interfaces listed:

-	UDR for subscription change in the list of PLMNs where the UE is authorized to perform V2X communication service over PC5 reference point

-	PCF handles service authorization parameters for V2X PC5 and V2X NG-Uu communication.

-	UE handles the policy/parameters for service authorization using existing 5GC procedures.

-	AMF transferring the policy/parameters to the UE using existing procedures.

-	Impacted interfaces: N1, N8, N15, N24.

To perform the UE provisioning the user plane set-up is not needed.

### 6.3.4	Topics for further study

None is identified.

### 6.3.5	Conclusions

TBD

***** End of Changes *****