SA WG2 Meeting #128	S2-186639

July 2 – 6 2018, Vilnius, Lithuania

Source:	AT&T

Title:	An addition of a new Key Issue on Network Status

Document for:	Decision

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes an addition of a new Key issue

Description

A Release 15 NEF currently supports a subset of SCEF Northbouund APIs. One of the APIs currently not supported by NEF is Reporting of Network Status. TS 29.122 describes the functionality of this API as follows

“The ReportingNetworkStatus API is a RESTful API that allows the SCS/AS to be one-time or continuous notified of the network status in a geographic area.”

The SCEF uses RCAF for retrieving this information on the southbound side. This is explained as below in TS 29.122

“The SCEF uses the reporting procedures based on the network status information from one or more RCAF(s).”

While there is no RCAF in 5GC, a similar functionality was introduced in 5G in the form of NWDAF that in Release 15 can provide slice level congestion information although it may not be UE specific.  This contribution proposes to introduce this topic as a new key issue for Release 16 study so that an equivalent information exposure from 5GC can be achieved.

Note: Based on some initial discussions the topic of exposure of network status is considered out of scope for 5G CIoT study

### ************************************First Change**********************************

## 5.2.X	Key Issue X: Support of Northbound Network Status Exposure

#### 5.2.X.1	Description

A Release 15 NEF currently supports a subset of SCEF Northbouund APIs. One of the APIs currently not supported by NEF is for Reporting of Network Status. TS 29.122 describes the functionality of this API as follows

“The ReportingNetworkStatus API is a RESTful API that allows the SCS/AS to be one-time or continuous notified of the network status in a geographic area.”

The SCEF uses RCAF for retrieving this information on the southbound side. This is explained as below in TS 29.122

“The SCEF uses the reporting procedures based on the network status information from one or more RCAF(s).”

While there is no RCAF in 5GC, a similar functionality was introduced in 5G in the form of NWDAF that in Release 15 can provide slice level congestion information although in Release 15 it is not UE specific.  This Key issue looks for solutions to this problem and would study the collection of types of information by NWDAF that would allow determination of network status being experienced by a UE in its current location.

#### 5.2.x.2	Requirement

NEF should be able to subscribe to this event from NWDAF in order to expose this information to authorized 3rd parties

### ************************************End Change**********************************