3GPP TSG-SA2 Meeting #128	S2-186947

# Vilnius, Lithuania, July 2nd - 6th  2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0621 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing NEF services. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS-Ph1 |  |  |  |  |  | Date: |  |  | 2018-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 3GPP TS 23.682, the following functionalities are specified for the SCEF: - Change the chargeable party at session set-up or during the session - Support of setting up an AS session with required QoS These functionalities are missing in 3GPP TS 23.502. Furthermore, in order to apply the background data transfer policy previously negociated with Nnef_BDTPNegotiation service, the AF may contact either directly the PCF via Npcf_PolicyAuthorization_Create or via NEF as mentioned in 3GPP TS 23.502 subclause 4.16.7.1. Thus, “Change the chargeable party at session set-up or during the session “functionality is required to apply, via NEF, the background data transfer policy previously negociated with Nnef_BDTPNegotiation service.Otherwise, the Nnef_BDTPNegotiation service will not work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 2 NEF services are described : - Change the chargeable party at session set-up or during the session - Support of setting up an AS session with required QoS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background data transfer policy previously negociated with Nnef_BDTPNegotiation service cannot be applied via NEF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6, 5.2.6.x (new), 5.2.6.x.1 (new), 5.2.6.x.2 (new), 5.2.6.x.3 (new), 5.2.6.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AsSessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |

**** Second Change ****

#### 5.2.6.x	Nnef_ChargeableParty service

##### 5.2.6.x.1	General

##### 5.2.6.x.2	Nnef_ChargeableParty_Create service operation

Service operation name: Nnef_ChargeableParty Create.

Description: The consumer requests to become the chargeable party for a data session for a UE.

Inputs (required): SCS/AS Identifier, UE IP address, Description of the application flows, Sponsor Information, Sponsoring Status.

Inputs (optional): time period, traffic volume, Reference ID for a previously selected policy of a background data transfer.

Outputs (required): TLTRI, result.

Output (optional): None.

##### 5.2.6.x.3	Nnef_ChargeableParty_Update service operation

Service operation name: Nnef_ChargeableParty Update.

Description: The consumer can change the chargeable party of a data session for a UE.

Inputs (required): SCS/AS Identifier, TLTRI, Sponsoring Status.

Inputs (optional): time period, traffic volume, Reference ID for a previously selected policy of a background data transfer.

Outputs (required): TLTRI, result.

Output (optional): None.

##### 5.2.6.x.4	Nnef_ChargeableParty_Notify service operation

Service operation name: Nnef_ChargeableParty Notify.

Description: NEF reports the bearer level event(s) to the consumer.

Inputs (required): event reports.

Inputs (optional): None.

Outputs (required): None.

Output (optional): None.

**** Third Change ****

#### 5.2.6.x	Nnef_AsSessionWithQoS service

##### 5.2.6.x.1	General

##### 5.2.6.x.2	Nnef_AsSessionWithQoS_Create service operation

Service operation name: Nnef_AsSessionWithQoS Create.

Description: The consumer requests the network to provide a specific QoS for an AS session.

Inputs (required): SCS/AS Identifier, UE IP address, Description of the application flows, QoS Reference.

Inputs (optional): time period, traffic volume.

Outputs (required): TLTRI, result.

Output (optional): None.

##### 5.2.6.x.4	Nnef_AsSessionWithQoS_Notify service operation

Service operation name: Nnef_AsSessionWithQoS Notify.

Description: NEF reports the bearer level event(s) to the consumer.

Inputs (required): event reports.

Inputs (optional): None.

Outputs (required): None.

Output (optional): None.

**** End of Changes ****