# SA WG2 Meeting #128	S2-187019

July 2 - 6, 2018, Vilnius, Lithuania

Source:	Huawei, Hisilicon

Title:	UL-CL insertion when ULCL is in different region comparing to A-SMF

Document for:	Discussion/Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: This contribution discusses how the UL-CL is inserted when ULCL is in different region comparing to A-SMF.

- Discussion

The following architecture has been adopted in the TR as solution 7:

Figure 1 UL-CL/BP insertion when UL-CL/BP is controlled by Local SMF

In this architecture, the UL-CL and local A-UPF are controlled by Local SMF which is within the service area of I-SMF.

This contribution proposes procedures to support UL-CL insertion for this arhictecture.

- Proposal

It is proposed to add the following text into TR 23.726.

/*************************** Start of the first change ************************/

6.7.2	Description of the solution

6.7.2.1	General

In this solution, the UPF that acts as UL-CL/BP is not controlled by A-SMF.

The UL-CL/BP may be inserted during or after the PDU session establishment, e.g.:

-	When UE moves to a location that supports to connect a local DN via UL-CL/BP; Or,

-	When UE starts an application that has local deployment via the UL-CL/BP.

6.7.2.2	UL-CL/BP in a same region as A-SMF

6.7.2.2.1	Overview: UL-CL/BP in the same region as A-SMF

When the UE is within the same administrative domain as the PSA of the PDU Session, it is possible that some UPF are not controlled by the A-SMF, e.g. the UPF belonged to the enterprise and exclusively controlled by the SMF also belonged to the enterprise.

In that case when those UPF is to be selected, e.g. UL-CL/BP insertion, the Local SMF need be inserted. The architecture is as in Figure 6.7.2.2.1-1:

Figure 6.7.2.2.1-1: UL-CL/BP insertion when UE is in the same region as A-SMF

The local SMF is a network function that controls the UPF deployed close to the edge, e.g. the UPFs are deployed in an enterprise premise. The local SMF has the following function:

-	UE IP address management in case of multi-homing (RA can be sent to UE via local SMF or A-SMF).

-	Selection and control of UP function.

-	Configures traffic steering at UPF to route traffic to proper destination.

-	Charging data collection.

The SMF that controls the PSA of the PDU session (i.e. A-SMF) determines whether a UL-CL/BP needs to be inserted and determines the DNAI the UL-CL/BP connects to. The SMF selects the local SMF that controls the UL-CL/BP based on DNAI and UE location information.

NOTE:	it is assumed that Nxy interface is based on the N4 interface. However the difference between the Nxy and N4 interface is FFS. For example the enhancement on N4 interface to convey the charging data and IP address.

6.7.2.2.2	Procedures

6.7.2.2.2.1	Insertion of UL-CL/BP when it is in the same region as A-SMF

The procedure in figure 6.7.2.2.2.1-1 is to insert a UL-CL/BP for a PDU session when the UE is located in the same region as the SMF that controls the PSA. It is assumed that local PSA is collocated with UL-CL/BP. The solution also applies to the case where the local PSA is not collocated with UL-CL/BP.

Figure 6.7.2.2.2.1-1: Procedure to support UL-CL/BP insertion when UE is in the same region of A-SMF

1.	Before the procedure, a PDU session anchored in A-SMF has been established. The A-SMF may receive some triggers to insert a UL-CL/BP for the PDU session. The triggers the A-SMF receives includes:

- The UE moves to a new location based on the area of interest subscription (from AMF); or

- Received trigger from PCF or from PSA that a new application has been detected (from PCF or PSA).

2.	Based on the trigger, the SMF determines whether to insert UL-CL/BP. If yes, the SMF determines the DNAI that the UL-CL/BP will connect to.

If the SMF cannot find a UPF that supports the DNAI by itself, the SMF selects a Local SMF based on the DNAI and UE location information.

3.	The A-SMF sends Nxy message to the Local SMF to request it to insert UL-CL/BP. The message includes UE location information, an indication of UL-CL or Multi-homing, AN tunnel information, PSA tunnel information, PDR to be installed in UL-CL/BP, and the DNAI.

4.	The Local SMF selects UL-CL/BP based on UE location, and optionally DNAI.

5.	I-SMF sends N4 Session establishment request to the UL-CL/BP, including AN tunnel info, PSA tunnel info and PDR to be installed in UL-CL/BP.

6.	Local SMF send Nxy message Ack to A-SMF, including the local UPF tunnel info (for both N3 and N9, service area of UL-CL/BP UPF).

The Local SMF return the service area of the UL-CL/BP UPF to the A-SMF. Based on that information, the A-SMF can subscribe the notification from AMF when the UE leave this service area.

In the case of Multi-homing, the Local SMF need allocate a local IPv6 prefix for the PDU Session. The Local SMF also includes local IPv6 prefix in this message, the A-SMF notifies the UE of the availability of the new IP prefix via IPV6 Router Advertisement.

7.	A-SMF send N4 session update request to PSA, to update it with local UPF tunnel info (N9).

8.	A-SMF send SM N2 request to AN via AMF to update AN with the local UPF tunnel info (N3).

6.7.2.2.2.2	Change of UL-CL/BP in the same region as A-SMF

Before this procedure, a UL-CL/BP (collocated with local PSA) controlled by Local SMF has been inserted. The A-SMF has received information of "no DNAI change" for the applications anchored at local PSA from PCF.

When the UE moves out of the service area of UL-CL/BP UPF, an I-UPF controlled by A-SMF is inserted. The I-UPF works as UL-CL/BP which routes uplink packets to PSA or local PSA accordingly.

Comparing to the clause 4.2.2.2.2 of TS 23.502 [3], the impact of the introduction of local SMF is on the step 18. When the AMF notify the UE location change to the A-SMF, the A-SMF update the original UL-CL/BP UPF via local SMF to establish a tunnel between the new I-UPF and the original UL-CL/BP UPF.

Figure 6.7.2.2.2.2-1 Change of UL-CL/BP in the same region as A-SMF

1.	When UE location changes, the AMF notify the UE location information to A-SMF per Area of Interest subscription.

2.	The A-SMF determines that UE has moved out of the service area of the local UL-CL/BP UPF, and the local anchor needs to be kept based on information provided by AF before. The A-SMF selects an UPF as new UL-CL.

3a.	If the new selected UPF is controlled by the A-SMF, the A-SMF sends N4 session establishment to the new I-UPF, which includes the UL CN tunnel info of the PSA and Local PSA.

3b.	If the new selected UPF is not controlled by the A-SMF, the AMF need insert a local SMF. Step 2 to 6 of figure 6.7.2.4.1-1 is executed.

4~6.	The A-SMF updates the local PSA with the DL CN tunnel info of the new I-UPF via local SMF.

7.	The A-SMF updates the PSA with the DL CN tunnel info of the new I-UPF.

8.	A-SMF send SM N2 request to 5G AN via AMF to update AN with the new I-UPF tunnel info (N3), if UP is to be activated.

6.7.2.2.2.3	Release of UL-CL/BP in the same region as A-SMF

Before this procedure, a UL-CL/BP (collocated with local PSA) controlled by Local SMF has been inserted. The A-SMF has not received information from PCF "no DNAI change" for the applications anchored at local PSA.

The UE moves out the service area of UL-CL/BP UPF, and the local SMF and local UL-CL/PSA can be released.

Comparing to the clause 4.2.2.2.2 of TS 23.502 [3], the impact of the introduction of local SMF is on the step 18. When the AMF notify the UE location change to the A-SMF, the A-SMF release the original UL-CL/BP UPF via local SMF.

Figure 6.7.2.2.2.3-1 Release of UL-CL/BP in the same region as A-SMF

1.	When UE location changes the AMF notify the UE location information to A-SMF per Area of Interest subscription.

2.	The A-SMF determines that UE has moved out of the service area of the UL-CL/BP UPF and the local anchor can be released. No UL-CL/BP UPF can be selected at the UE new location.

3~5.	The A-SMF releases UL-CL and local PSA via Local SMF.

6.	The A-SMF updates the PSA with DL tunnel information of 5G AN.

7.	A-SMF send SM N2 request to 5G AN via AMF to update AN with the new I-UPF tunnel info (N3) ), if UP is to be activated.

/*************************** Second change ************************/

6.7.2.3.2	UL-CL/BP controlled by local SMF in I-SMF service area

6.7.2.3.2.1	Overview: UL-CL/BP controlled by local SMF in I-SMF service area

For this scenario, the UE is in service area of I-SMF, but the UPF that acts as UL-CL/BP is controlled by a local SMF within the service area of I-SMF. The I-SMF in this solution corresponds to the I-SMF shown in the architectures described in solutions 6.

The UL-CL/BP may be inserted during or after the PDU session establishment, e.g.:

- When UE moves to a location that supports to connect a local DN via UL-CL/BP; Or,

- When UE starts an application that has local deployment via the UL-CL/BP

Before the UL-CL/BP insertion, the I-UPF controlled by I-SMF is already inserted into the PDU Session path. After the UL-CL/BP insertion, the architecture is as in figure 6.7.2.3.2.1-1

Figure 6.7.2.3.2.1-1 UL-CL/BP is controlled by local SMF within I-SMF service area

In this architecture, the UL-CL/BP and local PSA are controlled by local SMF that is located within I-SMF service area. The I-SMF determines whether the UL-CL/BP needs to be inserted and determines the DNAI the UL-CL/BP connects to. The I-SMF selects the local SMF based on DNAI and UE location information.

NOTE: it is assumed that Nxy interface is based on the N4 interface. However the difference between the Nxy and N4 interface is FFS.

6.7.2.3.2.2	Procedures: Insertion of UL-CL/BP controlled by local SMF within I-SMF service area

Editor's note:	How local SMF retrieves the policy information, e.g. DNAI is FFS.

Figure 6.7.2.3.2.2-1 UL-CL/BP insertion when UL-CL/BP is controlled by local SMF within I-SMF service area

- Before the procedure, a PDU session anchored in A-SMF has been established. An I-UPF controlled by I-SMF has been inserted.

- The PCC rule related to AF request is provisioned to I-SMF. Two alternatives can be used to provision PCC rules to I-SMF as described in clause 6.13 or 6.15.

- Based on the received triggers, the I-SMF keep track the UE and may determine to insert a UL-CL/BP for the PDU session. The triggers includes:

- The UE moves to a new location based on the area of interest subscription (from AMF); or

- Received trigger from PCF or from I-UPF that a new application has been detected (from PCF or I-UPF).

If UL-CL/BP is to be inserted, the I-SMF determines the DNAI that the UL-CL/BP will connect to.

If the I-SMF cannot find a UPF that supports the DNAI by itself, the I-SMF selects a Local SMF based on the DNAI and UE location information.

4.	The I-SMF sends Nxy message to the Local SMF to request it to insert UL-CL/BP. The message includes UE location information, an indication of UL-CL or Multi-homing, AN tunnel information, PSA tunnel information, PDR to be installed in UL-CL/BP, and the DNAI.

5.	The Local SMF selects UL-CL/BP and a local PSA based on UE location, and optionally DNAI.

6.	Local SMF sends N4 Session establishment request to the UL-CL/BP and local PSA, including AN tunnel info, PSA tunnel info and PDR to be installed in UL-CL/BP.

7.	Local SMF sends Nxy message Ack to I-SMF, including the local UPF tunnel info (for both N3 and N9, service area of UL-CL/BP UPF).

The Local SMF return the service area of the UL-CL/BP UPF to the I-SMF. Based on that information, the I-SMF can subscribe the notification from AMF when the UE leave this service area.

In case of Multi-homing, the Local SMF need allocate a local IPv6 prefix for the PDU Session. The Local SMF also includes local IPv6 prefix in this message.

8.	The I-SMF sends Nsmf_PDUSession_Update Request to A-SMF, providing tunnel info of UL-CL. If the local IP address is allocated, the local IP address is included in this message.

9. The A-SMF sends N4 session update to PSA to update the downlink tunnel info (N9) toward the UL-CL.

10. The A-SMF sends Nsmf_PDUSession_Update Response to I-SMF.

11.	A-SMF send SM N2 request to AN via AMF to update AN with the local UPF tunnel info (N3).

12. The I-SMF sends N4 session release to I-UPF, and releases resources in I-UPF.

******************************* Next Change *********************************

******************************* End of Changes *********************************