SA WG2 Meeting #128	S2-187025

July 2nd - 6th, 2018, Vilnius, Lithuania

Source:	Huawei, Hisilicon

Title:	Evaluation of solution #6 and conclusion proposal

Document for:	Discussion/Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: This contribution proposes evaluation to solution 6: SMF selection and reselection due to different region.

- Discussion

In this contribution, we discussed the impacts of solution 6 to existing entities, and proposed an evaluation of solution 6. We also give an initial conclusion proposal for the I-SMF additional/removal issue.

- Proposal

It is proposed to add the following text into TR 23.726.

/*************************** 1st change ************************/

6.6.3	Impact of the solution to existing entities

Impacts to AMF:

- AMF supports I-SMF selection function during PDU session establishment and during mobility.

- Trigger the resource release toward source I-SMF during mobility

Impacts to SMF:

- SMF supports SM context retrieval from another SMF

- Support forwarding tunnel establishment between old I-SMF during service request and N2 handover

- Support the function to determine whether forwarding tunnel is needed during service request

- SMF registers its service area into NRF

Impacts to NRF:

- NRF is registered with SMF service area, and is able to select SMF based on SMF service area

6.6.4	Evaluation of the solution

The whole solution can be evaluated as below:

Architecture:

The architecture is similar as the architecture for home routed roaming in Rel-15. So the procedures can reuse existing home routed roaming procedures with minimize change.

SMF selection:

Both I-SMF selection during PDU session establishment and during UE mobility has been considered. The AMF take the role of the I-SMF selection. When and based on which parameter for the I-SMF selection are also considered.

SM context retrieval:

The new I-SMF retrieves SM context from old I-SMF directly. Comparing to Rel-15 only a small enhancement to the existing NF is needed. And the SM context retrieval is always between SMFs, the AMF does not need to be enhanced for SM context retrieval. This is align with the SM and MM split principle in 5GC.

Forwarding tunnel establishment:

The forwarding tunnel is established between target I-SMF and source I-SMF directly. The benefits is that:

- AMF is not involved in this SM related handling, hence, aligned with SM/MM split principle

- Shorter latency, this solution reduces latency caused by AMF involvement

******************************* End of Changes *********************************