3GPP TSG-SA2 Meeting #128	S2-187060

# Vilnius, Lithuania, July 2nd - 6th  2018 										(revision of S2-186947)

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0621 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing NEF services. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS-Ph1 |  |  |  |  |  | Date: |  |  | 2018-07-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 3GPP TS 23.682, the following functionalities are specified for the SCEF: - Change the chargeable party at session set-up or during the session - Support of setting up an AS session with required QoS These functionalities are missing in 3GPP TS 23.502. Furthermore, in order to apply the background data transfer policy previously negociated with Nnef_BDTPNegotiation service, the AF may contact either directly the PCF via Npcf_PolicyAuthorization_Create or via NEF as mentioned in 3GPP TS 23.502 subclause 4.16.7.1. Thus, “Change the chargeable party at session set-up or during the session “functionality is required to apply, via NEF, the background data transfer policy previously negociated with Nnef_BDTPNegotiation service.Otherwise, the Nnef_BDTPNegotiation service will not work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 2 NEF services are described with their corresponding procedures: - Change the chargeable party at session set-up or during the session - Support of setting up an AS session with required QoS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background data transfer policy previously negociated with Nnef_BDTPNegotiation service cannot be applied via NEF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.x (new), 4.15.6.x+1 (new), 4.15.6.x+2, 5.2.6.1, 5.2.6.y (new), 5.2.6.y.1 (new), 5.2.6.y.2 (new), 5.2.6.y.3 (new), 5.2.6.y.4 (new), 5.2.6.y+1 (new), 5.2.6.y+1.1 (new), 5.2.6.y+1.2 (new), 5.2.6.y+1.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 4.15.6.x	Set a chargeable party at AF session setup

Figure 4.15.6.x-1: Set the chargeable party at AF session set-up

1.   When setting up the connection between the AF and UE, the AF may request to become the chargeable party for the session to be set up by sending a Nnef_ChargeableParty_Create request message (AF Identifier, UE IP address, Description of the application flows, Sponsor Information, Sponsoring Status, Reference ID) to the NEF, The Sponsoring Status indicates whether sponsoring is started or stopped, i.e. whether the 3rd party service provider is the chargeable party or not. The Reference ID parameter identifies a previously negotiated transfer policy for background data transfer as defined in clause. 4.16.7. The NEF asssigns a Transaction Reference ID  to the Nnef_ChargeableParty_Create request.

2.   The NEF authorizes the AF request to sponsor the application traffic and stores the sponsor information together with the AF Identifier and the Transaction Reference ID. If the authorisation is not granted, step 2 is skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

NOTE:   Based on operator configuration, the NEF may skip this step. In this case the authorization is performed by the PCF in step 3.

3.   The NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update request message and provides IP filter information, sponsored data connectivity information (as defined in TS 23.203 [27]), Reference ID (if received from the AF) and Sponsoring Status (if received from the AF) to the PCF.

4. The PCF determines whether the request is allowed and notifies the NEF if the request is not authorized. If the request is not authorized, NEF responds to the AF in step 5 with a Result value indicating that the authorization failed.

5.   The NEF sends a Nnef_ChargeableParty_Create response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

#### 4.15.6.x+1	Change the chargeable party during the session

Figure 4.15.6.x+1-1: Change the chargeable party during the session

1.   For the ongoing AF session, the AF may send a Nnef_ChargeableParty_Update request message (AF Identifier, Transaction Reference ID, Sponsoring Status, Reference ID) to the NEF. The Sponsoring Status indicates whether sponsoring is enabled or disabled, i.e. whether the 3rd party service provider is the chargeable party or not. The Reference ID parameter identifies a previously negotiated transfer policy for background data transfer as defined in clause 4.16.7. The Transaction Reference ID provided in the Change chargeable party request message is set to the Transaction Reference ID that was assigned, by the NEF, to the a Nnef_ChargeableParty_Create request.

2.   The NEF authorizes the AF request of changing the chargeable party. If the authorisation is not granted, step 3 is skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

NOTE:   Based on operator configuration, the NEF may skip this step. In this case the authorization is performed by the PCF in step 3.

3.   The NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update request and provides IP filter information, sponsored data connectivity information (as defined in TS 23.203 [27]), Reference ID (if received from the AF) and Sponsoring Status (if received from the AF) to the PCF.

4. The PCF determines whether the request is allowed and notifies the NEF if the request is not authorized. If the request is not authorized, NEF responds to the AF in step 5 with a Result value indicating that the authorization failed.

5.   The NEF sends a Nnef_ChargeableParty_Update response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

#### 4.15.6.x+2	Setting up an AF session with required QoS procedure

Figure 4.15.6.x+2-1: Setting up an AF session with required QoS procedure

1.   When setting up the connection between AF and the UE with required QoS for the service, the AF sends an Nnef_AFsessionWithQoS_Create request  message (UE IP address, AF Identifier, Description of the application flows, QoS reference) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The NEF assigns a Transaction Reference ID to the Nnef_AFsessionWithQoS_Create request.

2.   The NEF authorizes the AF request and may apply policies to control the overall amount of pre-defined QoS authorized for the AF. If the authorisation is not granted, steps 3 and 4 are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

3.   The NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update request and provides IP filter information, sponsored data connectivity information (as defined in TS 23.203 [27]), Reference ID (if received from the AF) and Sponsoring Status (if received from the AF) to the PCF.

The PCF derives the required QoS based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration for this AF), and notifies the result to the NEF.

The PCF notifies the NEF whether the transmission resources corresponding to the QoS request are established or not.

4. The PCF determines whether the request is allowed and notifies the NEF if the request is not authorized. If the request is not authorized, NEF responds to the AF in step 5 with a Result value indicating that the authorization failed.

5.   The NEF sends a Nnef_AFsessionWithQoS_Update response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

**** Second Change ****

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |

**** Third Change ****

#### 5.2.6.y	Nnef_ChargeableParty service

##### 5.2.6.y.1	General

##### See clauses 4.15.6.x and 4.15.6.x+1.

##### 5.2.6.y.2	Nnef_ChargeableParty_Create service operation

Service operation name: Nnef_ChargeableParty Create.

Description: The consumer requests to become the chargeable party for a data session for a UE.

Inputs (required): AF Identifier, UE IP address, Description of the application flows, Sponsor Information, Sponsoring Status.

Inputs (optional): time period, traffic volume, Reference ID for a previously selected policy of a background data transfer.

Outputs (required): Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.y.3	Nnef_ChargeableParty_Update service operation

Service operation name: Nnef_ChargeableParty Update.

Description: The consumer can change the chargeable party of a data session for a UE.

Inputs (required): AF Identifier, Transaction Reference ID, Sponsoring Status.

Inputs (optional): time period, traffic volume, Reference ID for a previously selected policy of a background data transfer.

Outputs (required): Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.y.4	Nnef_ChargeableParty_Notify service operation

Service operation name: Nnef_ChargeableParty Notify.

Description: NEF reports the bearer level event(s) to the consumer.

Inputs (required): event reports.

Inputs (optional): None.

Outputs (required): None.

Output (optional): None.

#### 5.2.6.y+1	Nnef_AFsessionWithQoS service

##### 5.2.6.y+1.1	General

See clause 4.15.6.x+2.

##### 5.2.6.y+1.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create.

Description: The consumer requests the network to provide a specific QoS for an AS session.

Inputs (required): AF Identifier, UE IP address, Description of the application flows, QoS Reference.

Inputs (optional): time period, traffic volume.

Outputs (required): Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.y+1.3	Nnef_AFsessionWithQoS_Notify service operation

Service operation name: Nnef_AFsessionWithQoS Notify.

Description: NEF reports the bearer level event(s) to the consumer.

Inputs (required): event reports.

Inputs (optional): None.

Outputs (required): None.

Output (optional): None.

**** End of Changes ****