# Generic Network Slice Template

Table of Contents

1	Introduction	4

1.1	Overview	4

1.2	Scope	4

1.3	Definitions	4

1.4	Abbreviations	4

1.5	References	6

1.6	Conventions	7

2	Generic Network Slice Template	8

2.1	Attribute Tagging	8

2.2	Attributes	10

2.2.1	Availability	10

2.2.2	Coverage	10

2.2.3	Delay tolerance	13

2.2.4	Deterministic communication	13

2.2.5	Downlink throughput per slice	15

2.2.6	Downlink throughput per user	16

2.2.7	Energy Efficiency	18

2.2.8	Group communication support	18

2.2.9	Isolation	19

2.2.10	Location based message delivery	19

2.2.11	Maximum supported packet size	20

2.2.12	Mission-critical capability support	21

2.2.13	Mission-critical service support	21

2.2.14	MMTel Support	22

2.2.15	Multicast	23

2.2.16	NSC Network Functions	23

2.2.17	Number of connections	24

2.2.18	Number of terminals	24

2.2.19	Performance monitoring	25

2.2.20	Performance prediction	26

2.2.21	Positioning support	28

2.2.22	Private Operation	31

2.2.23	Radio spectrum	31

2.2.24	Reliability	35

2.2.25	Root cause investigation	35

2.2.26	Session and service continuity support	36

2.2.27	Slice priority	37

2.2.28	Slice quality of service parameters	37

2.2.29	Support for non-IP traffic	44

2.2.30	Supported access technologies	45

2.2.31	Supported device velocity	46

2.2.32	Synchronicity	47

2.2.33	Terminal density	48

2.2.34	Uplink throughput per slice	49

2.2.35	Uplink throughput per user	50

2.2.36	User management openness	51

2.2.37	User data access	52

2.2.38	V2X support	53

# Introduction

## Overview

This document aims to provide a standardised view on how a Generic Network Slice (GST) can be used.

## Definitions

| Term | Description |
| --- | --- |
| UE/Terminal |  |
| N6 Interface |  |
| Network Slice | A logical network that provides specific network capabilities and network characteristics.[1] |
| Network Slice Instance | A set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which form a deployed Network Slice. [1] |
| Service Continuity | The uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change [1] |

## Abbreviations

| Term | Description |
| --- | --- |
| 5GS | 5G System |
| API | Application programming interface |
| APN | Access Point Name |
| CN | Core Network |
| CPE | Customer premises equipment |
| CUPS | Control and User Plane Separation |
| DL | Down Link |
| DNN | Data Network Name |
| DSL | Digital Subscriber Line |
| E2E | End to End |
| EPS | Evolved Packet System (Core) |
| E-UTRAN | Evolved Universal Terrestrial Radio Access Network |
| GBR | Guaranteed Bit Rate |
| GERAN | GSM/Edge Radio Access Network |
| GNSS | Global Navigation Satellite Systems |
| GPS | Global Positioning System |
| GRE | Generic Routing Encapsulation |
| GST | Generic Slice Template |
| HARQ | Hybrid automatic repeat request |
| IMS | IP Multimedia System |
| IOPS | Isolated E-UTRAN Operation for Public Safety |
| IoT | Internet of Things |
| KPI | Key Performance Indicators |
| KQI | Key Quality Indicators |
| L2TP | Layer 2 Tunnelling Protocol |
| LTE-M | Long Term Evolution for Machines |
| MCD | Mission-Critical Data |
| MCI | Mission-Critical Interworking |
| MCPTT | Mission-Critical Push-To-Talk |
| MCVideo | Mission-Critical Video |
| MEC | Multi-access Edge Computing |
| MIoT | Massive IoT |
| MTU | Maximum Transmission Unit |
| NB-IoT | Narrowband IoT |
| NEF | Network Exposure Function |
| NEST | NEtwork Slice Type |
| NR | New Radio |
| NSC | Network Slice Customer |
| NSP | Network Slice Provider |
| QoS | Quality of Service |
| PDB | Packet Delay budget |
| PER | Packet Error Rate |
| PDU | Protocol Data Unit |
| RAN | Radio Access Network |
| SDO | Standards developing organizations |
| SLA | Service Level Agreement |
| SMF | Session Management Function |
| SST | Slice/Service Type |
| TN | Transmission Network |
| TSN | Time-Sensitive Networking |
| UDR | User Data Register |
| UL | Up Link |
| UPF | User Plan Function |
| URLLC | Ultra-Reliable Low Latency Communication |
| UTRAN | Universal Terrestrial Radio Access Network |
| VPN | Virtual Private Network |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | 3GPP TS 23.501 | System Architecture for the 5G System; Stage 2 (Rel-15) |
|  | RFC2119 | Key words for use in RFCs to Indicate Requirement Levels |
|  | 3GPP TR 22.804 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |
|  | 3GPP TS 38.101 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone |
|  | 3GPP TS 36.104 | Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception |
|  | 3GPP TR 22.904 | Study on user centric identifiers and authentication (release 16), V16.1.0 |
|  | IEEE 1588 | Precision Time Protocol |
|  | IEEE 802.1 | TSN |
|  | ETSI GS MEC 016 | Mobile Edge Computing (MEC); UE application interface |
|  | 3GPP TS 23.214 | Architecture enhancements for control and user plane separation of EPC nodes |
|  | 3GPP TS 29.244 | Interface between the Control Plane and the User Plane nodes |
|  | 3GPP TS 29.561 | 5G System; Interworking between 5G Network and external Data Networks; Stage 3 |
|  | 3GPP TS 23.032 | Universal Geographical Area Description (GAD) |
|  | 3GPP TS 33.501 | Security architecture and procedures for 5G System |
|  | 3GPP TS 23.379 | Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2 |
|  | 3GPP TS 23.282 | Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2 |
|  | 3GPP TS 23.281 | Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2 |
|  | 3GPP TS 23.401 | General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access |
|  | 3GPP TS 33.401 | 3GPP System Architecture Evolution (SAE); Security architecture |
|  | 3GPP TS 23.283 | Mission Critical Communication Interworking with Land Mobile Radio Systems |
|  | ETSI EN 302 636-4-1 | Intelligent Transport Systems (ITS); Vehicular Communications; GeoNetworking; Part 4: Geographical addressing and forwarding for point-to-point and point-to-multipoint communications; Sub-part 1: Media-Independent Functionality |
|  | GSMA IR.92 | IMS Profile for Voice and SMS |
|  | 3GPP TS 38.101-1 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone |

# Generic Network Slice Template

The Generic Slice Template (GST) is a set of potential slice attributes that can characterise a network slice. GST is generic and is not tied to any specific network deployment. The NEtwork Slice Type (NEST) is a GST filled with values and is essential for network slice providers to instantiate equivalent slices.

- GST, NEST and Network slice management lifecycle

Each attribute has a defined presence in the GST:

Mandatory – the attribute’s value must be present (e.g. Binary attributes)

Conditional – the attribute is mandatory if a certain condition exists

Optional – the attribute doesn’t need to be present

## Attribute Tagging

The attributes present in the GST can be mapped into multiple groups.

General Attributes:

In general, attributes can be tagged into character attributes and scalability attributes.

Character attributes - characterise a slice (e.g. throughput, latency, APIs, etc.) and are independent of the Network Slice Customer (NSC) and the Network Slice Provider (NSP).

Scalability attributes - provide information about the scalability of the slice (e.g. coverage, number of terminals, etc.) and are specific for the NSC and the NSP.

An attribute can be either a character or a scalability attribute and not both.

Character Attributes:

Character attributes can be further tagged according to what is provided by them into:

Performance related - specify the KPIs supported by a slice (e.g. throughput, latencies, etc.). Performance related attributes are relevant before the slice is instantiated.

Function related – specify functionality provided by the slice (e.g. positioning, prediction, etc.). Function related attributes are relevant before the slice is instantiated.

Operation related - specify which methods are provided to the NSC in order to operate the slice. Operation related attributes are relevant after the slice is instantiated. Furthermore, operation related attributes comprise control and management aspects.

NSC Equipment Impact

The attributes can be tagged in order to highlight if they have an impact on the NSC equipment. The NSC equipment refers to network devices and terminals.

Attributes with impact on the NSC equipment - require a detailed specification of the attribute in the GST as this attribute needs support from the NSC equipment. Examples are:

Authentication algorithm - NSC equipment needs to support the algorithm

Positioning support – for some mechanism support by the terminal might be required, e.g. GPS

NSC user plane termination – tunnelling protocol needs to be supported by the network devices

Attributes without an impact on the NSC equipment - high-level specification of the attribute is sufficient as it does not need support from the NSC equipment. Accounts for almost all of the attributes which provide APIs (e.g. performance monitoring, performance prediction)

Network Attributes:

Attributes can also be tagged according to the sub-network they might impact:

Radio Access Network (RAN) impact

Core network impact

Transport network impact

Interaction Attributes:

The way the attributes interact with the NSC can be used for tagging:

API – these attributes provide an API to the NSC in order to get access to slice capabilities. Many of the functional- and operational related attributes provide APIs to the NSC.

KPI – these attributes do provide certain performance capabilities (e.g. throughput and delay).

## Attributes

### Availability

This parameter specifies the availability provided by an instance of this slice, expressed as a percentage.  This contributes to, and may be different from overall service availability.

| Parameters |  |
| --- | --- |
| Value | Float |
| Measurement unit | Percentage |
| - Example | 99.9% 99.999% 0% -- no special requirement |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional Information

This is the availability of the NSI.  Each NEST may have its own definition of how to measure availability (e.g. driven by SLA).  For example, availability may be defined from end-customer experience point of view, or from maintenance point of view, etc.

### Coverage

This attribute specifies the coverage area of the slice - the area where the terminals can access this particular slice.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 1: Global 2: National 3: Regional 4: Local (outdoor) 5: Local (indoor) |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional Information

There are different proposals on how to describe the coverage area of the slice:

Based on base station location and coverage

Location and coverage of the BSs, known by the NSP, is used. Coverage is the described by listing the base stations and/or sectors via which the slice is provided. Based on the example provided in the following figure the list looks: {13, 17, 18, 23, 27, 28, and 33}.

: Coverage based on base station locations and coverage

The advantages of this approach are:

It is easier for the NSP to describe the coverage as it is based on real deployments.

The disadvantages are:

Requires processing in order to determine the coverage regions of the base stations

Deployment information is provided to the NSC

Deployment changes require changes in the coverage descriptions

Generic based on geographical partitioning

This approach requires partitioning a geographical region into a set of zones/grids, which for better resource usage consists of defining a regular set of zones of predetermined dimensions. Coverage is then described by the zone numbers in which the slice should be available. For the example shown in the following figure the list is: {9, 10, 15, and 16}.

: Coverage based on geographical partitioning

It is up to the NSP to map the required coverage with BSs based on their location and coverage.

The advantages of this approach are:

Hides deployment information from the NSC

Independent of the network deployment, e.g. independent of deployment changes

The disadvantages are:

The coverage may not be provided in the desired areas

### Delay tolerance

- Provide the NSC with service delivery flexibility especially for the vertical services that are not chasing for high system performance. For instance, service will be delivered once mobile system has sufficient resources or during the off-peak hours. For this type of traffic, it is not too critical how long it takes to deliver the amount of data, e.g. within hours, days, weeks, etc.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

In principle, relevant mechanisms are available. For instance, this type of traffic could be scheduled for transmission in dedicated times of the day when the traffic load is low or this traffic could get an own traffic class which is de-prioritized over all other traffic.

Certain traffic flows should reach the end user within certain latency boundary. At the same time, there are use cases that are less sensitive to delay variations giving the mobile system some level of flexibility in scheduling traffic. For instance, in automotive industry, (non-critical) software/firmware update could be deprioritised and delivered when traffic is low such as during off-peak hours.

### Deterministic communication

This attribute defines if the slice supports deterministic communication for periodic user traffic. Periodic traffic refers to the type of traffic with periodic transmissions.

#### Availability

This parameter describes if the slice supports deterministic communication.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Parameter Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Periodicity

This parameter provides a list of periodicities supported by the slice.

| Parameters |  |
| --- | --- |
| Value | {Float, Float, Float…} |
| Measurement unit | Seconds |
| - Example | Motion control /printing machine): 2*103 seconds Motion control (machine tool): 500*106 seconds |
| - Tags |  |

| Parameter Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: This parameter is present when the “Availability” is set to 1.

#### Additional information

Periodically traffic means that a transmission interval (in which a single packet is transmitted) is repeated. For example, a transmission occurs every 15 ms. Reasons for a periodical transmission can be the periodic update of a position or the repeated monitoring of a characteristic parameter. Note that a transmission of a temperature every 15 minutes is a periodical transmission. However, most periodic intervals in communication for automation are rather short. The transmission is started once and continuous unless a stop command is provided [3].

Determinism refers to whether the delay between transmission of a message and receipt of the message at the destination address is stable (within bounds). Usually, communication is called deterministic if it is bounded by a given threshold for the latency/transmission time [3].

It should be noted that a slice could support multiple periodicities. It is then a matter of resource scheduling and the identification of the different packets in order to treat them in a way to not disturb their periodicity.

This attribute should not be mixed up with periodic communication demand in which periodically a specific amount of date (not only a single packet) needs to be transmitted. Hence, there is no periodicity between the single data packets but there is a periodicity between activities of a customer or device.

(R)AN/TN/CN may use this attribute to optimize the scheduling and performance.

### Downlink throughput per slice

The overall achievable data rate of the slice in downlink that is available ubiquitously across the coverage area of the slice.

#### Minimum Downlink Throughput

This attribute describes the minimum data rate supported by the slice in downlink.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| - Example | TBC |
| - Tags |  |

| Parameter Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Maximum Downlink Throughput

This attribute defines the maximum data rate supported by the slice in downlink.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| - Example | TBC |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: Either Downlink Throughput per slice or Downlink Throughput per user shall be present.

#### Additional information

This attribute might be used to set throughput guarantees per slice (and not per user).

Minimum throughput can be defined in order to guarantee a minimum performance required to achieve a sufficient quality experience (dependent on the selected service type).

Maximum throughput can be used to offer different slice contract qualities level, e.g. gold, silver and bronze which have different maximum throughput values.

### Downlink throughput per user

#### Minimum Downlink Throughput

This attribute describes the minimum data rate supported by the slice per UE in downlink, which is required to achieve a sufficient quality experience (dependent on the selected service type) and can be seen as a guaranteed throughput. If the value is 0, best effort traffic is expected where no minimum throughput is guaranteed

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| - Example | 180 Kbps for VoIP traffic 40.000-75.000 Kbps for entry level VR 25.000 Kbps for cloud gaming |
| - Tags | - Character attribute - Performance related - No impact on NSC equipment - KPI |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Maximum Downlink Throughput

This attribute describes the maximum data rate supported by the slice per UE in downlink. These parameters could be used in order to offer different contract qualities like gold, silver and bronze.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| - Example | Bronze customer: 50.000 Kbps Silver customer: 400.000 Kbps Gold customer: 1.000.000 Kbps latency. |
| - Tags | - Character attribute - Performance related - No impact on NSC equipment - KPI |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: Either Downlink Throughput per slice or Downlink Throughput per user must be present.

#### Additional information

This attribute might be used to set different guarantees in terms of throughput experienced by the customer in downlink.

Minimum throughput can be defined in order to guarantee a minimum performance required to achieve a sufficient quality experience (dependent on the selected service type).

Maximum throughput can be used to offer different contract qualities level, e.g. gold, silver and bronze which have different maximum throughput values.

Orchestrator may use this attribute to orchestrate the resources and (R)AN/CN may use this attribute to optimize the scheduling.

### Energy Efficiency

This attribute describes the energy efficiency of the network slice, i.e. the ratio between the performance indicator, in terms of data volume (DV), and the energy consumption (EC) when assessed during the same time frame.

- The energy efficiency is evaluated only when the network is running

This attribute describes if the slice supports energy efficient communication for battery powered devices with high battery lifetime requirements.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit |  |
| - Example | 0: not supported 1: Power Saving Mode (PSM) 2: Extended Discontinuous Reception (DRX) |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

### Group communication support

- This parameter describes which type of group communication is provided by the slice.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 0: not available 1: SC-PTM 2: Broadcast/Multicast 3: Broadcast/Multicast + SC-PTM |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional information

IMS MMTel is supported in EPS and need to be supported in 5GS.

TBD

### Isolation

| Parameters |  |
| --- | --- |
| Value |  |
| Measurement unit |  |
| - Example |  |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional |  |

#### Additional Information

### Location based message delivery

- This attribute describes the delivery of information in a particular geographic region.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional Information

This attribute describes the location-based delivery of information, e.g. GeoNetworking [21] .

This is not related to the geographical spread of the slice itself. This attribute can be used to distribute information, e.g. signalling messages, to terminals within a specific geographical area.

### Maximum supported packet size

This attribute describes the maximum packet size supported by the slice and may be important for URLLC and MIoT case, or to indicate a supported maximum transmission unit (MTU).

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | Bytes |
| - Example | eMBB 1500 Bytes IoT 40 Bytes URLLC 160 Bytes for 5 ms latency. |
| - Tags | - Character attribute - Performance related - No impact on NSC equipment - KPI |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

Additional Information

This attribute might serve different purposes:

Limitation of the packet size to achieve the required latencies for instance in Industry 4.0 or energy use cases

To improve the radio performance in case many very small packets are transmitted by man devices as like for MIoT use cases

To indicate the MTU provided by the slice

An UE or a server that uses this slice needs to be aware of the limitation, which could be of operational, or performance nature. For instance, the network does not guarantee SLA if packets are bigger than the specified size or too large packets could be fragmented which will increase latency.

(R)AN/CN may use this attribute to optimize scheduling and performance, especially for URLLC case and very small packets. Transport of very small packets is very inefficient is in some technologies is very inefficient. Knowledge about the maximum supported packet size might help to improve the efficiency, e.g. by scheduling resources more efficiently.

### Mission-critical capability support

This parameter specifies what capabilities are available to support mission-critical services.  More than one capability may be supported at once.  This parameter is optional when “Slice Priority” is set to 1 (meaning mission-critical slice).

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer, …} |
| Measurement unit | NA |
| - Example | 1: Inter-user prioritization 2: Pre-emption 3: Local control |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: This attribute must be present when “Slice Priority” is set to 1.

#### Additional information

Inter-user prioritization capability provides admission and scheduling priorities for PS users over non-PS users, and different priorities among PS users.

Pre-emption capability allows non-PS users to be pre-empted by PS users, and a PS user to be pre-empted by another PS user.

Local control capability allows dynamic and temporary assignment of inter-user prioritization and pre-emption levels to local PS users (e.g. local to an incident).

### Mission-critical service support

This attributed specifies whether or not the slice supports mission-critical push-to-talk (MCPTT) [15], mission-critical data (MCData) [16], mission-critical video (MCVideo) [17], Isolated E-UTRAN Operation for Public Safety (IOPS) [18], [19] or mission-critical interworking [20].

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer, …} |
| Measurement unit | NA |
| - Example | 1: MCPTT 2: MCData 3: MCVideo 4: IOPS 5: MC interworking |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: This attribute must be present when “Slice Priority” is set to 1.

#### Additional information

IMS MMTel is supported in EPS and need to be supported in 5GS.

### MMTel Support

This attribute describes whether or not the slice supports IP Multimedia Subsystem (IMS) Multimedia Telephony Service MMTel. This parameter describes whether an IR.92 [22] compliant MMTel deployment is supported in the slice.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

IMS MMTel is supported in EPS and need to be supported in 5GS.

### Multicast

Define whether the slice supports multicast functionality.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional Information

### NSC Network Functions

A NSC can own some Network Functions (e.g. UPF, UDM). This attribute provides a list of network functions to be provided by the NSC. If the list is empty, the NSC is not expected to provide any network function relevant for the slice.

| Parameters |  |
| --- | --- |
| Value | {String, String, String, …} |
| Measurement unit | NA |
| - Example | UPF UDM/AUSF AF PCF |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional information

The slice owner who owns a unique Mobile Country Codes (MCC) and Mobile Network Codes (MNC), stores all the subscriber information in his own User Data Management (UDM). This UDM can be connected to multiple networks within the same country.

### Number of connections

- This attribute describes the maximum number of concurrent sessions supported by the slice.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 100.000 sessions 10.000.000 sessions |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: Either Number of connections or Number of terminals shall be present.

### Number of terminals

- This attribute describes the maximum number of terminals supported by the slice.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 100.000 terminals 10.000.000 terminals (sensors) |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: Either Number of connections or Number of terminals shall be present.

#### Additional information

This is an important input in order to scale the slice and provide sufficient amount of resources to the slice. It is a significant difference if the slice is used to serve 10 users or 1.000.000 users.

### Performance monitoring

- This attribute provides the capability for NSC to monitor Key Quality Indicators (KQIs) and Key Performance Indicators (KPIs). KQIs reflect the end-to-end service performance and quality while KPIs reflect the performance of the network.

#### Availability

This parameter contains a list of KQIs and KPIs available for monitoring. If the list is empty this attribute is not available in the slice and the other parameters might be ignored.

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer…} |
| Measurement unit | NA |
| - Example | Service Request Success Rate |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Monitoring sample frequency

This parameter describes how often the KQIs and KPIs are monitored.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 1: per second 2: per minute 3: per-hour 4: conditional, e.g. in case a defined threshold is crossed |
| - Tags |  |

Note:  Only the KQIs of communication services offered by the NSP can be monitored. For over the top services, the NSP is not able to access the KQIs.

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional information

Only the KQIs of communication services offered by the network operator can be monitored. For over the top services, the network operator is not able to access the KQIs

### Performance prediction

- This attribute defines the capability to allow mobile system to predict the network and service status. Predictive QoS can be done for various Key Quality Indicators (KQIs) and Key Performance Indicators (KPIs). KQIs reflect the end-to-end service performance and quality while KPIs reflect the performance of the network. The prediction is done for a specific point of time in the future and for a specific geolocation.

- Note: Only the KQIs of communication services offered by the NSP can be predicted. For over the top services, the NSP is not able to access the KQIs.

#### Availability

This parameter contains a list of KQIs and KPIs available for prediction. If the list is empty this attribute is not available in the slice and the other parameters might be ignored.

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer…} |
| Measurement unit | NA |
| - Example | 1: Throughput 2: Coverage 3: Latency 4: Service Request Success Rate |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Prediction frequency

This parameter describes how often KQIs and KPIs prediction values are provided.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 1: per second 2: per minute 3: per-hour |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional information

This attribute recently raised a lot of attention and different organisations as like 5GAA and ITU-T (Machine Learning for Future Networks including 5G (FG ML5G)) are looking at it. Towards the NSC, an API would be provided allowing the NSC to send request (e.g. KPI prediction for a certain geo-location and a certain time in the future) and receiving the prediction.

Predictive QoS is an important feature allowing NSP to inform the service in advance about a quality drop. Predictive QoS can be applied to various KPIs, e.g. coverage, throughput, latency, etc. and KQIs.

Performance prediction could be implemented in different ways:

Active prediction – network actively informs the NSC and/or the terminal proactively about the predicted values. Alternatively, the NSC and/or terminal are only informed in case the predicted KPI or KQI value crossed a defined threshold.

Passive prediction – the NSC and/or terminal requests prediction from the network via APIs provided by the network.

It should be noted that performance prediction is not a pure action between NSC (normally refer to the vertical tenant like VW, or BMW) and the mobile system, but between terminal (like the car) and the mobile system.

A prediction (request as well as reply) is always associated with a point of time in the future and a geolocation. A prediction provided by the slice to the terminal and/or customer (prediction reply) should always be associated with a confidence interval to give an idea about how reliable the prediction is. The reliability depends on many parameters, e.g. which KPI to predict, look ahead of time, etc.

### Positioning support

- This attribute describes if the slice provides geo-localization methods or supporting methods.

#### Availability

This parameter describes if this attribute is provided by the slice and contains a list of positioning methods provided by the slice. If the list is empty this attribute is not available in the slice and the other parameters might be ignored.

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer…} |
| Measurement unit | NA |
| - Example | 1: CID 2: E-CID (LTE and NR) 3: OTDOA (LTE and NR) 4: RF fingerprinting 5: AECID 6: Hybrid positioning 7: NET-RTK |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Prediction frequency

This parameter describes how often location information is provided. This parameter simply defines how often the customer is allowed to request location information. This is not related to the time it takes to determine the location, which is a characteristic of the positioning method.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 1: per second 2: per minute 3: per-hour 4: conditional, e.g. in case a specific area is entered or left |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Accuracy

This parameter describes the accuracy of the location information. Accuracy depends on the respective positioning solution applied in the slice.

| Parameters |  |
| --- | --- |
| Value | Float |
| Measurement unit | meter |
| - Example | +/- 1m +/- 0,01m |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional Information

Many use cases have a strong demand for the capability of positioning (geo-localisation) devices. Different NSC may have different requirements in terms of accuracy, energy efficiency, indoor/outdoor support, and cost, etc. For some of the use cases, positioning techniques will have to work reliably under challenging conditions, e.g. deep indoors.

It should be noted that either for some use cases, such as many IoT use cases, GPS or other Global Navigation Satellite Systems (GNSS) are not an option because of the high energy consumption or because simple devices are not equipped with the suitable receiver.

For other use cases, e.g. automotive, GNSS is a suitable positioning solution for most of the times although there are a number of situations where this is not accessible e.g. tunnels, indoors, etc. Hence, it is beneficial to combine the advantages of these systems with the positioning capabilities of 5G to provide a solution that meets the NSC scenarios.

In general, the different positioning methods can be categorized into precise positioning, cellular-based positioning and new radio (NR)-based positioning.

Cellular positioning refers to positioning methods in which the cellular network is determining the position of the terminal. In [REF] a list of cellular positioning technologies is listed:

Cell ID (CID) is the basic method which utilizes cellular system knowledge about the serving cell of a specific user; the user location area is thus associated with the serving CID.

Enhanced Cell ID (E-CID) refers to a network-based method assisted by the UE. This method utilizes CIDs, RF measurements from multiple cells, timing advance, and Angle of Arrival (AoA) measurements.

Observed Time Difference of Arrival (OTDOA) is a UE-assisted method based on reference signal time difference (RSTD) measurements conducted on downlink positioning reference signals received from multiple locations, where the user location is calculated by multi-alteration.

RF fingerprinting is a method of finding a user position by mapping RF measurements obtained from the UE onto an RF map, where the map is typically based on detailed RF predictions or site surveying results

Adaptive Enhanced Cell Identity (AECID) is a method that enhances the performance of RF fingerprinting by extending the number of radio properties that are used, where at least CIDs, timing advance, RSTD, and AoA may be used in addition to received signal strengths, and where the corresponding databases are automatically built up by collecting high-precision OTDOA and A-GNSS positions, tagged with measured radio properties.

The following table provides an overview about the characteristics of the different cellular positioning methods.

| Positioning Method | Environment Limitations | UE Impact | Site Impact | System Impact | Response Time (RAN) | Horizontal uncertainty | Vertical Uncertainty |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CID Proximity location | No | No | No | Small | Very low | High | NA |
| E-CID | No | Small | Small | Medium | Low | Medium | NA |
| - E-CID/AoA | - Rich multipath | - Small | - Large | - Medium | - Low | - Medium | - NA |
| - RF fingerprinting | - Rural | - Small | - Small | - Large | - Low/medium | - Low/medium | - Medium |
| - AECID | - No | - Small | - Small | - Medium | - Low | - Low/medium | - Medium |
| - UTDOA | - Suburban/Rural | - Small | - Large | - Large | - Medium | - <100m | - Medium |
| - OTDOA | - Rural | - Medium | - Medium | - Medium | - Medium | - <100m | - Medium |
| - A-GNSS | - Indoor | - Large | - Small | - Medium | - Medium/high | - <5m | - <20m |

: Characteristics of cellular positioning methods [REF]

### Private Operation

This attribute describes the capability to operate private 5G networks/slices in a predefined area

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional Information

### Radio spectrum

- Defines the radio spectrum supported by the slice. This is important information, as some terminals might be restricted in terms of frequencies to be used.

| Parameters |  |
| --- | --- |
| Value | {String, String, String, …} |
| Measurement unit | NA |
| - Example | n1, n77 n38 |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

This attribute simply tells which frequencies can be used to access the slice.

3GPP [4] standardised 5G NR operating bands as follows:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780MHz | N/A | SUL |

E-UTRA is designed to operate in the operating bands defined in table below [5].

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41, 66, 70, 71, 72, 73, 74, 85.

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 6 (NOTE 1) | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
| 15 | Reserved |  |  | Reserved |  |  | FDD |
| 16 | Reserved |  |  | Reserved |  |  | FDD |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | FDD |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | FDD |
| 231 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| 24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD (NOTE 2) |
| 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | FDD |
| 32 | N/A |  |  | 1452 MHz | – | 1496 MHz | FDD (NOTE 2) |
| 33 | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | TDD |
| 34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| 35 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | TDD |
| 36 | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | TDD |
| 37 | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | TDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
| 44 | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | TDD |
| 45 | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | TDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD (NOTE 3, NOTE 4) |
| 47 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | TDD |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| 49 | 3550 MHz – 3700 MHz |  |  | 3550 MHz – 3700 MHz |  |  | TDD (NOTE 8) |
| 50 | 1432 MHz | - | 1517 MHz | 1432 MHz | - | 1517 MHz | TDD |
| 51 | 1427 MHz | - | 1432 MHz | 1427 MHz | - | 1432 MHz | TDD |
| 52 | 3300 MHz | - | 3400 MHz | 3300 MHz | - | 3400 MHz | TDD |
| 65 | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz | FDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD (NOTE 5) |
| 67 | N/A |  |  | 738 MHz | – | 758 MHz | FDD (NOTE 2) |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |
| 69 | N/A |  |  | 2570 MHz | – | 2620 MHz | FDD (NOTE 2) |
| 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD6 |
| 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| 72 | 451 MHz | – | 456 MHz | 461 MHz | – | 466 MHz | FDD |
| 73 | 450 MHz | – | 455 MHz | 460 MHz | – | 465 MHz | FDD |
| 74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
| 75 | N/A |  |  | 1432 MHz | – | 1517 MHz | FDD (NOTE 2) |
| 76 | N/A |  |  | 1427 MHz | – | 1432 MHz | FDD (NOTE 2) |
| 85 | 698 MHz – 716 MHz |  |  | 728 MHz | – | 746 MHz | FDD |
| NOTE 1: Band 6, 23 are not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3. NOTE 4: Band 46 is divided into four sub-bands as in Table 5.5-1A. NOTE 5: The range 2180 – 2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 6: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 7: Void NOTE 8: This band is restricted to licensed-assisted operation using Frame Structure Type 3. |  |  |  |  |  |  |  |

### Reliability

This attribute describes in percentage the amount of sent network layer packets successfully delivered to a given system node (incl. the UE) within the time constraint required by the targeted service, divided by the total number of sent network layer packets.

The reliability is evaluated only when the network is available

| Parameters |  |
| --- | --- |
| Value | Float |
| Measurement unit | Percentage |
| - Example | Automotive (infotainment): 99% Motion control: 99,9999% |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional information

For the very high reliability requirements, it is not sure if they can be achieved with available approaches. Multi connectivity, network coding and packet duplication are examples of measures in order to improve reliability. However, now it is not clear what can be achieved.

Orchestrator may use this attribute to orchestrate the resources and network functions.

### Root cause investigation

Root cause investigation is the capability provided to NSC to understand or investigate the root cause of network service performance degradation or failure.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 0: not available 1: passive investigation 2: active investigation |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional information

This attribute could be implemented in different facets: passive investigation or activate investigation.

In passive investigation, the NSC is informed about the root cause of the network service performance degradation of failure in case there is a problem with the slice.

In active investigation, if something is wrong in the network, a NSC could perform investigation itself, for instance call for the log files of different technical domain, to understand where the problem is, then it is not just an API telling the NSC if there is a problem or not.

It should be clear that this attribute is only about the investigation of a problem. This attribute does not provide any means to solve the problem.

### Session and service continuity support

The attribute defines the continuity of a PDU session. The following three SSC modes are specified [1]:

SSC mode 1 - the network preserves the connectivity service provided to the UE (the IP address is preserved)

SSC mode 2 - the network may release the connectivity service delivered to the UE and release the corresponding PDU Session (the network may release IP address(es) that had been allocated to the UE)

SSC mode 3 - changes to the user plane can be visible to the UE, while the network ensures that the UE suffers no loss of connectivity service (the IP address is not preserved in this mode when the PDU Session Anchor changes)

None – UE loses the connectivity service

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 0: none 1: SSC mode 1 2: SSC mode 2 3: SSC mode 3 |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

### Slice priority

Mission-critical (MC) leads to a priority of the slice relative to others, for C-plane and U-plane decisions. This is relative to a customer provider relationship and to a PLMN.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: non-mission-critical 1: mission-critical |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

TBD

### Slice quality of service parameters

This attribute defines all the QoS relevant parameters supported by the slice. For some of these parameters 3GPP already defined standard values [1]. By preselecting a 5G QoS Identifier (5QI) these parameters will automatically be filled with the standardised values. A list of available 5QIs and the standardised values can be found in Additional Information 2.2.28.8.

#### 5QI Value

A 5QI is a scalar that is used as a reference to 5G QoS characteristics defined in clause  [1], i.e. access node-specific parameters that control QoS forwarding treatment for the QoS Flow (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

Standardized 5QI values have one-to-one mapping to a standardized combination of 5G QoS characteristics as [1], see Additional Information 2.2.28.8.

The 5G QoS characteristics for pre-configured 5QI values are pre-configured in the AN. The 5G QoS characteristics for QoS Flows with dynamically assigned 5QI are signalled as part of the QoS profile [1].

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer…} |
| Measurement unit | NA |
| - Example | 1,2,3…See 5QI value in the table below 0: Other (customised) |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Resource Type

The Resource Type determines if dedicated network resources related QoS Flow-level Guaranteed Flow Bit Rate (GFBR) value are permanently allocated [1].

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 0: GBR (Mission Critical Video user plane) 1: Delay critical GBR (Intelligent Transport Systems) 2: Non-GBR (Voice, AR) |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: If customised 5QI value is selected, this value needs to be provided

#### Priority Level

The Priority level associated with 5G QoS characteristics indicates a priority in scheduling resources among QoS Flows. The Priority level shall be used to differentiate between QoS Flows of the same UE, and it shall also be used to differentiate between QoS Flows from different UEs. Once all QoS requirements up to GFBR are fulfilled for all the GBR QoS Flows, the Priority Level may also be used to distribute resources between GBR QoS Flows (for rates above GFBR up to MFBR) and non-GBR QoS Flows, in an implementation specific manner. The lowest Priority level value corresponds to the highest Priority.[1].

| Parameters |  |
| --- | --- |
| Value | Float |
| Measurement unit | Seconds |
| - Example | Cooperative driving: 20*103 seconds Virtual reality: 10 – 30*103 seconds |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: If customised 5QI value is selected, this value needs to be provided

#### Packet Delay Budget

The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the UPF that terminates the N6 interface. For a certain 5QI the value of the PDB is the same in UL and DL. In the case of 3GPP access, the PDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points) [1].

If the value is set to 0, no special measures are used to bring latency down to a minimum required by low-latency use cases.

| Parameters |  |
| --- | --- |
| Value | Float |
| Measurement unit | Seconds |
| - Example | Cooperative driving: 20*103 seconds Virtual reality: 10 – 30*103 seconds |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: If customised 5QI value is selected, this value needs to be provided

#### Packet Error Rate

The Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender but that are not successfully delivered by the corresponding receiver. The purpose of the PER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For all 5QIs the value of the PER is the same in UL and DL. For GBR QoS Flows with Delay critical GBR resource type, a packet which is delayed more than PDB (but which complies with the GFBR and MDBV requirements) is counted as lost, and included in the PER. Delayed packets are not included in the PER if a GBR QoS Flow with a Delay critical resource type is exceeding the GFBR and the Maximum Data Burst Volume. [1].

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 10-6 – mission critical data 10-2 V2X messaging |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: If customised 5QI value is selected, this value needs to be provided

#### Jitter

Jitter is defined as a variation in the delay of received packets. At the sending side, packets are sent in a continuous stream with the packets spaced evenly apart. Due to network congestion, improper queuing, or configuration errors, this steady stream can become lumpy, or the delay between each packet can vary instead of remaining constant.

| Parameters |  |
| --- | --- |
| Value | Float |
| Measurement unit | Seconds |
| - Example |  |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Maximum Packet Loss Rate

The Maximum Packet Loss Rate (UL, DL) indicates the maximum rate for lost packets of the QoS flow that can be tolerated in the uplink and downlink direction.

Note: The Maximum Packet Loss Rate (UL, DL) can only be provided for a GBR QoS flow belonging to voice media.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | Percentage (%) |
| - Example |  |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional Information

3GPP already defined standardized 5QI to QoS characteristics mapping. As long as these classes are used, the 5G (or the 3GPP) system is able to support these requirements. For different combinations of QoS characteristics it needs to be checked if and how they can be supported.

For low latency there are measures discussed within the context of 5G to enable low latency communication. However, not sure what is the limit of these measures and if all requirements can be met.

Orchestrator may use this attribute to orchestrate the resources and (R)AN/CN may use this attribute to optimize the scheduling.

The parameter of this attribute can be filled separately. However, by selecting an already specified standardized 5QI the relevant parameters might be filled automatically.

3GPP [1] standardised 5G QoS values as follows:

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | NOTE 1 | 40 | 150 ms | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages Electricity distribution – medium voltage, Process automation - monitoring |
| 4 |  | 50 | 300 ms | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 65 |  | 7 | 75 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 |  | 20 | 100 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 67 |  | 15 | 100 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 75 |  | 25 | 50 ms | 10-2 | N/A | 2000 ms | V2X messages |
| 5 | Non-GBR | 10 | 100 ms | 10-6 | N/A | N/A | IMS Signalling |
| 6 | NOTE 1 | 60 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 70 | 100 ms | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  | 90 |  |  |  |  | video, etc.) |
| 69 |  | 5 | 60 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 |  | 55 | 200 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 |  | 65 | 50 ms | 10-2 | N/A | N/A | V2X messages |
| 80 |  | 68 | 10 ms | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 81 | Delay Critical GBR | 11 | 5 ms | 10-5 | 160 B | 2000 ms | Remote control (see TS 22.261 [2]) |
| 82 |  | 12 | 10 ms NOTE 5 | 10-5 | 320 B | 2000 ms | Intelligent transport systems |
| 83 |  | 13 | 20 ms | 10-5 | 640 B | 2000 ms | Intelligent Transport Systems |
| 84 |  | 19 | 10 ms | 10-4 | 255 B | 2000 ms | Discrete Automation |
| 85 |  | 22 | 10 ms | 10-4 | 1358 B NOTE 3 | 2000 ms | Discrete Automation |
| NOTE 1: a packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: it is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: This MDBV value is intended to avoid IP fragmentation on an IPv6 based, IPSec protected, and GTP tunnel to the 5G-AN node. NOTE 4: A delay of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 5: The jitter for this service is assumed to be 20 ms according to TS 22.261 [2]. |  |  |  |  |  |  |  |

: Standardized 5QI to QoS characteristics mapping

### Support for non-IP traffic

This attribute provides non-IP Session support (Ethernet session and forwarding support) of communication devices.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

The most important case is to support packet exchange in the power deferential protection:

Transmission between a mobile device (CPE) and UPF

Between adjacent UPF.

Core Network (UPF) may use Ethernet session and forwarding to transmit package as customized slice ability to fully meet the communication requirement of some vertical industries application scenarios.

There are mechanisms for DNN described in [1].

A DNN is equivalent to an APN and both identifiers have an equivalent meaning and carry the same information.

The DNN may be used e.g. to:

Select a SMF and UPF(s) for a PDU Session.

Select N6 interface(s) for a PDU Session.

Determine policies to apply to this PDU Session.

Today some scenarios of industries have requirements that protocol transport between communications devices need to support Non-IP session, like power differential protection.

End to End 5G network slicing needs to support the Ethernet session and forwarding to transmit package as customized slice ability to fully meet the communication requirement of the power differential protection.

### Supported access technologies

This attribute defines which access technologies are supported by the slice.

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer…} |
| Measurement unit | NA |
| - Example | 1: GERAN 2: UTRAN 3: E-UTRA 4: NR 5: LTE-M 6: NB-IoT 7: Wi-Fi 8: Bluetooth 9: Fixed access, e.g. DSL, Fibre |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

### Supported device velocity

Maximum speed supported by the slice at which a defined QoS and seamless transfer between TRxPs, which may belong to different deployment layers and/or radio access technologies (multi-layer /-RAT), can be achieved.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | Kilometres per hour (km/h) |
| - Example | 1: Stationary: 0 km/h 2: Pedestrian: 0 km/h to 10 km/h 3: Vehicular: 10 km/h to 120 km/h 4: High speed vehicular: 120 km/h to 500 km/h |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional information

For non-low latency requirements, the expected speeds might not be a problem. More important is the support of URLLC services under high mobility scenarios. Not sure, which reliabilities could be achieved.

Orchestrator may use this attribute to orchestrate the resources and network functions. If value is set to 0, it means no mobility is supported (e.g., the UE is not moving).

### Synchronicity

This attribute provides synchronicity of communication devices. Two cases are most important in this context:

Synchronicity between a base station and a mobile device and

Synchronicity between mobile devices.

#### Availability

The synchronicity between devices over PC5 in absence of the network is not in scope of this attribute.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 0: not available 1: between BS and UE 2: between BS and UE & UE and UE |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Accuracy

This parameter describes the accuracy of the synchronicity.

| Parameters |  |
| --- | --- |
| Value | Float |
| Measurement unit | seconds |
| - Example | 0,0000001 seconds (1µs) |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Additional Information

Today these requirements are met by the deploying cable connections/networks as like industrial Ethernet systems or fieldbuses. These networks are normally closed solutions by a single vendor in which all the equipment is perfectly aligned. Based on standards like IEEE 802.1 AS [9] or IEEE 1588 [8] very high synchronicity can be achieved in the networks.

### Terminal density

This attribute described the maximum number of connected and/or accessible devices per unit area (per km2) supported by the slice.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | Number per km2 |
| - Example | 10.000 devices per km2 for some industry 4.0 use cases (but this is required a campus coverage only) 1.000.000 devices per km2 for MIoT use cases |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

Additional information

Most of the required densities can be supported with 5G. Most challenging are the MIoT use cases for which it is not sure of the required device densities can be fulfilled.

This attribute describes the maximum number of users/terminals supported per area. In case fewer users are present at moment, the slice can be scaled down, e.g. resources can be released and for instance be used by other slices.

### Uplink throughput per slice

The overall achievable data rate of the slice in uplink that is available ubiquitously across the coverage area of the slice.

#### Minimum Uplink Throughput

This attribute describes the minimum data rate supported by the slice in uplink.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | Bytes |
| - Example | TBC |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Maximum Uplink Throughput

This attribute describes the maximum data rate supported by the slice in uplink.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | Bytes |
| - Example | TBC |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: Either Uplink Throughput per slice or Uplink Throughput per slice shall be present.

#### Additional information

This attribute might be used to set throughput guarantees per slice (and not per user).

Minimum throughput can be defined in order to guarantee a minimum performance required to achieve a sufficient quality experience (dependent on the selected service type).

Maximum throughput can be used to offer different slice contract qualities level, e.g. gold, silver and bronze which have different maximum throughput values.

### Uplink throughput per user

#### Minimum Uplink Throughput

This attribute describes the minimum data rate supported by the slice per UE in uplink, which is required to achieve a sufficient quality experience (dependent on the selected service type). If the value is 0, best effort traffic is expected where no minimum throughput is guaranteed.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| - Example | 180 Kbps for VoIP traffic 75.000 Kbps for entry level VR latency. |
| - Attribute Presence | - Conditional |
| - Tags |  |

| Parameter Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| - Optional | - X |

#### Maximum Uplink Throughput

This attribute describes the maximum data rate supported by the slice per UE in uplink. These parameters could be used in order to offer different contract qualities like gold, silver and bronze.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| - Example | Bronze customer: 10.000 Kbps Silver customer: 100.000 Kbps Gold customer: 200.000 Kbps |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: Either Uplink Throughput per slice or Uplink Throughput per slice must be present.

#### Availability

This attribute might be used to set different guarantees in terms of throughput experienced by the customer in uplink.

Minimum throughput can be defined in order to guarantee a minimum performance required to achieve a sufficient quality experience (dependent on the selected service type).

Maximum throughput can be used to offer different contract qualities level, e.g. gold, silver and bronze which have different maximum throughput values.

Orchestrator may use this attribute to orchestrate the resources and (R)AN/CN may use this attribute to optimize the scheduling.

### User management openness

- This attribute describes the capability that NSC could manage their users or groups of users’ network services and corresponding requirements. For instance, if NSC Y orders a slice which is capable to support X users of Y, then Y should be capable to decide which X users could use this slice. Hence, Y could manage the users, in terms of add, modify or delete users to receive network services provided by the specific slice.

| Parameters |  |
| --- | --- |
| Value | Binary |
| Measurement unit | NA |
| - Example | 0: not supported 1: supported |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

### User data access

The attribute defines how the slice (or mobile network) should handle the user data. The options are as follows:

The device has access to the Internet

All data traffic is routed to the private network (e.g. via tunnelling mechanism such as L2TP, VPN tunnel, etc.)

All data traffic stays local and the devices do not have access to the Internet or private network

#### Data access

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer…} |
| Measurement unit | NA |
| - Example | 0: Direct internet access 1: Termination in the private network 2: Local traffic (no internet access) |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Tunnelling Mechanism

The attribute defines the tunnelling mechanism how the user data can be delivered to the external private data network. 3GPP [12] lists the interworking with data networks and tunnelling mechanism used.

| Parameters |  |
| --- | --- |
| Value | {Integer, Integer, Integer..} |
| Measurement unit | NA |
| - Example | 0: L2TP Tunnel 1: GRE Tunnel 2: VPN Tunnel 3: Label bases routing 4: Other |
| - Tags |  |

| Parameter Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| - Optional |  |

Note: This parameter is present if Data Access is 1(Termination in the private network).

#### Additional information

Many solutions are already available today as like Control and User Plane Separation (CUPS) [10], [11], N6 tunnelling mechanism [12], etc.

### V2X support

This parameter describes if the V2X communication mode is supported by the slice.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| - Example | 0: NO 1: YES-EUTRA 2: YES- NR 3: YES -NR and E-UTRA |
| - Tags |  |

| Attribute Presence |  |
| --- | --- |
| Mandatory | X |
| Conditional |  |
| - Optional |  |

#### Additional information

This is supported in EPS and its support is being developed in 5GS