SA WG2 Meeting #129BIS	S2-1811987

26 – 30 November, West Palm Beach, USA

Source:	Oracle, Deutsche Telekom, Cisco, AT&T, Sprint, Verizon, KDDI, NTT Docomo, China Mobile

Title:	Conclusion on KI#3 for Service Framework Implicit Discovery

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes conclusions for key issue 3 on service framework implicit discovery.

# 1 Discussion

This contribution uses Service Framework Support Functions SFSF to denote the entity that can perform common service framework functions on behalf of a consumer or producer.  This contribution does not identify all possible common service framework functions.  For the purposes of this contribution, common service framework functions includes a message passing system, service discovery, and service selection functions.  Explicit service discovery and service selection is shown in figure 1.  Implicit service discovery and service selection is shown in figure 2.

Figure 1: Explicit Service Discovery and Selection

Figure 2: Implicit Service Discovery and Selection

This document proposes a conclusion on the implicit discovery via the Service Framework Support Function (SFSF) based on figures 1 and 2 in order to avoid any misinterpretation.

# 2 Proposal

This document proposes the following changes into TR 23.742.

**************** Start change all new text **********

## 8.x	Conclusions for the service framework

Principles related to registration and discovery of services

A. 1)	It shall be possible that NF Services are discovered and selected by the Service Framework Support Function without explicit discovery request from the consumer, e.g. by metadata.

A.2) 	Service selection and discovery mode shall be supported as configuration at time of deployment.

********************  End of changes  ********************