SA WG2 Meeting #129BIS	S2-1812313

26 – 30 November, 2018, Westpalm Beach, USA

Source:	Samsung

Title:	KI#5 KI#11 Solution 3 Update

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_eV2XARC / Rel-16

Abstract of the contribution: It is proposed to update solution 3 to capture detail procedural operation.

# Discussion

In order to support eV2X in 5G, the PCF should be able to update the service authorization and provisioning parameters to the UE at any time if the condition is met. Therefore, when UE performs registration procedure, the AMF needs to establish Policy Association with the PCF for V2X. The UE needs to indicate its capability of V2X communication in the registration request, as described in the solution 6 (6.6.2.1). Then the AMF discovers the PCF supporting V2X by utilizing NRF service or local configuration in the AMF. If the UE wants to receive V2X communication parameters, the UE includes UE Policy Container in the registration request message with indicating request of V2X Policy Provisioning.

Proposal 1) AMF discover the PCF for V2X communication if UE indicates V2X capability during the registration procedure, and the AMF establishes policy Association.

There is optional behaviour that UE triggered V2X Policy provisioning procedure. The triggering condition for this procedure should be clarified, in order to make sure the procedure is not triggered unnecessarily.

Proposal 2) Clarify the trigger condition of UE triggered V2X Policy Provisioning procedure.

# Proposal

* * * First change * * *

## 6.3	Solution #3: Service Authorization and Provisioning to UE for eV2X communications over PC5 and NG-Uu reference points

### 6.3.1	Functional Description

This solution addresses Key Issue #5 and Key Issue #11.

This solution applies to the architecture option Alternative#1 in Annex A.

In 5GS, the service authorization parameters for V2X PC5 and V2X NG-Uu communication can be made available in the UE in different ways, i.e.

-	pre-configured in the ME, or

-	configured in the UICC, or

-	preconfigured in the ME and configured in the UICC, or

-	provided/updated by the V2X AS via PCF, or

-	provided/updated by the PCF to the UE

When the service authorization parameters for V2X PC5 and NG-Uu communication are provided by the PCF, the existing procedures will be used:

-	When the UE performs initial Registration as defined in clause 4.2.2 "Registration Management procedures" in TS 23.502 [9], the UE indicates to AMF the V2X capability, so that the AMF discovers the PCF supporting V2X by utilizing the NRF or local configuration in the AMF. If the UE supports V2X communication and it doesn’t have valid V2X communication Policy/Parameter, the UE includes in the registration request message the UE Policy Container with indicating the V2X Policy Provisioning request. Based on the indication in the UE Policy Container, the PCF determines whether to provision the eV2X PC5 and eV2X NG-Uu service authorization parameters to the UE using procedures as defined clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy delivery" in TS 23.502 [9] and clause 6.1.2 "Non-session management related policy control" in TS 23.503 [10].

-	During mobility, e.g. UE moves from one PLMN to another PLMN, the UE performs Registration and the PCF may update the UE with new service authorization parameters for the new PLMN.

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform V2X communication service over PC5 and/or NG-Uu reference point, the PCF may need to update the policy/parameters to the UE. If the serving PLMN is removed from the list when the UE is authorized to perform V2X communication service over PC5 and/or NG-Uu, it means the service authorization is revoked in the UE.

-	When the user is roaming, the subscription change resulting in updates of V2X policy/parameters will be transferred by H-PCF via V-PCF.

-	If the V2X configuration parameters is located in a database external to the UDR the V2X configuration parameters will be transferred to the PCF via the AF (i.e. the external database interacts like a 3rd Party AS).

NOTE 1:	Update of provisioning information directly to the UE (i.e. V1 reference point) is available in this option as well.

NOTE 2:	The UE provisioning is controlled by the PCF and in this solution no UE triggered provisioning update is needed. If the UE, due to a failure case or of some other reason, see a need to update the provisioning, the UE has to perform initial registration in the network. The solution assumes this registration trigger is very rare occurrence since the UE can be kept up to date by the PCF if any change in configuration is to occur. If UE triggered policy provisioning is adopted in general (see clause 6.3.2a), then such procedure can be also applied for V2X policy update from the UE.

NOTE 3:	Deployment of PCF for the purposes of supporting UE policies for V2X via AMF does not depend on deployment of PCF for Session based PCC.

### 6.3.2	Procedures

Procedure for UE policy update can be reused:

-	Clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy delivery" in TS 23.502 [9].

-	Clause 6.1.2 "Non-session management related policy control" in TS 23.503 [10].

#### 6.3.2a	Option of UE triggered V2X Policy provisioning procedure

The procedure is initiated when the UE determines the V2X Policy is invalid in following cases:

- 	if the parameter is outdated (e.g. the validity timer for the eV2X communication policy/parameter expires),

-	if there are no valid parameters for current area, or

-	parameters lost or deleted locally by abnormal situation.

This procedure, if agreed to be adopted for general UE triggered policy provisioning then it can also be applied for V2X.

Figure 6.X.2a-1: UE triggered V2X Policy provisioning procedure

- UE sends UE Policy provisioning request including UE Policy container (Indication of V2X Policy provisioning request) to AMF.

- The AMF sends Npcf_UEPolicyControl_Update request to the PCF to establish an AM policy control association with the PCF included the UE Policy container received from UE.

- The UE Policy delivery procedure defined in clause 4.2.4.3 of 23.502 [9] is triggered.

NOTE 3: Step 2 and 3 reuse existing messages.

### 6.3.3	Impact on existing entities and interfaces

Impacts on existing entities, interfaces and procedures are limited to adding new policies/parameters, no new functionality is introduced. Below are the impacted entities and interfaces listed:

-	UDR for subscription change in the list of PLMNs where the UE is authorized to perform V2X communication service over PC5 reference point

-	PCF handles service authorization and provisioning of parameters for V2X PC5 and V2X NG-Uu communication based on the the Indication of V2X Policy provisioning request in the UE Policy Container.

-	UE handles the policy/parameters for service authorization using existing 5GC procedures with supporting UE triggered V2X Policy provisioning procedure if available. UE decides whether to include the Indication of V2X Policy provisioning request in the UE Policy Container in the initial Registration procedure or UE triggered V2X Policy provisioning procedure.

-	AMF transferring the policy/parameters to the UE using existing procedures. AMF requests to create UE Policy Association to the PCF for V2X communication, based on the V2X capability indication in the registration request message.

-	Impacted interfaces: N1, N8, N15, N24.

To perform the UE provisioning the user plane set-up is not needed.

### 6.3.4	Topics for further study

None is identified.

### 6.3.5	Conclusions

TBD

* * * End of change * * *