# SA WG2 Meeting #129bis	S2-1812512

# 26-30 November, 2018, West Palm Beach, FL, USA	(revision of S2-1812104)

Source:	Apple

Title:	23.793: QUIC Datagram Tunnel User Plane Convergence Method

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution proposes an alternate convergence method for traffic of non-multipath capable transport protocols.

# 1.	Introduction

This proposal introduces a convergence method for traffic of non-multipath capable transport protocols, based on QUIC. By utilizing QUIC’s loss detection and congestion control features, it allows valuable information such as smoothed RTT, RTT variance, packet loss and throughput to be provided to the Performance Measurement Function (PMF) of ATSSS to assist with path determination decisions. Because this information is already built into the transport protocol, it allows for path characteristics to be passively-measured. Therefore, it does not rely on active probing which does not scale well and might consume unnecessary system resources, including power.

This proposal also leverages the connection migration feature of QUIC for ATSSS path switching. The transport comes with path validation mechanisms, such that traffic of a given QUIC connection can go over the new path upon a successful migration. This provides a way to maintain packet ordering when traffic of a flow is switched from one path to another.

This solution addresses Key Issues #1, #2 and #4.

# 2.	Proposal

It is proposed to modify TR 23.793 1.1.0 as follows:

***** First Change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[3]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[4]	3GPP TS 23.139: "3GPP system - fixed broadband access network interworking".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[8]	IETF RFC 6824: "TCP Extensions for Multipath Operation with Multiple Addresses".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	IETF RFC 1928: "SOCKS Protocol Version 5".

[10]	draft-ietf-tcpm-converters-01: "0-RTT TCP Convert Protocol".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[11]	3GPP TS 38.215: "Physical layer measurements".

[12]	3GPP TS 38.300: "NR and NG-RAN Overall Description".

[13]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Overall Description".

[14]	IETF RFC 1323: "TCP Extensions for High Performance".

[15]	IETF RFC 4960: "Stream Control Transmission Protocol".

[16]	IETF RFC 7829: "SCTP-PF: A Quick Failover Algorithm for the Stream Control Transmission Protocol".

[17]	IETF RFC 3758: "Stream Control Transmission Protocol (SCTP) Partial Reliability Extensions".

[18]	IETF RFC 5681: " TCP Congestion Control".

[19]	IETF RFC 5348: "TCP Friendly Rate Control (TFRC): Protocol Specification".

[20]	IETF RFC 4340: "Datagram Congestion Control Protocol (DCCP)".

[21]	IETF RFC 2890: "Key and Sequence Number Extensions to GRE".

[aa]	draft-ietf-quic-transport-16: “QUIC: A UDP-Based Multiplexed and Secure Transport”

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[bb]	draft-ietf-quic-recovery-16: “QUIC Loss Detection and Congestion Control”

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[cc]	draft-pauly-quic-datagram-00: “An Unreliable Datagram Extension to QUIC”

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

***** Next Change *****

#### 6.1.7.4	User Plane Convergence Method using L4 Multipath Solutions (MP-TCP, QUIC, MP-QUIC, SCTP, UDP generic)

This method allows transport of the IP PDUs across both accesses between the UE-AT3SF and UPu-AT3SF. It uses a Layer 4 multipath transport service enabling IP flows to use multiple paths simultaneously.

The L4 multipath transport service may use either MPTCP, QUIC, MP-QUIC, SCTP or UDP generic. MP-TCP sets up multiple TCP subflows over the different access networks, MP-QUIC several UDP flows, QUIC several UDP flows, SCTP several SCTP flows and the UDP generic approach several UDP flows.  UE-AT3SF and UP-AT3SF are responsible for managing the MP-TCP/QUIC/MP-QUIC Hybrid Access paths, including establishment and tear down.

The peers may terminate the end user layer 4 sessions before transporting the data over the access paths, effectively executing a proxy function for these end user sessions.

One or more IP addresses may be allocated, if needed, on the UPFu-AT3SF to differentiate between the flows over both accesses. Other mechanisms to provide such a differentiation are precluded. The subsequent assigned IP addresses are pushed to the peers of the layer 4 sessions using internal messages of either MP-TCP, MP-QUIC, SCTP or UDP generic (e.g. ADD_ADDRESS for MP-TCP).

The associated user plane protocol stack is shown in Figure 6.1.7.4-1.

a)	MPTCP transport.

Figure 6.1.7.4-1a: User Plane Protocol Stack with L4 Multipath between UE-AT3SF and UPu-AT3SF

b)	MP-QUIC transport.

Figure 6.1.7.4-1b: User Plane Protocol Stack with L4 Multipath between UE-AT3SF and UPu-AT3SF

c)	SCTP transport.

Figure 6.1.7.4-1c: User Plane Protocol Stack with L4 Multipath between UE-AT3SF and UPu-AT3SF

d)		UDP generic transport.

Figure 6.1.7.4-1d: User Plane Protocol Stack with L4 Multipath between UE-AT3SF and UPu-AT3SF

e)		QUIC transport.

Figure 6.1.7.4-1e: User Plane Protocol Stack with L4 Multipath between UE-AT3SF and UPu-AT3SF

***** Next Change *****

##### 6.1.7.4.5 	Traffic distribution with QUIC

###### 6.1.7.4.5.1	Traffic distribution over multi access using QUIC between the UE and UPu-AT3SF

In this mode of operation QUIC [aa] is used as a tunneling transport between the UE-AT3SF and UPu-AT3SF. This solution enables single communication flows (e.g.: IP flows) to/from the UE to use alternative paths between UE-AT3SF and UPu-AT3SF.

By utilizing QUIC’s loss detection and congestion control features [bb], valuable information such as smoothed RTT, RTT variance, packet loss and throughput can be provided to the Performance Measurement Function (PMF) of ATSSS to assist with path determination decisions. Because this information is already built into the transport protocol, it allows for path characteristics to be passively-measured. Therefore, it does not rely on active probing which does not scale well and might consume unnecessary system resources, including power.

The QUIC tunnel service between UE-AT3SF and UPu-AT3SF sets up one or more QUIC connections over the different accesses. By leveraging the connection migration feature built into QUIC, each QUIC connection can be steered or switched from one path to another. The QUIC protocol is equipped with path validation mechanisms, such that traffic of a given QUIC connection can go over the new path upon a successful migration. This provides a way to maintain packet ordering when traffic of the QUIC connection is moved from one path to another.

Establishing multiple QUIC connections between UE-AT3SF and UPu-AT3SF provides flexibility in grouping tunneled traffic based on criteria such as:

- QoS – Each QUIC connection can represent a range of QFI values, including a single value.

- IP tuples – Inner IP flow destined to a particular address may get its own dedicated QUIC connection.

- Reliability – Each QUIC connection can represent a reliability class.

Multiple streams may optionally be established within a QUIC connection to provide finer-grained traffic differentiation and characterization, e.g. each stream may represent a Quality of Service level. In addition, one or more streams within the connection may also be used to transfer unreliable datagrams [24] if desired.

The QUIC tunnel service uses NULL encryption, as traffic for the QUIC connections from UE-AT3SF to UPu-AT3SF is already protected by the underlying access network connectivity to 5GC.  This protection is provided by 5G-AN and by the IPsec protocols, for 3GPP and non-3GPP access networks, respectively. Doing so avoids redundant crypto operations which may introduce additional processing and power costs.

6.1.7.4.5.1.1 	Usability

This solution can be used for any PDN type and addresses traffic whose transports that are not multipath capable. For multipath capable transports such as MPTCP, SCTP, or MP-QUIC, their traffic should go directly over the corresponding access network.

Because a QUIC connection may migrate from one path to another, but not be split across the paths, this solution covers only traffic steering and switching. Because this works at the connection level, traffic belonging to a particular QUIC connection will migrate together as a group. Traffic splitting is therefore not supported by this solution.

6.1.7.4.5.1.2 	IP addressing

In this solution the UE-AT3SF is assigned with the IP address values to be used over each access. These IP address values are provided to the UE-AT3SF by the UPc-AT3SF. The UPu-AT3SF makes use of a single IP address for the QUIC tunnel endpoint.

***** End of Changes *****