SA WG2 Meeting #129BIS	S2-1812562

26-30 November, 2018, Westpalm Beach, USA

Source:	Samsung, SK Telecom, OPPO

Title:	Solution for key issue #2: Provisioning of UE Radio Capability Information

Document for:	Proposal

Agenda Item:	6.28

Work Item / Release:	FS_RACS / Rel-16

Abstract of the contribution: It is proposed to provisioning method of UE Radio Capabilities and its representation from AF to the network functions.

# Discussion

This proposal was submitted in SA2 #129 meeting but it wasn’t handled due to lack of time.

This proposal addresses key issue #2 “Where are the UE radio capabilities stored”.

Whether assigning the identifier to be used to represent the UE Radio Capabilities should be discussed in solution for key issue #1: “How are the UE Radio Capabilities identified?”. It could be three of them:

- Assignment by the UE manufacturer

- Assignment by the network operator

- Assignment by a 3rd party (e.g., GSMA)

In this proposal, it is described how to distribute the assigned identifier and corresponding UE Radio Capability to the network, especially to AMF. Provisioning of UE Radio capability information to the network is performed at an early stage of release of the new type of device (with new combination of radio capabilities), so that the UE Radio Capability information is distributed to all AMFs in the PLMN. Therefore, the proposed methods minimise the chances that a network receives an unknown identifier, so that it can reduce NGAP and RRC signalling to retrieve full radio capabilities from the UE. Also as this information is provisioned by authorized AF, it improves the reliability the process.

This solution proposes to use bulk provisioning similar to that defined in TS 23.502, section 4.1.5.3.2.4. The AF requests to the NEF for provisioning of Radio Capability information (Identifier and corresponding Radio Capabilities). The NEF performs bulk provisioning to all AMFs in a PLMN. Upon receiving bulk provisioning from the NEF, the AMF stores this information.

In the revision,

- Clarify the AF is integrated with data base which stores full set of UE Radio Capability Information. The data can be provisioned to the data base by the manufacturer or the operator. The AF can trigger the proposed API if the data base is updated.

- Remove AMF behaviour for ID allocation.

- Merged text from S2-1812563 for on-demand operation.

# Proposal

## 6.X	Solution #X: Bulk Provisioning of UE Radio Capability from AF

### 6.X.1	Introduction

The solution addresses key issue #2. It proposes that the AF provisions to the AMF via NEF the representation of UE radio capability (e.g., UE Radio Capability ID) and corresponding UE radio capabilities. In this solution, UE Radio Capability Information is composed of the UE Radio Capability and corresponding UE Radio Capability ID. The NEF provisions the UE Radio Capability Information to the AMFs in the PLMN by using bulk provisioning method.

### 6.X.2	Functional Description

This solution has the following characteristics:

- The AF is integrated with the data base for UE Radio Capability Information, in which the information can be provided by UE manufacture or operator. The AF is able to provision UE Radio Capability Information (UE Radio Capability ID and corresponding UE Radio Capabilities) to the AMF(s). The AMF can trigger the API for UE Radio Capability provisioning if the UE Radio Capability information is updated.

- The NEF exposes API for UE Radio Capability provisioning to the AF. The NEF authorizes that the AF provisions the UE Radio Capability Information.

- The NEF retrieves AMF address from the NRF.

- The NEF performs bulk provisioning to all AMFs in the PLMN.

- The AMF exhibits service operation for UE Radio Capability Provisioning. Consumer of this service operation is NEF.

- The AMF can retrieve the UE Radio Capability information from AF via NEF if AMF has no UE Radio Capability Information stored.

### 6.X.3	Procedures

### 6.X.3.1	Bulk provisioning of UE Radio Capability

In this clause, the procedure for bulk provisioning of UE Radio Capability is described.

Figure 6.X.3-1: Bulk Provisioning of UE Radio Capability information to AMFs

1.	The AF triggers Nnef_RadioCapability_Provisioning Request service operation to the NEF. The AF includes UE Radio Capability Information in this message. The UE Radio Capability information is composed of UE Radio Capabilities and the corresponding identifier (e.g., UE Radio Capability ID). The AF can include multiple sets of UE Radio Capability Information.

If CAPIF is not supported, the AF is locally configured with the API termination points for this service operation. If CAPIF is supported, the AF obtains the service API information from the CAPIF core function as specified in TS 23.222 [4].

2.	The NEF authorized the AF request based on the operator policy.

3.	The NEF acknowledges the execution of Nnef_RadioCapability_Provisioning_request. The NEF stores the received UE Radio Capability Information in order to provision to AMFs in the PLMN after invoking NF discovery service operation. The NEF may cache the UE Radio Capability Information provisioned by the AF.

4. 	The NEF invokes Nnrf_NFDiscovery_Request to the NRF. In this message, the NEF indicates NF Type as AMF. The NEF includes the expected NF service name for the provisioning request (i.e., Namf_Provisioning service operation).

5.	The NRF authorizes the Nnrf_NFDiscovery_Request. If allowed, the NRF provides the set of address of discovered AMFs to the NEF via Nnrf_NFDiscovery_Request Response message. The NEF stores the set of address of AMFs.

The NEF can register with the NRF for any newly registered AMF by invoking Nnrf_NFStatus_Subscribe. If NEF gets notified from the NRF for the newly registered AMF in the PLMN, the NEF can trigger Namf_Provisioning_Request service operation in order to provide the cached UE Radio Capability Information to the AMF.

6.	Based on the information of the discovered AMFs in the PLMN from step 5, the NEF invokes bulk Namf_Provisioning_Request to discovered AMFs in order to provision UE Radio Capability Information. In this message, one or set of UE Radio Capability Information received from the step 3 are included.

NOTE: Namf_Provisioning service operation is not per UE operation.

7.	The AMF stores the received UE Radio Capability Information. The AMF acknowledges to the NEF by sending Namf_Provisioning_Response. The AMF uses the UE Radio Capability Information to identify UE Radio Capability when UE requests registration with UE Radio Capability ID.

8.	The NEF may notify to the AF that the UE Radio Capability Information is successfully provisioned to the all AMF in the PLMN, based on the local policy.

If the AF is trusted in the PLMN and allowed to invoke Namf_Provisioning service operation, the AF can directly use Nnrf service operation to discover AMFs and can directly use Namf service operation to provision the UE Radio Capability Information.

#### 6.X.3.2	On-demand UE Radio Capability Information Retrieval from AF

#### This section applies the case that AMF has no UE Radio Capability Information for the AF information or cannot derive UE capabilities according to the capability ID received from the UE (e.g. the database is not up to date), the AMF acquires UE Radio Capability Information from the AF via NEF. This procedure ensures the AMF always has the up to date UE Radio Capability information.

Figure 6.X.3-2: On-demand UE Radio Capability from AF

Step 1: UE provides UE capability ID and AF information e.g. manufacture information or PLMN information to the AMF.

Editor’s Note: Whether AF information is needed or not is FFS.

Step 2: AMF obtains UE capabilities according to the stored UE Radio Capability Information for the received UE Capability ID.;

Step 3: If AMF has no UE Radio Capability Information for the AF information or cannot derive UE capabilities according to the capability ID received from the UE, the AMF acquires UE Radio Capability information from the AF via NEF.

Regardless of step 1 and step 2, if AMF determines that the stored UE Radio Capability Information is not up to date information, based on local configuration, the AMF can trigger step 3 to retrieve the up to date UE Radio Capability Information from the AF.

Editor’s Note:	Service operation for step 3 will be further specified.

Step 4: AMF transfers UE capabilities to the RAN if applicable.

### 6.X.4	Impacts on existing entities and interfaces

Editor's note:	This clause describes impacts to existing entities and interfaces.

NEF: new API exposure for UE Radio Capability Information provisioning. The NEF can cache the UE Radio Capability Information received by the AF to provision to the AMF. The NEF can store the address of AMFs in the PLMN after querying to the NRF.

AMF: new Namf service to allow for provisioning of UE Radio Capability Information. AMF can trigger service operation to retrieve UE Radio Capability Information from the AF.

### 6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.