SA WG2 Meeting #129bis	S2-1812818

West Palm Beach, Florida, USA, November 26 - 30, 2018

Source:	Nokia, Nokia Shanghai Bell, Motorola Mobility, Lenovo, Convida Wireless LLC,Telecom Italia, NEC

Title:	Evaluation of solution in clause 6.3.2

Document for:	Approval

Agenda Item:	6.21

Work Item / Release:	FS_eNS/Rel-16

Abstract of the contribution:

# 1.	Proposal

This document provides some update and conclusion on clause 6.3.2 in TR 23.740.

It is proposed that the following text changes and proposals are added to TR 23.740.

***********************PROPOSED TEXT********************************

### 6.3.2	Solution #3.2: Solution to KI#3 on Slice Specific Authentication and Authorization using non 3GPP credentials

#### 6.3.2.1	Introduction

The scenario considered for the solution is that there is an Authentication, Authorisation and Accounting Server (AAA-S) deployed in a PLMN or in a third-party network that allows Slice-Specific secondary authentication and authorisation of users who have the right of access to certain slices. The AAA Server handles the User IDs and credentials for a Secondary authentication at a non 3GPP User ID level, after the UE has been authenticated by the 3GPP system for PLMN access. A AAA proxy function (AAA-F) is also defined to provide a single point of interaction from the PLMN with the third parties. The AAA-F can be a standalone or its services could be supported by other NFs (i.e. AUSF or NEF).

#### 6.3.2.2	Functional description

If a UE subscription includes an S-NSSAI of a slice that requires Slice-Specific Secondary Authentication and Authorisation, the UDM stores a flag indicating that this step is needed for the S-NSSAI. The IP address or FQDN/realm of the AAA Server that would perform the authentication and authorisation may be stored in the AAA-F per S-NSSAI. Alternately, if the User ID in the third party is defined as a NAI (see RFC 4282 [8]), i.e. the User ID is in the form user@domain, the IP Address is not needed in AAA-F and the correct AAA-S is derived at AAA-F by resolving the domain part of the NAI.

When a UE performs a registration request which includes S-NSSAI(s) in the Requested NSSAI which needs Slice-Specific Secondary Authentication and Authorisation (as detected based on the subscription information, as outlined above), the AMF executes on top of any required PLMN specific Authentication and Authorization step, an Authentication and Authorization step that is run with the UE and involves the AAA Server for the specific S-NSSAI(s). In the worst case, one such step is needed for each S-NSSAI. In general, there can be a mix of slices requiring Slice-Specific Secondary Authentication and Authorisation, and slices that do not require this.

For the S-NSSAIs which require Slice-Specific Secondary Authentication and Authorisation, the UE needs to be provided with the necessary credentials and algorithms necessary to authenticate itself with the AAA Server. This aspect is left out of the scope of this solution.

In addition, the UE includes a security capability indicating its support for Slice-Specific Secondary Authentication and Authorisation in the registration request facilitating the AMF to determine whether it can execute Slice-Specific Secondary Authentication and Authorisation or not. If the security capability is not included in the registration request (e.g. in case of Rel-15 UEs), the AMF shall not allow UE to access to any Network Slice for which Slice-specific Secondary authentication is required.

The proposal assumes that the assumed transport protocol is EAP and is summarized by the following figure:

Figure 6.3.2.2-1: Registration with Slice-Specific Secondary Authentication and Authorisation for Slice Access (high level concept)

Step 4 is conditional to both the UE security capability and the S-NSSAI subscription data, checked at step 3, including a flag that indicates Slice-Specific Secondary Authentication and Authorisation is needed. If this is needed and UE supports it, this step is executed. The start of this step suspends the current Registration procedure timer. This timer is restarted when all the pending EAP procedure for slices that are subject to Slice-Specific Secondary Authentication are completed.

Editor’s note: This approach needs to be checked whether it is acceptable by CT1 and based on the feedback changes to the solution may be done in normative phase to align with the preferred way to handle NAS and EAP timers interaction.

Once it is executed, status can be kept by the AMF in the UE context, so extra authentication is not repeated at subsequent registrations until a re-authentication is required by the AAA Server or the PLMN, based on policy. If this is needed but UE does not support it, this step is skipped and the AMF shall not include an NSSAI to the Allowed NSSAI in step 5.

Explicit challenges used to re-authenticate the UE, or a request to revoke the authorization of the UE may come from the AAA Server and the AAA-F routes to the current serving AMF based on a binding between the User ID and the GPSI of the UE established when the UE is authorised. The AAA Server keeps a coupling between the User ID of the UE and the UE GPSI so it can pinpoint the UE for the User ID when it needs to be re-authenticated or its authorization status changes and needs to be revoked.

Once step 4 is executed successfully, and the registration accepted, the UE, after optional NSSF interaction (not explicitly shown above), is provided an allowed NSSAI including the slices that were successfully authorised. SM procedures can take place in the authorised slices for the UE.

It the UE fails Slice-Specific Secondary Authentication, and this results in the UE not having any Allowed NSSAI, the UE may be Accepted with no Allowed NSSAI and, depending on operator policy, remain registered with operator-specific service restriction in the same AMF or it may be re-allocated to a specific AMF which applies specific service restrictions. The AMF then may deregister the UE after a certain time configurable by the operator.

If the UE fails Slice-Specific Secondary Authentication for a specific slice, how the UE obtains fresh/valid credentials from third parties is outside the scope of 3GPP.

NOTE:	A UE may gain access to a slice that needs Slice-Specific Secondary Authentication and Authorisation and not trigger any PDU session establishment (e.g. SMS only device) or require a PDU session only after SMS trigger after is assigned an Allowed NSSAI and is successfully registered.

#### 6.3.2.3	Procedures

##### 6.3.2.3.1	Secondary Authentication and Authorization During Registration

During the registration procedure, before step 15 occurs, in figure 6.3.2.3.1-1, the AMF knows whether some S-NSSAIs require Slice-Specific Secondary Authentication and Authorisation based on information from the UDM.

Figure 6.3.2.3.1-1: TS 23.502 [3] Figure 4.2.2.2.2-1: Registration procedure

If the AMF detects the need to execute the Slice-Specific Secondary Authentication and Authorisation, the steps marked as "2" in Figure 6.3.2.3.1-2 are executed. In the event the Allowed NSSAI needs to be computed and potential new AMF determined by a NSSF, then these Authentication and Authorization steps are executed upfront so that the NSSF receives from the AMF only the authorized by Slice-Specific Secondary Authentication and Authorisation and subscribed-to slices.

Figure 6.3.2.3.1-2: Registration with third party Authentication and Authorisation

1.	Steps 1-14 of the Registration procedure. Steps 2 are not executed if no S-NSSAI in the UE subscription is requiring Slice-Specific Secondary Authentication and Authorization or the UE does not have a security capability.

2a.	[Conditional] The AMF requests the UE User ID for the S-NSSAI. The NAS message includes the S-NSSAI value so the right Network Slice authentication is executed and so the related User ID and credentials are used by the UE. The AMF may start in parallel several such authentication and Authorisations procedures if more than one S-NSSAI is subject to Secondary Slice-Specific Authentication and Authorisation. This stepsuspends the current Registration procedure timer and it is resumed when the EAP interactions for Slice-Specific Secondary Authentication complete.

Editor’s note: This approach needs to be checked whether it is acceptable by CT1 and based on the feedback changes to the solution may be done in normative phase to align with the preferred way to handle NAS and EAP timers interaction.

2b.	[Conditional] The UE provides the User ID for the S-NSSAI alongside the S-NSSAI

2c-2d.	[Conditional] The AMF contacts the AAA Server via AAA-F. It includes also the S-NSSAI and the UE GPSI for binding of Network Slice User ID to the UE GPSI (so the AAA Server is allowed to contact the UE for further reauthentication based on its policy, or to revoke the UE authorisation, or to optimise future Authorisations based on the AAA server trusting the GPSI presented by the PLMN. If the AAA Server trusts the GPSI provided, by the PLMN, and it had if it had a previous mapping of the GPSI to the User ID, the AAA Server may go directly to step 2k.

2e-2j.	[Conditional] The UE EAP based authentication happens. The number of steps is indicative, more may occur.

2k-2m.	[Conditional] The EAP authentication completes successfully. Optionally, in steps 2k and 2l the AAA Server and the AAA-F convey to the AMF a list of S-NSSAIs the successful Authentication and Authorisation applies to, so the AMF can avoid executing multiple Authentications and Authorisations towards the same AAA Server for the same User ID for the S-NSSAIs in the list. This option should not be used if the AAA-s intends to collect accounting information for S-NSSAI usage by the UE.

3.	The AMF stores in the UE context the outcome of steps 2 and if a list of S-NSSAIs was provided sharing the same User ID, then the AMF stores that also. The AMF can now determine, assisted by NSSF if needed, the Allowed NSSAI and this may trigger selection of another AMF.

4.	The registration completes, and eventually a Registration Accept is sent to the UE including the Allowed NSSAI and the GUTI allocated by the target AMF if the AMF is relocated in step 3.

##### 6.3.2.3.2	Secondary Re-Authentication and Re-Authorization after Registration

The AAA-S may, based on application layer event, prompt the network to Re-authenticate and Re-authorize the user by sending a request to the AAA-F. For example, the AAA-S may initiate the request based on security policies, in order to revoke the User's authorization to access the slice.

In response to the request from the AAA-S, the AAA-F will initiate re-authentication and re-authorization by sending a request to the AMF. Upon receiving the request, the AMF initiates step 2 of Figure 6.3.2.3.1-2. If the purpose of the request is to revoke the User's authorization to access the slice, then the UE's Allowed NSSAI is updated accordingly via a Configuration Update per clause 4.2.4.2 of TS 23.502 [3].

#### 6.3.2.4	Impacts on existing entities and interfaces

Impacted functions:

UE: to be able to participate in Slice-Specific Secondary Authentication and Authorisation per S-NSSAI. Indicating UE's security capability to support the Slice-Specific Secondary Authentication and Authorisation.

AMF: triggering of Secondary Authentication and Authorisation per slice and ability to store outcome and any list of S-NSSAIs sharing the same authorised User ID. The Change of AMF happens to match the allowed NSSAI may happen after evaluation of the Secondary Authorisation Status. This happens after the UE is detected to be authorised for 3GPP access in the source AMF. The AMF takes into account of UE's security capability to trigger the Slice-Specific Secondary Authentication and Authorisation towards the UE.

UDM: Slice-Specific Support of Secondary Authentication info.

#### 6.3.2.5	Evaluation

This solution  describes slice-based authentication/authorisation when UE requests registration to the associated S-NSSAI (e.g. during Registration procedure). The network (e.g. AMF) determines if the requested S-NSSAI(s) are subject to Slice-Specific Secondary Authentication and Authorisation. It allows for upfront determination whether the requested S-NSSAI is allowed to be used, and thus, the network grants Allowed NSSAI to the UE before any PDU session is established.

Considering that a Serving AMF is part of the used network slices (i.e. slices part of the Allowed NSSAI), this solution allows to selection a serving AMF after the successful slice-based authentication/authorisation.

There is an open issue that requires CT1 interaction in normative phase.

***********************END of PROPOSED TEXT***************************