SA WG2 Meeting #129bis	S2-1811790

November 26 - 30, 2018, West Palm Beach, USA	(revision of S2-18xxxx)

Source:	NICT

Title:	FS_Vertical_LAN: Proposed conclusion options for solution to key issues 1 and 2 for stand-alone NPN

Document for:	Discussion

Agenda Item:	6.15: 5GS Enhanced support of Vertical and LAN Services

Work Item / Release:	FS_Vertical_LAN/Rel-16

Abstract of the contribution:

This document proposes to select Solution 3 or Solution 1 as a baseline solution to key issues 1 and 2 for stand-alone NPN.

# Discussion

As analysed in S2-181xxyy presented during Nov 13 conference call, for the purpose of network identification, discovery and selection in case of stand-alone NPN:

- Solution 1 proposes to broadcast a combination of PLMN ID and NPN ID

  - One cell can broadcast multiple NPN IDs

- Solution 3 proposes to broadcast a combination of PLMN ID, NPN ID, and SP ID

  - One cell can broadcast multiple SP IDs.

Also, in both solution 1 and 3 network selection is up to UE implementation.

As was mentioned in S2-1810551:

- Different users could be subscribed to different services. For network selection in addition to non-public network identification an information of available services may be required to make decision on network selection for both automatic and manual network selection.

Looks like this could be taken into account for different set of services by using:

- Different NPN IDs in the same cell in case of Solution 1

- Using explicitly different SP IDs in case of Solution 3.

Proposal:

It is proposed to select Solution 3 or Solution 1 as a baseline solution to key issues 1 and 2 for stand-alone NPNs.