SA WG2 Meeting #129BIS	S2-1811801

November 26 - 30, 2018, West Palm Beach, USA

Source:	China Mobile, Huawei

Title:	        Evaluation of solutions for 5G URLLC KI #1

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC

Abstract of the contribution: This contribution proposes evaluations of solutions and conclusions for KI#1.

1 Discussion

The following contribution discusses the scenarios behinds solutions of KI#1 and presents an overall evaluation to the solutions. Based on the evaluation conclusions for KI #1 are proposed.

2 Proposal

It is proposed to make the following changes to the TR 23.725.

* * * * Start of Changes * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.X Evaluation of KI#1

It is worth noting that the solutions proposed to KI #, solutions #1, #2, #3, #4, #7 and #10 are based on different assumptions and therefore do not necessarily address identical scenarios.

Solution #1, #2 and #10 are only valid if DN and UE application layer support traffic replication and elimination functions (e.g. TSN FRER mechanism defined in IEEE 802.1CB[x]) ;

Solution #3 and #4 are valid on the generic cases and has no dependency on the DN/application mechanism. Therein solution #3 assumes that reliability is not enough on both single RAN node and N3/N9 interface, while solution #4 assume that reliability is not enough on N3/N9 interface only.

Solutions #7 can be considered as complementary to solution #1(but with single UPF), #3 and #4 by allowing the UPF to detect the possible replication in DN then determine whether replication is needed or not in 3GPP domain.

| Scenarios | Solutions |
| --- | --- |
| DN/Application layer of UE supports redundant transmission mechanism | Solution #1,#2,#10 |
| DN/Application layer of UE doesn’t necessarily support redundant transmission mechanism | Solution #3,#4,#7 |

The following table shows special impacts/assumptions of the above solutions to different NFs including UE, RAN, CN and DN.

| Solution | Main impacts to RAN | Main impacts to Core | Main impacts to UE | Assumption on Application/DN |
| --- | --- | --- | --- | --- |
| #1 | - M-RAN switches the second session to S-RAN based on instruction from SMF. | - SMF indicates M-RAN whether a session needs to switch to S-RAN during PDU session setup. - SMF/UPF selection to ensure redundant PDU sessions can be served by different SMF and UPF. | - UE triggers the setup of redundant PDU sessions. - Support for TSN FRER in UE application layer. | - Support for TSN FRER at application layer. - Ability to indicate the redundant TSN streams to the 3GPP system so that they can be transferred via different UPFs? |
| #2 | - RG broadcasting - Target RAN selection based on RG match to UE RG during HO | - RG based duplication network deployment, each RG has a full disjoint CN. | - RG based RAN selection. - Support for TSN FRER in device level. | - same as solution #1 |
| #3 | - M-RAN sets up two N3 tunnels for a QoS Flow towards M-RAN and S-RAN separately. - For option 2, RAN maps or reuses SN in GTP-U to PDCP SN and vice versa. | - UPF supports duplication/elimination in GTP-U or HRP layer. | - UE supports duplication/elimination in PDCP or HRP layer. | None |
| #4 | - RAN set up two tunnels for a QoS Flows. - RAN supports duplication/elimination in GTP-U layer. | - Same as solution #3 | None | None |
| #10 | Same as solution #2 | Same as solution #2 | Same as solution #2, but the UEs with the device need to negotiate with each other to ensure they will access to RAN nodes of different RGs. | Same as #2 |

For scenario 1, i.e. in case the DN/Application layer of UE supports redundant transmission mechanism, we have 3 alternative solutions: 1#, 2# and 10#. Based on the above comparisons of the solutions, Solution #1 has less modifications to the RAN, CN and UE, while solution #2 and #10 will introduce requirements on e.g. fully disjointed network deployment for each RG, and inter-UE coordination which is out of scope of 3GPP. Hence it is proposed to take solution #1 as basis for scenario 1.

For scenario 2, i.e. in case the DN/Application layer of UE doesn’t necessarily support redundant transmission mechanism, we have 3 alternative solutions: 3#, 4# and 7#. Furthermore, solution 3# and 4# depend upon whether a single RAN node can satisfy the reliability requirement of the services, but the behaviour of SMF/UPF is the same, except for RAN terminated the duplication/elimination in solution 4# but relay the packet in solution 3#. Solution 7# probably further consider UPF can detect the possible duplicated packets from DN then reuse them instead of performing duplication by itself. Hence it is proposed to take solution 3# as basis for scenario 2, and integrate RAN behaviour in solution 4# and UPF redundant detection behaviour of solution 7# as optimization.

8	Conclusions

8.X Conclusions for KI#1

For the sake of two different deployment cases, it is concluded to specify two solutions for two different scenarios in for normative work in the Rel-16:

For the scenario that DN/Application layer of UE supports redundant transmission mechanism (i.e. TSN FRER defined in IEEE 802.1CB [x]) and also IP type traffic is not applicable, it is recommended that the principle of solution #1 be used for normative work.

For the scenario that DN/Application layer of UE doesn’t necessarily support redundant transmission mechanism, it is recommended that the principle of solution #3 be used for normative work, and RAN behaviour in solution 4# and UPF redundant detection behaviour of solution 7# be considered as optimization.

* * * End of Change * * *