SA WG2 Meeting #129-bis	S2-1812333

26 – 30 November, West Palm Beach, FL, US	(revision of S2-18xxxx)

Source:	Vodafone

Title:	Update of solution 4

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: This pCR proposes an update of solution #4 for FS_UDICOM

# 1 Proposal

Solution #4 is addressing procedures for the scenario in which N26 interface is supported between EPC and 5GC and this pCR introduces the support of interworking scenarios without N26.

Additionally, it is proposed to introduce a new key for retrieving the SUPI associated to an IMPU that may have been used for a specific IMS interaction.

******************First change***********************

### 6.4.1	Introduction

This solution applies to Key Issue 2 and describes how HSS FE and UDM interact when data repositories are common.

******************Next change***********************

#### 6.4.3.4	Handover procedures

##### 6.4.3.4.0	General

For handover procedures, three cases are differentiated according to TS 23.501 [6] and TS 23.502 [2]:

- Single registration mode with N26 support

- Single registration mode without N26 support

- Dual registration

For single registration mode with N26 support, any registration/attachment shall convey a cancelation of the location in the source system.

For single registration without N26 support, registration/attachment shall also convey a cancelation of the location in the source system if simultaneous registration of both MME and AMF is not supported, according to TS 23.501 clause 5.17.2.3.1.

NOTE 1: 	Since UDM and HSS FE are split but common repository exists, the support of simultanous registration of MME and AMF should be configurable in UDM, HSS FE and in the proposed Access Data Layer

This solution proposes that, in single registration modes with and without N26, the PGW-C+SMF with corresponding APN/DNN is stored in the common repository, either in a single record or separate ones, and updated/queried by either party in the different handover scenarios. I.e. at handover 5GS-EPS, HSS FE will query from EPS UDR and at EPS-5GS, UDM will query the 5G UDR.

NOTE 2: 	Storing of PGW-C+SMF and APN/DNN information in the same EPS UDR record will preclude the possibility of proper behaviour of dual registration procedures.

This can be achieved as follows:

Figure 6.4.3.4.0-1 Storage of PGW-C+SMF and APN/DNN with UE camping in EPS in single registration mode

While for 5GS, it is stored as following:

Figure 6.4.3.4.0-2 Storage of PGW-C+SMF and APN/DNN with UE camping in 5GS in single registration mode

For handover procedures in dual registration mode, the information needs to be stored in different records of the EPS UDR. The UE will remain registered/attached to the two system simultaneously with valid subscription data for both systems at the same time. Moving sessions between systems remains as a UE implementation option.

##### 6.4.3.4.1	5GS to EPS with N26

In 5GS to EPS handovers, the attach in the EPS will generate a Ud Request to modify data by HSS FE followed by a Response by the repository and a notification to the Data Access Layer. This will trigger a notification to the UDM to indicate the UE is now under EPS control. UDM should in turn modify the location data in the 5G profile and will proceed to deregister the AMF the UE moved from.

Figure 6.4.3.4.1-1 Notification to the UDM at UE attachment to EPS

******************Next change***********************

##### 6.4.3.4.2	EPS to 5GS with N26

In EPS to 5GS handovers, the registration in the 5GS will generate a Nudr request and subsequent notification when data is stored in the common repository. The Data Access Layer will capture that and generate a notification towards the HSS FE indicating the UE has left the EPS.

Figure 6.4.3.4.2-1 Notification to the HSS FE at UE registration in 5GS

NOTE:	HSS FE subscribed to changes in the common repository when UE data was stored.

The procedure follows the specification in TS 23.502 [2] clause 4.11.1.2.2 with the modification that it is the HSS FE the one initiating the relevant cancel location towards the MME node the UE has moved from.

##### 6.4.3.4.3	5GS to EPS without N26 – single registration

For single registration without N26, HSS FE will be configured as not supporting dual registration. Thus, the attach to the EPS when UE moves from 5GS to EPS, will imply a cancelation of location in the AMF from where the UE moved.

This procedure is aligned with what is described in clause 6.4.3.4.1 of this solution.

The EPS attach is expected to trigger a Ud Update operation towards the EPS UDR, and a response to HSS FE including the PGW-C+SMF address (together with APN/DNN information) being used.

##### 6.4.3.4.4	EPS to 5GS without N26 – single registration

For single registration without N26, UDM will be configured as not supporting dual registration. Thus, the registration to the 5GS when UE moves from EPS to 5GS, will imply a cancelation of location in the MME from where the UE moved.

It must be noticed that the current procedure in TS 23.502 [2], step 5 in clause 4.11.2.3, mandates the AMF to send an Update Location Request to the combo UDM+HSS, forcing the AMF to implement additional logic and interfaces that are used for just this scenario.

For that reason, two alternatives are considered in this handover scenario:

Alternative 1: At registration in the 5GS, the UE signals it is moving from EPS (mobility registration update) and indicates support of "handover" for PDN connectivity. Since the AMF is configured to support interworking without N26, it can register to the UDM (Nudm_UECM_Registration) followed by a query of SMF data/intersystem information by means of Nudm_SDM_Get in order to proceed with continuity according to the UE request.

The following figure shows the complete modified procedure:

Figure 6.4.3.4.4-1 EPS to 5GS without N26 – single registration

NOTE:	The result of the operation Nudm_SDM_Get needs to indicate whether UDM (and HSS FE) support simultaneous registration, in order to inform the UE that “interworking without N26 is supported” in step 7.

The registration in the 5GS (step 5 in the previous figure), would trigger a notification to the Access Data Layer that, in turn, would imply a Ud notification to the HSS FE to cancel the location in the MME. This procedure is aligned with what is described in clause 6.4.3.4.2 of this solution.

Alternative 2: AMF still makes use of legacy point-to-point interface between AMF and HSS FE in order to retrieve the PGW-C+SMF & APN/DNN used when the UE camped in the EPS.

The way HSS FE retrieves such information is according to legacy Ud interactions between EPS UDR and HSS FE.

******************Next change***********************

##### 6.4.3.4.5	Handovers without N26 – dual registration

For handover procedures in dual registration mode, the UE will remain registered/attached to the two system simultaneously with valid subscription data for both systems at the same time, including valid session information for both systems. Moving sessions between systems remains as a UE implementation option.

Attachment/Registration of the UE in either system does not convey any cancellation in the source system. This is achieved by analysing the UDM and HSS FE configuration to support or not dual registrations.

#### 6.4.3.5	IMS interaction

##### 6.4.3.5.1	Access to IMS data

******************Next change***********************

##### 6.4.3.5.2	P-CSCF Restoration procedures

The P-CSCF restoration procedure via UDM is an optional mechanism according to TS 23.380 [8].

The following two options are considered regardless the support of N26 and UE mode of operation (single or dual registration).

Option 1: When UDM and HSS FE are split, the preferred P-CSCF restoration procedure would be that performed by detection of the failure by SMF/UPF according to TS 23.380 [8] clause 5.8.3, or using the PCF based P-CSCF restoration as specified in clause 5.8.5 of TS 23.380 [8].

NOTE 1:	The adaptation of IMS to service based architecture, making use of proper virtualisation/cloud mechanisms for e.g.self-healing, high availability, redundancy, etc, would most likely make unnecessary the use of the current restoration procedures.

Option 2: To enable the P-CSCF restoration via either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the S-CSCF to request the relevant P-CSCF restoration mechanism to the relevant entity. This procedure is according to TS 29.228 [10] clause 6.4.

Figure 6.4.3.5.2-1 User identity to HSS FE identity or UDM identity resolution for S-CSCF

NOTE 2:	This assumes that UDM implements the Cx operations SAR/SAA to trigger the P-CSCF restoration. Potential conversion of Diameter Cx operation to an Nudm service operation is out of the scope of this study.

NOTE 3:	It is assumed that S-CSCF is aware of the IMS registration status of the UE prior to request P-CSCF restoration.

Both HSS FE and UDM will check the UE registration status in the corresponding network (either 5GS or EPS) in order to either execute the procedure (according to TS 23.380 [8]) or reject it.

No other impact is expected for restoration.

##### 6.4.3.5.3	Terminating domain selection information for IMS voice

According to TS 23.501 [6], clause 5.16.3.6, the UDM/HSS shall be able to query the serving AMF for T-ADS related information, and AMF shall respond to the query with the relevant information unless the UE is detached:

-	whether or not IMS voice over PS Session is supported in the registration area(s) where the UE is currently registered;

-	whether or not IMS voice over PS Session Supported Indication over non-3GPP access is supported in the WLAN where the UE is currently registered;

-	the time of the last radio contact with the UE; and

-	the current Access Type and RAT type.

Retrieval of T-ADS information is initiated by the IMS AS.

To retrieve the T-ADS information from either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the IMS AS to request the information for access domain selection. This procedure is according to TS 29.328 [11] clause 6.5.

If the UE is attached to IMS via 5GS, the T-ADS information request to the IMS HSS/SLF will end up in a Sh request (UDR/UDA) to UDM, which in turn will invoke the AMF service Namf_MT_ ProvideDomainSelectionInfo as specified in TS 23.501 [6] clause 5.16.3.6 and TS 23.228 [7] annex Y.

To translate IMPU to SUPI, for the cases in which the Sh request is based on IMPU, the 5G UDR can incorporate a new data key (=IMPU) for the Subscription Data data set to be used for the Nudr_DM_Query operation, in order to request identifier translation from IMPU to SUPI.:

The query to the 5G UDR is simply converted by the Access Data Layer into a Ud request to the EPS UDR repository hosting the IMS data, in order to retrieve the IMSI that is associated with the IMPU received in the Sh request.

The request from IMS AS, when the UE is attached to the EPS, should follow the same procedures as defined in TS 23.228 [7] clause 4.2.4a.

NOTE 1:	It is assumed that IMS AS is aware of the IMS registration status of the UE prior to request T-ADS information.

Both UDM and HSS FE will check the registration status in the corresponding network in order to either execute the procedure (according to TS 23.228 [7]) or reject it.

In case of dual registration, UDM and HSS FE shall be configured with indication regarding being the default system for IMS voice. Both will inspect this default indication in order to execute the T-ADS information retrieval procedure, or reject it.

Figure 6.4.3.5.3-1 User identity to HSS FE identity or UDM identity resolution for IMS AS

NOTE 2:	This assumes that UDM implements the Sh operations UDR/UDA to trigger the T-ADS retrieval procedure. Potential conversion of Diameter Sh operation to an Nudm service operation is out of the scope of this study.

### 6.4.4	Impacts on existing services and interfaces

#### 6.4.4.1	General

This solution addresses scenarios in which the operator deployment is already a layered architecture according to TS 23.335 [3] or when user data are stored in an external entity via non standard interfaces, according to TS 23.002 [5], clause 4.1.1. Purely monolithic scenarios, in which the user data are stored within the HSS itself, would need to migrate towards a layered architecture.

The solution is basically an operator choice restricted to the introduction of a Data Access Layer with the capabilities described in clauses 6.4.3.1 to 6.4.3.4, and does not impact the specified interfaces with new operations.

The solution is also based on network configuration, by including SLF functionality for every IMS operation directed towards a location server (either HSS FE, IMS HSS or UDM). The SLF is configured as the contact point for I-CSCF, S-CSCF and IMS AS, and needs to be provisioned with the list of location servers serving the UEs in the network.

The solution proposes to add information regarding the system registration status, i.e. the system serving the UE:

-	For 5G profile, this extension can be handled as the resources Amf3GppAccessRegistration and AmfNon3GppAccessRegistration in the Nudr_DM_Update service operation as specified in TS 29.505 [9].

-	For 4G profile, this extension can be mapped to the repository field where MME reference is held. Since the organisation of data in the EPS UDR is not standardised, this mapping would be left for implementation.

Finally, the solution proposes adding the Sh pair UDR/UDA (Diameter) to UDM as a temporary solution until IMS is adapted to service based architecture or any other solution is specified for T-ADS information retrieval by IMS.

It also proposes to add a new data key (=IMPU) to the Subscription Data data set in the Nudr_DM_Query operation.

#### 6.4.4.2	Summary of impacts on UDM

-	For handovers without N26 support:

Add configuration indication regarding support or not of dual registrations

-	For EPS to 5GS handover without N26 support in single registration mode, alternative 1:

Include a new result in the operation Nudm_SDM_Get to indicate to AMF dual registration supported or not

-	For T-ADS information retrieval: Add Sh pair UDR/UDA

-	For P-CSCF restoration:

For Option 1, no impact (i.e. make use of PCF based restoration or detection of failure by SMF/UPF)

For Option 2: Add Cx pair SAR/SAA for P-CSCF restoration

#### 6.4.4.3	Summary of impacts on HSS FE

No impact

For handovers without N26 support:

- Add configuration indication regarding support or not of dual registrations

#### 6.4.4.4	Summary of impacts on EPS UDR

The impacts on the EPS UDR are left for implementation, since the protocols to access and the organisation of data is implementation specific.

#### 6.4.4.5	Other impacts

For EPS to 5GS handover without N26 support in single registration mode, alternative 1, AMF should be updated in order to make use of 5GC procedures based on SBA/SBI (instead of using p2p interfaces towards legacy systems).

******************End of changes***********************