SA WG2 Meeting #129	S2- 1810178

October 15 - 19, 2018, Dongguan, P. R. China

Source:	Sprint, AT&T, Deutsche Telekom, Vodafone

Title:	Conclusions for key issue 4 – Stateless NF Services

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes a set of conclusions for key issue 4 of the Study on Enhancements to the Service-Based 5G System Architecture, TR 23.742.

# 1 Introduction

This document proposes a set of conclusions for key issue 4.

# 2 Proposal

This document proposes a set of conclusions for key issue 4. It addresses only the aspects rendering NF Services as stateless, i.e., separating functional processing from state repository. The conclusions gather together principles to agree upon that already exist in the TR as a way to construct a solution from drawing upon portions of several proposed solutions.

* * * Start of Changes * * *

# 8	Conclusions

Normative solution(s) for Key Issue #4 shall achieve stateless NF Services by adhering to the following principles:

- NF Services shall be rendered stateless by separating functional processing from a context repository. The context repository shall support a standardised data service that can be used by all NF Services for storing and retrieving service context data.

  - The service context data information shall be standardised.

- The context repository shall support service context information sharing between NF Services enabling multivendor interoperability between NF services.

  - Relevant state information of finalised transactions shall be pushed to the context repository and made available to other NF Services which require the specific data for further processing.

- The context repository shall support standardised service context information necessary for the recovery of failed instances of the same NF service to achieve multi-vendor deployments of instances of the same NF service.

* * * End of Changes * * *