SA WG2 Meeting #130	S2-1900067

21 – 25 January, Kochi, India	(revision of S2-19xxxx)

Source:	Vodafone

Title:	Evaluation of solutions 4 and 5

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: This pCR proposes evaluation of solutions #4 and #5 for FS_UDICOM

# 1 Proposal

This pCR proposes evaluation for solutions #4 and #5.

******************First change***********************

### 6.4.5	Evaluation

Solution 4 proposes an architecture that does not require direct interaction between the UDM and the HSS FE.

The solution makes use of an adaptation layer, named Access Data Layer, that separates the applications front end logic from database technology/protocol in the back end repositories.

The solution addresses all interworking scenarios depicted in TS 23.501 [6] and TS 23.502 [2], including support for IMS procedures.

The advantages of this solution is that:

-	It addresses effective separation of HSS FE and UDM to enable multivendor deployments

-	It avoids new interface and new operations specification between the HSS and UDM

-	It is possible to introduce only UDM when deploying a 5GS system, without having to update/upgrade the HSS FE, or having to introduce this functionality in deployments with single core network.

-	Standardisation effort is minimised and restricted to potential incorporation of Sh/Cx operations for UDM, in case FS_eIMS5G [14] study does not incorporate the changes needed for P-CSCF restoration and T-ADS information retrieval.

-	Introduction of this solution does not impact other network functions that may be accessing the different repositories, while it provides the means to further split other network functions defined in rel 15 (i.e PCF/PCRF).

The disadvantage of this solution is to support proprietary Ud implementations and making use of SLF functionality for IMS when some deployments may not be using it.

******************Next change***********************

### 6.5.5	Evaluation

Solution 5 proposes an architecture that does not require direct interaction between the UDM and the HSS FE or interaction between EPS UDR and 5G UDR.

The solution makes use of an adaptation layer, named Access Data Layer, that separates the applications front end logic from database technology/protocol in the back end repositories.

The solution addresses all interworking scenarios depicted in TS 23.501 [6] and TS 23.502 [2], including support for IMS procedures.

The advantages of this solution is that:

-	It addresses effective separation of HSS FE and UDM and EPS UDR from 5G UDR to enable multivendor deployments

-	It avoids new interface and new operations specification between the HSS and UDM

-	It is possible to introduce only UDM and 5G UDR when deploying a 5GS system, without having to update/upgrade either HSS FE or the EPS UDR, or having to introduce these functionalities in deployments with single core network.

-	Standardisation effort is minimised and restricted to potential incorporation of Sh/Cx operations for UDM, in case FS_eIMS5G [14] study does not incorporate the changes needed for P-CSCF restoration and T-ADS information retrieval.

-	Introduction of this solution does not impact other network functions that may be accessing the different repositories, while it provides the means to further split other network functions defined in rel 15 (i.e PCF/PCRF).

The disadvantage of this solution is to support proprietary Ud implementations and making use of SLF functionality for IMS when some deployments may not be using it

******************End of changes***********************