3GPP TSG-SA2 Meeting #130	S2-1900861

# Kochi, India, 21st Jan 2019 - 25th Jan 2019			 	(revision S2-1900758 of S2-1900613)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0967 | rev | 2 | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI-#5 specific AF influence on traffic routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are deployment scenarios studied and concluded in ETSUN TR 23.726 where a signle SMF may not always control all the UPFs, and thus requiring addition, modification or removal of an intermediate SMF or I-SMF. Agreed solutions to address these cases were to introduce a new interface between SMF and I-SMF, with required functionalities including those related to Handling AF influence on traffic routing. Corresponding changes were discussed and solution #15 for KI#5 was agreed and included in ETSUN TR 23.726. These needs to be captured in clause 4.3.6 of TS 23.502. Below changes are made in Ver-01, based on the received comments: - Changed the clause-structure i.e. instead of under 4.3.6, move them to sub-clause 4.x.y - Keep the figure more general e.g. remove AF in the flow and focus on interface 16a - Add a note for further details addition to 16a interface - Included NTT DOCOMO as source company: this is merged tdoc Below changes made in Ver-02, to capture received comments: - Modified clause titles - Added “Step” in the 4.x.y.1 procedure 4 - Added “DNAI list reported by the I-SMF” in step 3, 4.x.y.1 - Modified figure 4.x.y-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional text/steps related to Handling AF influence on traffic routing, those agreed in ETSUN TR 23.726, specific to KI#5, solution #15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ETSUN conclusions related to KI#5 not captured in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2, 4.3.6.3, 4.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** 1st Change *****

4.x.y	I-SMF Related Procedures with PCF

This clause provides PCC related details for scenarios including an I-SMF.

4.x.y.1 Policy Update Procedures with I-SMF

Below figure 4.x.y-1 shows procedures related to provisioning of PCC rules containing traffic steering information related to an I-SMF.

Figure 4.x.y-1: Policy Update procedure

In cases where step 1a in figure 4.x.y-1 is triggered in response to PCF receiving AF request, below steps 3 and 4 are applicable, in addition to those steps as explained in 4.3.6.1.

Step 3:   SMF provides to I-SMF information based on PCC rules received from PCF and DNAI list reported by the I-SMF. Theses information allows I-SMF in e.g. traffic routing related decisions, usage reporting, I-UPF selection, QoS enforcement to support scenarios with UL-CL/BP controlled by I-SMF.

Step 4: I-SMF may take appropriate actions to reconfigure the User plane in its service area of the PDU Session such as:

-	Adding, replacing or removing a UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.

-	Updating the UPF in the target DNAI with new traffic steering rules

Subscribe to notifications from the AMF for an Area Of Interest via Namf_EventExposure_Subscribe service operation

4.x.y.2  I-SMF Notification to SMF

Below figure 4.x.y-2 shows procedures related to I-SMF providing event notification to SMF.

Figure 4.x.y-2: I-SMF specific event Notification

Clause 4.3.6.3 provides details on notification of user plane management events. In case AF has subscribed for user plane management event notifications, then steps 2a and 4a in figure 4.x.y-2 are applicable as below for early and late notifications, respectively:

1a. Condition for an AF notification, as described in 4.3.6.2, are met. I-SMF sends notification to the SMF as shown in step 2a.

2a. If early notification is requested by SMF on be half of AF, as described in 4.3.6.2, then I-SMF notifies the SMF of user plane management events by invoking Nsmf_EventExposure_Notify service operation. SMF then includes these as inputs to its notification to NEF/AF in step 2b.

3.  I-SMF enforces the change of DNAI or addition, change, or removal of UPF.

4a. In case of late notification I-SMF sends to the SMF of user plane management events e.g. newly added UL CL and I-UPF details by invoking Nsmf_EventExposure_Notify service operation. SMF then includes these as inputs to its notification to NEF/AF in step 4b.

***** End of Changes *****