3GPP TSG-SA WG2 Meeting #130	S2-1900953

21 – 25 January 2019, Kochi, India	(revision of S2-1900079)

Source:	KDDI, TOYOTA ITC

Title:	7.4 NWADF-assisted Background Data Transfer

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to add the new featurture related to NWDAF-assited Background Data Transfer into TS 23.288.

- Discussion

As discussed and studied in TR 23.791, Solution 18 and 28 are selected as the concluded solution for NWDAF-Assisted Future Background Data Transfer. This PCR proposes to add the new feature about NWDAF-Assisted Future Background Data Transfer into TS 23.288 as an implementation.

Here, we propose that the same information as NWDAF input and output is used for the features based on Solution 18 and 28, respectively.

- Proposal

The following changes are proposed in TS 23.288.

* * * Start of Change (all text is new)  * * *

7.4	Future Background Data Transfer related network data analytics

7.4.1	General

The NWDAF shall provide the analytics related to the network condition in order to help the PCF in deciding the transfer policy for the background data transfer. .

The NWDAF shall also provide the analytics related the network condition in order to help the PCF in judging whether the background data traffic is impacted.

When the PCF sends this, the AF may try to negotiate with the MNO regarding the transfer policy again. The network condition might be changed, after the transfer policy is agreed between MNO and ASP.

7.4.2	Input Data

The UE level information related to background data transfer is defined in Table 7.4.2-1 in order that the NWDAF /provides the  analytics.

There are 2 variants of the information related to the UE location:

- AF knowledge of UE area: in this variant the AF provides the target area where Future Background Data Transfer is expected. This variant focuses on the fact that the AF has a better knowledge than the network on where the UE(s) should be at the expected time of the background data transfer.

- NWDAF prediction of the UE area: in this variant the AF does not provide the target area where Future Background Data Transfer is expected but the NWDAF has to guess it based on:

‐	the knowledge of the list of UE corresponding to the AF transfer. To avoid AF providing huge lists of UE(s) it is assumed that all these UE are configured to belong to a group and the AF only refers to the Group ID.

‐	predictions on UE mobility: NWDAF derives the UE Moving Trajectory by collecting the UE level information such as location information from AMF.

Table 7.4.2-1: NWDAF input data

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| UE ID or Internal group Identifier | C |  | Present if NWDAF requests location information |
| Location info |  |  |  |
| >Timestamp | C | AMF | The age of the information |
| >Location Info | C | AMF | The location info for the UE e.g. Cell ID or TA ID |
| Network performance info |  |  | It is collected from OAM |
| Area information | C | OAM | Present if NWDAF requests network performance information |
| Load information | C |  | Statistics on load in the area. |

7.4.3	Output Analytics

Table 7.4.3-1 and 7.4.3-2 show the event id and the analytics information output by the NWDAF based on the input information described in Table 7.4.2-1.

Table 7.4.3-1: Event ID to provide analytics for background data transfer

| Event ID | Event Filter | Description |
| --- | --- | --- |
| Network Condition list | List of UE identities, Time, Date, Network area | Provides the network condition list in the area where the ASP will send background data traffic. |

Table 7.4.3-2: Content of Network Condition List

| IE/Group Name | Presence | Range | Semantics description |
| --- | --- | --- | --- |
| Network Condition list |  | 1 |  |
| > Network Condition IEs |  | 1 to M |  |
| >>list of UE IDs | C |  |  |
| >> Expected Moving Trajectory | C | 1 to N | UE´s expected geographical movement (e.g. as described in TS 23.502 [3] clause 14.5.6.3). |
| >>Network Performance Info | C |  | Performance information on the NF where the UE is expected to be according to the expected moving trajectory |
| >>Time and Date | C |  |  |

7.4.3	Procedure

Editor’s note: Add new procedures that are not listed in 23.502and  reference 23.502 when existing procedures are used.

7.4.3.1	Negotiation for Future Background Data Transfer

The PCF may subscribe to the NWDAF or periodically requests a network condition list for an ASP based on AF request. Through the negotiations for future background data transfer in clause 4.16.7 TS 23.502 [x], the PCF determines one or more transfer policies, based on information provided by the AF and the information from the NWDAF.

7.4.3.2	Notification to AF  for Future Background Data Transfer

The AF in the ASP can subscribe the BDT warning notification to the PCF through the NEF. Then, the PCF subscribes to the NWDAF or periodically requests a network condition list for an ASP based on AF request. When the PCF judges that the background data traffic is impacted based on the information from the NWDAF,  the PCF sends the BDT warning notification described in Tables 7.4.3.2-1 and 7.4.3.2-2 to the AF through the NEF.

Editor’s note Wthether this call flow is put in TS 23.502, or TS 23.288 is FFS.

- The AF subscribes to or cancels subscription to BDTNotification by invoking the Nnef_BDTNotification_Subscribe/ Nnef_BDTNotification_Unsubscribe service operation.

- The NEF subscribes to or cancels subscription to BDTNofication by invoking the Npcf_BDTNotification_Subscribe/ Npcf_BDTNotification_Unsubscribe service operation.

- The PCF subscribes to or cancels subscription to analytic information for background data transfer by invoking the Nnwdaf_EventsSubscription_Subscribe/Unsubscribe.

- The NWDAF notifies the PCF with the analytic information by invoking Nnwdaf_EventsSubscription_Notify service operation.

- The PCF judges whether that the background data traffic is impacted or not.

- When the PCF judges that the background data traffic is impacted,  the PCF notifies the NEF with BDT notification by invoking Npcf_BDTNotification_Notify service operation.

- The NEF notifies the AF with BDT notification by invoking Nnef_BDTNotification_Notify service operation.

Table 7.4.3.2-1-4: BDT warning notification to renegotiate the BDT policy

| Event ID | Event Filter | Description |
| --- | --- | --- |
| BDT warning notification | ASP identifier, Time, Date, Network area information | PCF requests the ASP to renegotiate the BDT policy, due to potential delays in the transmission of background data. The ASP may trigger a procedure to negotiate the BDT policy with the MNO. |

Table 7.4.3.2-2: Content of List of BDT warning notification

| IE/Group Name | Presence | Description |
| --- | --- | --- |
| List of BDT warning notificaiton |  |  |
| > IEs |  |  |
| >> ASP identifier | M | Identifier to identify the ASP |
| >>BDT policy reference ID | M | Reference ID of agreed BDT policy |
| >>Time | O | It indicates the time when the background data of the UEs of this ASP may be delayed due to network conditions |
| >>Date | O | It indicates the date when background data of the UEs of this ASP may be delayed due to network conditions |
| >>Network area | O | indicates the network area where background data of the UEs of this ASP may be delayed due to network conditions |
| >>UE ID | O | Indicates the list of UEs whose background data of the UEs of this ASP may be delayed due to network conditions |

* * * End of change * * *