3GPP TSG-SA2 Meeting #131	S2-1901971

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0753 | rev | 6 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General description of solution 1 in 23.725 for user plane redundancy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Impement the conclusion of the FS_URLLC for user plane redundancy for solution 1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add dual connectivity based solution for user plane redundancy. Rev6: - The SMF also indicates to the UE that the PDU Session is to be handled redundantly so the UE can initiate second PDU Session establishment for redundancy towards the same DN - Clarified that based on the RSN indicating that the user plane goes via the Secondary NG-RAN using dual connectivity, the Master NG-RAN sets up dual connectivity. If the NG-RAN cannot setup dual connectivity the NG-RAN shall reject the second PDU session Resource Setup Request. The SMF may reject the establishment of the PDU Session based on local policy |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of FS_URLLC not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, New Clause 5.X.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 0877 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | General 5G URLLC new clause 5.X introduced by CR#810 and this new clause will be a subclause under that |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

2	References

…

[X]		IEEE 802.1CB-2017: IEEE Standard for Local and metropolitan area networks--Frame Replication and Elimination for Reliability.

* * * * Next Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

5G-AN	5G Access Network

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5QI	5G QoS Identifier

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

AUSF	Authentication Server Function

BSF	Binding Support Function

CAPIF	Common API Framework for 3GPP northbound APIs

CP	Control Plane

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

FAR	Forwarding Action Rule

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LRF	Location Retrieval Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

N3IWF	Non-3GPP InterWorking Function

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDR	Packet Detection Rule

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

QFI	QoS Flow Identifier

QoE	Quality of Experience

(R)AN	(Radio) Access Network

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

S-NSSAI	Single Network Slice Selection Assistance Information

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSP	Traffic Steering Policy

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

* * * * Next Change * * * *

5.X.2.1 Dual Connectivity based end to end Redundant User Plane Paths

In order to support highly reliable URLLC services, a UE may set up two redundant PDU Sessions over the 5G network, such that the 5GS sets up the user plane paths of the two redundant PDU sessions to be disjoint. The user’s subscription indicates if user is allowed to have RSN and this indication is provided to SMF from UDM.

NOTE 1: 	It is out of scope of 3GPP how to make use of the duplicate paths for redundant traffic delivery end-to-end. It is possible to rely on upper layer protocols, such as the IEEE TSN (Time Sensitive Networking) FRER (Frame Replication and Elimination for Reliability) [X], to manage the replication and elimination of redundant packets/frames over the duplicate paths which can span both the 3GPP segments and possibly fixed network segments as well.

NOTE 2:	The following redundant network deployment aspects are within the responsibility of the operator and are not subject to 3GPP standardization.

-	RAN supports dual connectivity, and there is sufficient RAN coverage for dual connectivity in the target area.

-	UEs support dual connectivity.

-	The core network UPF deployment is aligned with RAN deployment and supports redundant user plane paths.

-	The underlying transport topology is aligned with the RAN and UPF deployment and supports redundant user plane paths.

-	The physical network topology and geographical distribution of functions also supports the redundant user plane paths to the extent deemed necessary by the operator.

-	The operation of the redundant user plane paths is made sufficiently independent, to the extent deemed necessary by the operator, e.g., independent power supplies.

Figure 5.X.2.1 illustrates dual PDU sessions when RSN is applied. One PDU Session spans from the UE via Master NG-RAN to UPF1 acting as the PDU Session Anchor, and the other PDU Session spans from the UE via Secondary NG-RAN to UPF2 acting as the PDU Session Anchor. Based on these two PDU Sessions, two independent paths are set up. UPF1 and UPF2 connect to the same Data Network (DN), even though the traffic via UPF1 and UPF2 may be routed via different user plane nodes within the DN.

Editor’s Note: How multiple SMFs are supported in case of RSN is FFS.

The redundant user plane set up applies to both IP and Ethernet PDU sessions.

Figure 5.x.2.1-1 Architecture for end to end redundant User Plane paths using Dual Connectivity

During the establishment of the PDU Session:

-	the SMF determines whether the PDU Session is to be handled redundantly. The determination is based on the combination of the S-NSSAI, DNN, user subscription and local policies. The SMF also uses these inputs to determine the RSN which differentiates the PDU Sessions that are handled redundantly and determines whether the PDU Session’s user plane should go via the Master or the Secondary NG-RAN. The SMF also indicates to the UE that the PDU Session is to be handled redundantly so the UE can initiate second PDU Session establishment for redundancy towards the same DN.

-	Operator configuration of UPF selection ensures the appropriate UPF selection for disjoint paths.

-	When the NG-RAN contexts are established for the redundant PDU Sessions (i.e., during the establishment of the PDU Sessions or at transitions to CM-CONNECTED state), for one PDU Session the RSN parameter explicitly requests that the user plane goes via the Master NG-RAN, and for the other PDU Session the RSN parameter explicitly requests that the user plane goes via the Secondary NG-RAN using dual connectivity. This request is made by indicating the RSN to the RAN node on a per PDU Session granularity. Based on the RSN indicating that the user plane goes via the Secondary NG-RAN using dual connectivity, the Master NG-RAN sets up dual connectivity as defined in TS 37.340 [31] so that the sessions have end to end redundant paths. If the NG-RAN cannot setup dual connectivity the NG-RAN shall reject the second PDU session Resource Setup Request. The SMF may reject the establishment of the PDU Session based on local policy.

NOTE 3:	The decision to set up dual connectivity remains in NG-RAN as defined today. NG-RAN takes into account the RSN  provided by the CN.

-	In the case of Ethernet PDU Sessions, the SMF has the possibility to change the UPF (acting as the PSA) and select a new UPF based on the identity of the Secondary RAN for the second PDU Session in case the Secondary RAN is modified (or added/released), using the Ethernet PDU Session Anchor Relocation procedure described in clause 4.3.5.X.

-	The SMF’s charging record may reflect the RSN information.

-	The RSN indication is transferred from Source NG-RAN to Target-NG-RAN.

* * * * Next Change * * * *

6.3.3.3	Selection of an UPF for a particular PDU Session

If there is an existing PDU Session, and the SMF receives another PDU Session request to the same DNN and S-NSSAI, and the UE subscription data indicates the support for interworking with EPC for this DNN and S-NSSAI, the same UPF should be selected. Otherwise, if UE subscription data does not indicate the support for interworking with EPS for this DNN and S-NSSAI, a different UPF may be selected.

For the same DNN and S-NSSAI if different UPF are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only per PDU Session AMBR enforcement applies.

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

-	RSN when redundant handling is applicable.

NOTE:	How the SMF determines information about the user plane network topology from information listed above, and what information is considered by the SMF, is based on operator configuration.

* * * * Next Change * * * *

## 5.11	Support for Dual Connectivity, Multi-Connectivity

### 5.11.1	Support for Dual Connectivity

Dual Connectivity involves two radio network nodes in providing radio resources to a given UE (with active radio bearers), while a single N2 termination point exists for the UE between an AMF and the RAN. The RAN architecture and related functions to support Dual Connectivity is further described in RAN specifications (e.g. TS 37.340 [31]).

The RAN node at which the N2 terminates, performs all necessary N2 related functions such as mobility management, relaying of NAS signalling, etc. and manages the handling of user plane connection (e.g. transfer over N3). It is called the Master RAN Node. It may use resources of another RAN node, the Secondary RAN node, to exchange User Plane traffic of an UE.  Master RAN takes into account the RSN to determine if dual connectivity shall be set up and ensure appropriate bearer handling ensures fully redundant user plane path as described in clause 5.X.2.1.

If the UE has Mobility Restriction (either signalled from the UDM, or, locally generated by VPLMN policy in the AMF) the AMF signals these restrictions to the Master RAN Node as Mobility Restriction List; This may prevent the Master RAN node from setting up a Dual Connectivity for an UE.

NOTE 1:	Subject to policies in the NG-RAN, configuration of Dual Connectivity for a Data Radio Bearer can also be based on the Network Slice that the PDU Session belongs to.

Dual Connectivity provides the possibility for the Master node RAN to request SMF:

-	For some or all PDU Sessions of an UE: Direct all the DL User Plane traffic of the PDU Session to the either the Master RAN Node or to the Secondary RAN Node. In this case, there is a single N3 tunnel termination at the RAN for such PDU Session.

NOTE 2:	The terminating RAN Node, can decide to keep traffic for specific QFI(s) in a PDU Session for a UE on a single RAT, or split them across the two RATs.

-	For some other PDU Sessions of an UE: Direct the DL User Plane traffic of some QoS Flows of the PDU Session to the Secondary (respectively Master) RAN Node while the remaining QoS Flows of the PDU Session are directed to the Master (respectively Secondary) RAN Node. In this case there are, irrespective of the number of QoS Flows, two N3 tunnel terminations at the RAN for such PDU Session.

The Master RAN may create and change this assignment for the user plane of a PDU Session at any time during the life time of the PDU Session;

In both cases, a single PDU Session Id is used to identify the PDU Session.

Additional functional characteristics are:

-	User location information reporting is based on the identity of the cell that is serving the UE in the Master RAN node.

-	Path update signalling related with Dual Connectivity and UPF re-allocation cannot occur at the same time.

* * * * End of Change * * * *