SA WG2 Meeting #131	S2-1902595

25 Feb – 1 Mar 2019, Tenerife, Spain	(was S2-1901920)

Source:	Samsung

Title:	Procedures for Nnrf service operation

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_eV2XARC / Rel-16

Abstract of the contribution: This paper proposes to address additional operation for Nnrf_NFDiscovery service.

# 1	Introduction

According to solution 3 which has been concluded for normative work, the AMF needs to discover the PCF for both V2X and non-Session based PCC, in order to avoid multiple PCC association for the UE. In the solution, two mechanisms have been addressed, 1) using local configuration in the AMF, and 2) utilizing NRF service.

In this paper, it is proposed to capture PCF discovery when the AMF utilizes Nnrf_NFDiscovery service. If the requested S-NSSAI is for V2X service and it is authorized to the UE, the AMF includes the S-NSSAI for V2X service in Nnrf_NFDiscovery_request message. Alternatively, the AMF may include V2X support indication in the Nnrf_NFDiscovery_Request message. Therefore, the NRF can return the appropriate output (e.g., PCF instance that support both V2X and non-Session based PCC) to the AMF.

In the revision, second bullet has been removed. (Not shown in the revision)

In addition, it has been clarified how AMF determines to include V2X support indication to the NRF. It should be based on the indication from the UE during registration procedure, and/or UE subscription information.

# 2	Text Proposal

It is suggested to include the following changes into TS 23.287.

*************** Beginning of Changes **************

### 6.2.X	Nnrf_NFDiscovery service

In order to discover the PCF for both V2X and non-Session based PCC, the AMF may utilize the Nnrf_NFDiscovery service as defined in TS 23.502 [7] with the following additions:

-	Based on the indication from the UE and/or UE subscription data during the Registration procedure, the AMF may include V2X support indication in the Nnrf_NFDiscovery_Request message as optional input parameter. If provided, the NRF takes the information into account for discovering the PCF.