3GPP TSG-SA2 Meeting #132	S2-1903086

# Xi'an, China, 8th Apr 2019 - 12th Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.786 | CR | 0001 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conclusion of KI#15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eV2XARC |  |  |  |  |  | Date: |  |  | 2019-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Conclusions for KI#15 missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes to proceed with Solution 29 as basis for normative work |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#15 to be postponed to a future release |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

================FIRST change====================

## 7.2	Conclusions for 5G System

For the architectural reference model it is concluded to take Alternative #1 in Annex A.1 as the baseline for normative work.

For Key Issue #1 (Support of eV2X Group Communication), it is concluded that Solution #21 documented in clause 6.21 is used as the baseline for normative work.

For Key Issue #2 (3GPP PC5 RAT selection for a V2X application), it is concluded that Solution #12 documented in clause 6.12 is selected for normative work.

For Key Issue #3 (QoS Support for eV2X over Uu interface),

-	Regarding QoS characteristics and new standardized 5QI value, it is concluded to take Solution #2 documented in clause 6.2 as the baseline for normative work. No additional QoS parameters have been identified in addition to the ones already specified in TS 23.501 [7]. Whether the new combinations of QoS parameter values proposed in Solution #2 are supported or not is pending RAN WG2's feedback.

-	Regarding QoS Notification Control enhancements, it is concluded that Solution #27 documented in clause 6.27 is taken as the baseline for normative work.

NOTE 0:	As RAN WGs had been requested to evaluate the RAN impacts, solution#27 may be updated and aligned with RAN WG feedbacks in the normative phase.

For Key Issue #4 (Support of PC5 QoS framework enhancement for eV2X), depends on RAN WG decisions, it is concluded that Solution #19 documented in clause 6.19 is used as the baseline for normative work, with the working assumption that VQI can be supported by NR PC5, and a new set of VQIs would be defined in normative phase.

For Key Issue #5 (Service Authorization and Provisioning to UE for eV2X communications over PC5 reference point) and Key Issue #11 (Service Authorization and Provisioning to UE over NG-Uu reference point), it is concluded that Solution #3 documented in clause 6.3, Solution #20 documented in clause 6.20 and Solution #24 documented in clause 6.24 are selected for normative work.

For Key Issue #6 (Service Authorization to NG-RAN for eV2X communications over PC5 reference point), Solution #6 documented in clause 6.6 is selected for the normative work.

NOTE 1:	For V2X capability indication and V2X related information per PC5 RAT, alignment based on progress/decision in RAN WGs will be performed as part of the normative phase.

For Key Issue #7 (Network Slicing for eV2X Services),

-	To facilitate deployment of dedicated network slice for use of, for example, automotive industry and to facilitate roaming support, it is concluded to reuse the Network Slicing functionality for 5GS (see TS 23.501 [7], TS 23.502 [9]) with specifying a new standardized SST value dedicated for V2X services.

-	Existing values (both standardized and non-standardized SST) defined in TS 23.501 [7] can also be used for any V2X services e.g. eMBB, URLLC, etc.

For Key Issue #8 (Support of edge computing), it is concluded that the mechanisms to support edge computing defined in TS 23.501 [7] and TS 23.502 [9] can be used. No additional normative work has been identified for this key issue. Potential normative work may be identified by FS_5G_URLLC for this key issue.

For Key Issue #9 (Support of unicast/multicast for sensor sharing over PC5), it is concluded that Solution #11 documented in clause 6.11 is adopted as the baseline for normative work, with the following potential updates based on other Working Groups' feedbacks:

-	the signalling message definition for unicast link establishment and management, e.g. if and how RRC signalling is used for unicast link;

-	the choice of per packet QoS model or bearer based QoS model for broadcast, groupcast, and unicast based on RAN decisions;

-	signal to the base station regarding the service used when network scheduled mode is used;

-	the potential security related procedure updates for unicast communication over PC5.

NOTE 2:	In Key Issue #9 and Solution #11, multicast means groupcast.

For Key Issue #10 (eV2X message transmission and reception) on the support of non-IP based V2X message over NG-Uu reference point, it is concluded to take Solution #18 documented in clause 6.18 for normative work.

For Key Issue #12 (System migration and interworking for eV2X), it is concluded to take Solution #9 documented in clause 6.9 and Solution #20 documented in clause 6.20 for normative work. PC5 interworking can be supported with proper UE configurations by taking into account e.g. regional regulations, deployments, and UE PC5 RAT capability.

For Key Issue #14 (Support of broadcast over NG-Uu), it is concluded to stop further solution work on 5GS MBMS for Release 16. No normative work shall be performed for this key issue in Release 16.

For Key Issue #15 (Enhancements to assist Application Adjustment), it is concluded to proceed by developing normative specification based on Solution #29 documented in clause 6.29, i.e. the NWDAF produces a service allowing consumers to be notified in advance of potential QoS change.

================End of changes ====================