3GPP TSG-SA WG2 Meeting #132	S2-1903099

8 - 12 Apr 2019, Xi'an, China

Source:	ORANGE

Title:	Abnormal behaviour related network data analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes a additional clauses on request parameters, data collection, outputs and procedures for NWDAF abnormal behaviour detection.

# Discussion

This contribution defines additional proposals on request parameters, data collection, outputs and the prcedures for abnormal behaviour detection.

- Parameters containing the risks IDs and the expected normal behaviour

- Reuse of the data collection performed in § 6.7

- Definition of the outputs as a table of risk levels

- Procedures

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.8 Abnormal behaviour related network data analytics

### 6.8.1	General

The clause 6.8 defines how NWDAF identifies a group of UEs or a specific UE being misused or hijacked based on the expected UE behavioural parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE.

The service consumer may be a 5GC NF (e.g. AMF, UDM, PCF), an AF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	AnalyticsId set to "Abnormal behaviour”

-	The Target of Analytic Reporting can be one UE, any UE or an Internal Group Identifier.

-	optionaly an Application ID, DNN or S-NSSAI

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Analytic Filter Information: optional expected behaviour, list of risk IDs to monitor with associated thresholds

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.8.3.

For instance, if PCF subscribes the notification, the NWDAF sends the PCF a notification about the risk or suspicious events, which triggers the PCF to update the AM/SM policies. The NWDAF can also send the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF can, based on operator configurable policies, adopt pre-defined anti-risk mechanisms.

### 6.8.2	Input Data

The expected UE behavioural parameters that are provided to the NWDAF are defined in Table 6.7.3-1 and 7.6.3-2.

The UE behavioural information collected from 5GC NFs refers to the Table 4.15.3.1-1 in TS 23.502 [3].

The NWDAF may help to build the expected behaviour, by producing behaviour analytics as stated in 6.7.

On request of the service consumer, the NWDAF shall collect and analyse the behaviour analytics from the 5GC NFs (SMF, AMF) or AF.

### 6.8.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to exposure the abnormal behaviour information together with a UE ID list from the NWDAF to the consumer NF (e.g. PCF or AF), indicating e.g. the UEs are misused or hijacked.

NWDAF detects some UEs with serious misbehaviours, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF determines whether to notify or alert the related 5GC NFs or application layer based on the input parameters defined in 6.8.1.

Table 6.8.3-1:  Output analytics produced by the NWDAF

| Information | Source | Description |
| --- | --- | --- |
| UE identification |  |  |
| List of risk analytics (1..n) |  |  |
| > Risk ID | AMF/SMF | The risk ID as per table 6.8.3-1 |
| > Risk level | AMF/SMF | Measured level, compared to the threshold |
| > Risk trend | AMF/SMF | Measured trend (up/down/unknown/stable) |
| > Additional measurement | AMF/SMF | Specific information for each risk |

The following table gives examples of risk, policies and actions for risk solving. The exact policies and actions to solve specific risks depend on operator's configurations

Table 6.8.3-1: Examples of policies and actions for risk solving

| Risk ID and description | AM/SM policy | Actions of NFs |
| --- | --- | --- |
| Unexpected UE location | Add the area of current UE location into mobility restriction | AMF Applies mobility restriction |
| Unexpected long-live/large rate flows | Decrease the MBR for the related QoS flow | SMF updates the QoS rule |
| Unexpected wakeup | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Release the PDU session and Apply SM back-off timer | SMF releases the PDU session and applies SM back-off timer |
| Wrong destination address | Update the packet filter of the related QoS flow to block the wrong SDF | SMF updates the packet filter of the related QoS flow and configures the UPF |
| Ping-pong stationary UE | NWDAF notifies the AMF or AF (Service Provider) | AMF may adjust UE registration area. AF (Service Provider) may adjusts UE location. |

### 6.8.3	Procedures

The NWDAF may receive from the consumer NF the expected behavioural information from the consumer NF, or may retrieve it from the UDM.

The NWDAF may receive detection thresholds from the consumer NF, or use intenal detection thresholds, both configurable by the operator.

The NWDAF shall apply internally the procedure described in 6.7 in order to collect the measured behaviour.

The NWDAF may collect additional events from AMF of SMF in order to detect specific risks.

## When a risk level trespasses above or below the threshold, the NWDAF shall notify the appropriate risk with the appropriate value.

| Risk description | Detection policy and thresholds |
| --- | --- |
| Unexpected UE location | Ratio of unexpected UE location in the time period |
| Unexpected long-live/large rate flows | Ratio of volume increase |
| Unexpected wakeup | FFS |
| Suspicion of DDoS attack | Ratio of usage of a destination IP address |
| Wrong destination address | FFS |
| Ping-pong stationary UE | FFS |

NF/AF/OAM

NWDAF

1-Nnwdaf_AnalyticsInfo_Request

/ Nnwdaf_AnalyticsInfo_Subscribe

AMF

SMF

2a-Namf_EventExposure_Subscribe

2b-Namf_EventExposure_Notify

2c-Nsmf_EventExposure_Subscribe

2d-Nsm_EventExposure_Notify

3- NWDAF derives new analytics

4-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsInfo_Notify

6-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsInfo_Notify

5- NWDAF derives new analytics

* * * End of changes * * * *