3GPP TSG-SA WG2 Meeting #132	S2-1903100

8 - 12 Apr 2019, Xi'an, China

Source:	ORANGE

Title:	Expected UE behavioural parameters related network data analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes analytics in order to determine the behaviour of UEs

# Discussion

The TS 23.288 proposes analytics in order to analyse the behaviour of UEs.

The following contribution extends the service on several aspects:

- The service can be triggered at the initiative of NWDAF consumer.

- The storage (e.g. in UDSF) of the resulting information is under the responsibility of the service consumer

- The analytics produced are splitted in two blocks for modularity: mobility and communication.

- Several elements of the analytics produced for communication are removed, because not available to the NWDAF by data collection (on maximum latency, maximum response time)..

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.7 Expected UE behavioural parameters related network data analytics

### 6.7.1	General

The clause 6.7 defines how a service consumer learns from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE.

The service consumer may be an NF (e.g. AMF, UDM), an AF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	AnalyticsId set to "Mobility behaviour" or “Communication behaviour”.

-	The Target of Analytic Reporting can be any UE or an Internal Group Identifier. This target refers to the UEs that are presumed to be located in the Area of Interest at the time indicated in the Observation period.

-	An Observation period indicates the time when the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.7.3.

### 6.7.2	Input Data

In order to produce “Mobility behaviour analytics”, the NWDAF collects UE mobility analytics as defined in clause 6.10.3. The NWDAF may also collect additional data from AMF as defined in Table 6.7.2-1.

Table 6.7.2-1: UE behavioural information collected from 5GC NF(s)

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| Frequent mobility re-registration | AMF | A stationary UE may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple re-registrations if the cells belong to different registration areas. The number of re-registrations may be an indication for abnormal behaviour. |

Editor's Note:	How the NWDAF gets input information related to frequent mobility registration is FFS.

In order to produce “Communication behaviour analytics”, the NWDAF collects UE communication analytics as defined in clause 6.11.3. The NWDAF may also collect  UE behavioural information defined in the Table 6.7.2-2

Table 6.7.2-1: UE behavioural information collected from 5GC NF(s)

| Information | Source | Description |
| --- | --- | --- |
| UE ID |  |  |  | SUPI |  |
| UL or DL Packet Latency |  | Indicating the delay for uplink or downlink packets transfers for the UE. |
| UL or DL data rate |  | Indicating the bit rate for uplink or downlink packets transfers for the UE. |

Editor's Note:	How the NWDAF gets input information (e.g. from OAM) related to UL or DL Packet latency or data rate is FFS.

### 6.7.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the expected UE behaviour parameters as defined in Table 6.7.3-1 together with a UE group ID or a specific UE ID from the NWDAF to the consumer NF (e.g. UDM), indicating

The Analytic ID named "Mobility behaviour" provides the information elements, described in Table 6.7.3-1.

| Information | Source | Description |
| --- | --- | --- |
| UE group ID or UE ID | AMF | Identifies a group of UEs or a specific UE, e.g. internal group ID, SUPI List or SUPI. |
| Stationary indication | AMF | Identifies whether the UE is stationary or mobile, e.g. only on demand. (TS 23.682 [5], clause 5.10.1). |
| UE Moving Trajectory | AMF | Identifies the UE's expected geographical movement (TS 23.502 [3], clause 4.15.6). Example: A planned path of movement |

Table 6.7.3-1: Expected UE mobility behavioural analytics

The Analytic ID named "Communication behaviour" provides the information elements, described in Table 6.7.3-1.

| Information | Source | Description |
| --- | --- | --- |
| UE group ID or UE ID | SMF | Identifies a group of UEs or a specific UE, e.g. internal group ID, SUPI List or SUPI. |
| Periodic communication indicator | SMF | Identifies whether the UE communicates periodically or not, e.g. only on demand. (TS 23.682 [5], clause 5.10.1). |
| Communication duration time | SMF | Duration interval time of periodic communication (may be used together with 1) (TS 23.682 [5], clause 5.10.1). Example: 5 minutes |
| Periodic time | SMF | Interval Time of periodic communication (may be used together with 1) (TS 23.682 [5], clause 5.10.1). Example: every hour |
| Scheduled communication time | SMF | Time zone and Day of the week when the UE is available for communication (TS 23.682 [5], clause 5.10.1). Example: Time: 13:00-20:00, Day: Monday |
| Traffic characterization | SMF | Estimated UL/DL traffic volume |

Table 6.7.3-2: Expected UE behavioural parameters

NOTE: other parameters as stated in TS 23.682 [5] clause 5.10.1 which characterize the communication behaviour can only be provided by the service consumer (e.g. AF, on maximum latency, maximum response time).

### 6.7.4	Procedures

#### 6.7.4.1	NWDAF assisted expected UE behavioural parameters provisioning

NF/AF/OAM

NWDAF

1-Nnwdaf_AnalyticsInfo_Request

/ Nnwdaf_AnalyticsInfo_Subscribe

AMF

SMF

2a-Namf_EventExposure_Subscribe

2b-Namf_EventExposure_Notify

2c-Nsmf_EventExposure_Subscribe

2d-Nsm_EventExposure_Notify

3- NWDAF derives new analytics

4-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsInfo_Notify

6-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsInfo_Notify

5- NWDAF derives new analytics

Figure 6.7.4.1-1: NWDAF assisted expected UE behavioural parameters provisioning procedure

- 5GC NF to NWDAF: Nnwdaf_AnalyticsInfo_Request (Analytic ID, Analytic Filter=SUPI) or Nnwdaf_ AnalyticsSubscription_Subscribe (Analytic ID, Target of Analytic Reporting =SUPI).
The analytic IDs can be “mobility behaviour” or “communication behaviour”

2a-b.	if analytic ID “Mobility behaviour” is requested, the NWDAF requests to AMF Namf_ EventExposure_Subscribe (Event ID, external UE ID) as described in 6.10.

Event ID indicates location report for the UE. 
The external UE ID is obtained by the NWDAF based on UE internal ID, i.e., SUPI.

2c-d- if analytic ID “Communication behaviour” is requested, NWDAF requests to SMF: Nsmf_ EventExposure_Subscribe (Event ID, SUPI) as described in 6.11.

Event ID indicates communication report for the UE. 
The external UE ID is obtained by the NWDAF based on UE internal ID, i.e., SUPI.

3.	The NWDAF derives requested analytics, in the form of UE communication pattern for mobility and communication.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Response or Nnwdaf_ AnalyticsSubscription_Notify.

The NWDAF provides requested UE communication analytics to the NF, using either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_ AnalyticsSubscription _Notify, depending on the service used in step 1.

5-6. If the NF subscribed UE communication analytics at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

* * * End of changes * * * *