3GPP TSG-SA WG2 Meeting #132	S2-1903119

Xi’an - China, 8 – 12 April 2019	(revision of S2-19xxxxx)

Source:	Ericsson

Title:	UDM information storage

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This pCR propose to define Privacy profile and MO-LR subscription data stored in UDM.

.

# 1. Discussion

In clause 5.4 the handling of “privacy profiles” is being defined. The privacy data is defined to be stored in UDM/UDR. From 23.271 it is found beneficial to have a normative definition of privacy data and structure that is stored in UDM/UDR.

Further the MO-LR procedure includes checking of whether the subscription profile allows the specific MO-LR service.

Thus, it proposed to include in clause 7.1 the data stored in UDM related to privacy profile and MO-LR.

This pCR is based on a snapshoot of current discussion on privacy profile content and handling and deviates from text in TS 23.271 V0.2.0.

Further this pCR is based on assumption that a reference is added to listing of SDM data types in 23.502 5.2.3.3.1 as below:

Table 5.2.3.3.1-1: UE Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| LCS subscription data | Privacy Profile data | See clause 7.1 of 23.273 [51] |
|  | Mobile Originated data | See clause 7.1 of 23.273 [51] |

# 2. Proposal

It is proposed to include below text in TS 23.273

***** BEGIN 1st CHANGE *****

## 7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy class, refer to clause 5.4.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Override Class | M | Indication of one of the following mutually exclusive options: - Location is overridden - Single location is not overridden - Single and multiple locations are not overridden |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client not in the external LCS client list the following data may be present: - One of mutually exclusive options: - Location not allowed (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or Codeword value to be checked in GMLC External LCS client list: a list of zero or more LCS clients, with the following data for each LCS client: - One of mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [4]. The following data may be present for each service type in the list: - One of mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or Codeword value to be checked in GMLC |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE |

The Mobile Originating data is defined in table 7.1-1 containing the LCS MO-LR services that a UE can receive.

Table 7.1-2: LCS Mobile Originated data for a UE Subscriber

| MO-LR Data | Presence | UDM data |
| --- | --- | --- |
| Mobile Originated data | M | List of MO-LR services allowed for a UE subscriber: - Basic Self Location (UE can receive its own location) - Autonomous Self Location (UE can receive location assistance data) - Transfer to Third Party |

***** End of CHANGE *****