3GPP TSG-SA2 Meeting #132	S2-1903237

# April 8 - 12, 2019, Xi’an, China

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1084 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing QoS differentiation for access to PLMN services via non-public networks and vice versa_option1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Charter Communications, Orange, China Telecom, China Unicom, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is still FFS for support of QoS differentiation for access to PLMN services via non-public networks and vice versa. In order to support "Accessing PLMN services via SNPN networks and vice versa", the N3IWF based architecture was reused. The N3IWF is used for N3GPP before, the QoS is not guaranteed by N3GPP, but QoS can be guaranteed by the SNPN. The UE’s PLMN PDU session and PLMN NAS signalling are carried over the IPsec SA over Nwu interface. And the related IPsec SA data will be transmitted over the SNPN PDU session.The SNPN will be transparent to the PLMN’s NAS signalling and user plane data and QoS differentiation within the SNPN for PLMN’s services (e.g. IMS voice) is unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing QoS differentiation for access to PLMN services via non-public networks and vice versa |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of QoS differentiation for access to PLMN services via non-public networks and vice versa |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.30.2.7.Y, 5.30.2.8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

5.30.2.7	Access to PLMN services via stand-alone non-public networks

To access PLMN services, a UE in SNPN access mode that has successfully registered with an SNPN may perform another registration via the SNPN with a PLMN (using the credentials of that PLMN) following the same architectural principles as specified in clause 4.2.8 and the SNPN taking the role of "Untrusted non-3GPP access".

5.30.2.7.X	Support for QoS differentiation when access to PLMN services via stand-alone non-public networks

When a UE accesses a PLMN’s services via a SNPN, in order to achieve QoS differentiation of the NAS signaling and User Plane data of the PLMN, the follows are applied:

-	The UE gets access to the PLMN’s services via a PDU Session established with the DNN for accessing the PLMN. For the PDU session established with the DNN for accessing the PLMN, the UE applies as follows:

-	Upon reception of the signalling IPsec SA setup (i.e. for NAS signalling) from the N3IWF of the PLMN, the UE may request the SNPN to setup a QoS flow solely used for the signalling IPsec SA. For the requested QoS flow, the UE shall provide the SPI of the signalling IPsec SA as the packet filter and a 5QI used for the signalling.

-	Upon reception of the IPsec child SA setup (i.e. for user plane data) from the N3IWF of the PLMN, the UE may request the SNPN to setup the QoS flow for each IPsec child SA. For the requested QoS flow, the UE shall provide the requested QoS flow description based on the QoS flow description (e.g. standardized 5QI(s)) associated with the IPsec child SA and the SPI of the IPsec child SA as the packet filter.

-	If the SPI is updated for the IPsec SA (e.g. due to SA rekey), the UE shall update the packet filter of the QoS flow used for the IPsec SA accordingly.

-	The PLMN’s N3IWF may be configured by the OAM to allocate different IPsec child SA for different QoS flow.

-	The SNPN PCF or SMF applies as follows:

-	Be configured with an inter-operator agreement between the SNPN and the PLMN, including whether authorize the QoS request from the UE.

-	Configure to the UE a DNN used for accessing the PLMN.

-	Authorize the UE’s requested QoS flow description and requested QoS rule in the PDU session with the DNN for accessing the PLMN, based on the inter-operator agreement between the SNPN and PLMN.

5.30.2.8	Access to stand-alone non-public network services via PLMN

To access SNPN services, a UE that has successfully registered with a PLMN may perform another registration via the PLMN with an SNPN (using the credentials of that SNPN) following the same architectural principles as specified in clause 4.2.8 and the PLMN taking the role of "Untrusted non-3GPP access".

5.30.2.8.X	QoS differentiation when access to stand-alone non-public network services via PLMN

When a UE accesses a SNPN’s services via a PLMN, in order to achieve QoS differentiation of the NAS signaling and User Plane data of the SNPN, the follows are applied:

-	The UE gets access to the SNPN’s services via a PDU Session established with the DNN for accessing the SNPN. For the PDU session established with the DNN for accessing the SNPN, the UE applies as follows:

-	Upon reception of the signalling IPsec SA setup (i.e. for NAS signalling) from the N3IWF of the SNPN, the UE may request the PLMN to setup a QoS flow solely used for the signalling IPsec SA. For the requested QoS flow, the UE shall provide the SPI of the signalling IPsec SA as the packet filter and a 5QI used for the signalling.

-	Upon reception of the IPsec child SA setup (i.e. for user plane data) from the N3IWF of the SNPN, the UE may request the PLMN to setup the QoS flow for each IPsec child SA. For the requested QoS flow, the UE shall provide the requested QoS flow description based on the QoS flow description (e.g. standardized 5QI(s)) associated with the IPsec child SA and the SPI of the IPsec child SA as the packet filter.

-	If the SPI is updated for the IPsec SA (e.g. due to SA rekey), the UE shall update the packet filter of the QoS flow used for the IPsec SA accordingly.

-	The SNPN’s N3IWF may be configured by the OAM to allocate different IPsec child SA for different QoS flow.

-	The PLMN PCF or SMF applies as follows:

-	Be configured with an inter-operator agreement between the PLMN and the SNPN, including whether authorize the QoS request from the UE.

-	Configure to the UE a DNN used for accessing the SNPN.

-	Authorize the UE’s requested QoS flow description and requested QoS rule in the PDU session with the DNN for accessing the SNPN, based on the inter-operator agreement between the PLMN and SNPN.

<<< End of changes >>>