3GPP TSG-SA2 Meeting #132	S2-1903239

# April 8 - 12, 2019, Xi’an, China

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1086 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Emergency services in a CAG cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-03-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is still FFS whether Emergency services are to be supported in a CAG cell. According to 38.304 and 36.304, when a UE treats a cell as if cell status is "barred", the UE cannot obtain the limited services (i.e. Emergency services) in this cell. It has been agreed that the CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell. So for the UE not supporting CAG, e.g. rel-15 UE, it cannot check whether it is the member of the CAG and treat the CAG cell as barred cell, so rel15 the UE cannot obtain Emergency services in CAG cell even if CAG cell supporting Emergency services. In order to support rel15 UE to obtain Emergency Services, the PLMN cell coverage has to cover the area of the CAG cell, so even if a UE is barred by a CAG cell, the UE still can access to the PLMN cell. So it is not attractive to additionally require CAG cell to support Emergency Services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Emergency Services (i.e. Emergency Registration) is not supported in Public network integrated NPN in this release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether Emergency services are to be supported in a CAG cell or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

5.30.3	Public network integrated NPN

5.30.3.1	General

Public network integrated NPNs are NPNs made available via PLMNs e.g. by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN. The existing network slicing functionalities apply as described in clause 5.15. When an NPN is made available via a PLMN, then the UE has a subscription for the PLMN.

NOTE:	Annex D provides additional consideration to consider when supporting Non-Public Network as a Network Slice of a PLMN.

As network slicing does not enable the possibility to prevent UEs from trying to access the network in areas which the UE is not allowed to use the Network Slice allocated for the NPN, Closed Access Groups may optionally be used in addition to network slicing to apply access control.

A Closed Access Group identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG.

CAG is used for the Public network integrated NPNs to prevent UE(s), which are not allowed to access the NPN via the associated cell(s), from automatically selecting and accessing the associated cell(s).

The following clauses describes the functionality needed for supporting CAGs.

NOTE: Emergency Services (i.e. Emergency Registration) is not supported in Public network integrated NPN in this release.

<<< End of changes >>>