3GPP TSG-SA WG2 Meeting #132	S2-1903300

8 - 12 Apr 2019, Xi'an, China

Source:	ORANGE

Title:	NF Load Analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes additions on request parameters, data collection, outputs and procedures for NWDAF “NF Load analytics”.

# Discussion

The present definition stated in §6.5 present the general principle of NF load analytics, but does not define which NF instance is selected.

This contribution defines additional proposals on request parameters, data collection, outputs and the procedures

- Parameters in requests or subscriptions: explicit list of Instance IDs, or implicitely the instances serving the UE.

- Definition of the inputs and outputs (average load, peak load) per NF instance

- Procedures

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.5	NF load analytics

### 6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF, an AF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	AnalyticsId set to "NF load information”.

-	The Target of Analytic Reporting is set to “any UE” or to a specific UE.

-	Analytic Filter Information: an optional list of Instance IDs

-	An Observation period indicates the time when the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

If the NF Instance IDs are provided, the NWDAF shall provide the analytics for each designated NF instance. In such case the Target of Analytic Reporting should be ignored. Otherwise, the NWDAF shall use the specific UE ID to determine which instances (AMF, N3IWF, SMF, PCF).are serving this specific UE

Editor's note:	The list of possible instances is limited to AMF, N3IWF, SMF, PCF in release 16

### 6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1.

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF resource status | OAM | The status of assigned resources such as CPU and memory for a specific NF instance |
| NF load | OAM or NRF | The load of a specific NF instance |

Editor's note:	The need for other data to collect is FFS.

### 6.5.3	Output analytics

### The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics as defined in Table 6.5.3-1.
The result is not specific to a specific UE and concerns the global NF instances as defined in 6.5.1.

Table 6.5.3-1: NF load analytics statistics or predictions

| Information |  | Source |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| List of resource status (1..n) |  |  | List of load information for each NF instance |  |
| > Instance ID |  | NF consumer | Identification of the NF instance |  |  |
| > NF resource status |  | OAM |  | The status of assigned resources such as CPU and memory |  |
| > NF load |  | OAM or NRF |  | The average load of the NF instance |  |
| > NF peak load (optional) |  | OAM or NRF | The peak load of the NF instance |  |

Editor's note:	The details for NF load information are FFS.

### 6.5.4	Procedure

The procedure depicted in Figure 6.5.4-1 allows a consumer NF to request analytics to NWDAF for NF load of various NF instances as defined in 6.5.1.

Figure 6.5.4-1: NF load analytics provided by NWDAF

1.	The NF sends a request to the NWDAF for analytics for NF load for a specific NF, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The analytics ID is set to NF load information, the target for analytics is set to NF id. The NF can request statistics or predictions or both and can provide a time window.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may need for each NF targeted instance to subscribe to OAM services to retrieve the target NF load information and the target NF resource status. This step, together with steps 3, 4 and 5, may be skipped when e.g. the NWDAF already has the requested analytics.

Editor's note:	The exact SA5 service used is FFS.

3.	OAM performs the required configuration in order to provide the information requested by the NWDAF's subscription request. It is up to the OAM to identify if already deployed/configured support for data collection related to the subscription request will be reused for each request from NWDAF or if new support for data collection of information associated with the subscription needs to be deployed/configured.

4.	OAM performs the tasks, e.g. data collection, data processing, associated with the subscribed request from NWDAF.

5.	OAM services send notification(s) to NWDAF about subscribed information.

NOTE:	The call flow only shows a subscription/notification model for the simplicity, however both request-response and subscription-notification models should be supported.

6.	The NWDAF may retrieve the NF load from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation for each NF instance.

Editor's note:	Either step 6 or step 2-5 is performed.

7.	The NWDAF derives requested analytics.

8.	The NWDAF provide requested NF load analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

9-11. If at step 1 the NF has subscribed to receive continuous reporting of NF load analytics, the NWDAF may generate new analytics and provide them to the NF upon reception of new NF load information from OAM.

* * * End of changes * * * *