3GPP TSG- SA WG2 Meeting #132	S2-1903303

April 8 – 12 Xi’an, China

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1175 | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Nupf eventexposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 01-04-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NWDAF performs data collection from 5GS NF using Nnf_EventExposure services. However, in those procedures described in 23.288 that require input data that is stored in UPF, no solution is provided by Editor´s note were added in several places. For example: “Editor´s note: The NWDAF needs information on the observed QoE at the UPF, how this information is provided to NWDAF needs to be studied. This contribution proposes that NWDAF collects data from UPF using Nsmf_EventExposure while the Notification endpoint of NWDAF is included. This implies that UPF notifies to NWDAF of the requested event information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Nupf_EventExposure service and procedures are included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not identifed mechanism to retrieve data for analytics available at UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.x (new), 5.2.x(new), 5.2.x.1 (new), 5.2.x.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 Tdoc: S2-190xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****** Start of Changes ******

#### 4.15.4.x	Procedure for Data Collection from UPF

The procedure in Figure 4.15.4X-1 is used by NWDAF to subscribe/unsubscribe at SMF in order to be notified for UPF data collection on a related Event Id(s).

Figure 4.15.4.X-1: Event Exposure Subscribe/unsubscribe for UPF

- The NWDAF subscribes to or cancels subscription for a (set of) Event ID(s) by invoking the Nsmf_EventExposure_Subscribe/ Nsmf_EventExposure_Unsubscribe service operation. In the subscription NWDAF includes the (set of) Event-ID(s) corresponding to the UPF’s data to collect, Event Filter information indicating either a GPSI or a SUPI or an Internal Group Id or an Application Id, NWDAF IP address as Notification Correlation Address and Notification Correlation ID. The SMF provides a Subcription Correlation Id to NWDAF.

Editor´s note:	The Event ID (s) shall be also exposed by Nupf_EventExposure. They can be defined in TS 23.502 [3] or new Event ID(s) could be needed which depends on the use cases.

- The SMF subscribes to or cancels subscription for the (set of) Event ID(s) to UPF. In the subscription request SMF includes the (set of) Event-ID(s), Notification Target Address (i.e. NWDAF’s IP address) and Notification Correlation ID.

Editor’s note: How SMF subscribes to Nupf_EventExposure on behalf NWDAF is FFS.

- The UPF notifies the NWDAF by invoking Nupf_EventsExposure_Notify service operation, including the Event report along with the Notification Correlation ID.

****** Next Change ******

### 5.2.X	UPF services

#### 5.2.X.1	General

The following table shows the UPF Services and UPF Service Operations.

Table 5.2.8.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Notify | Notify | NWDAF |

Editor’s note: Whether Subscribe and Unsubscribe operations or a Request/Reponse are needed is FFS.

#### 5.2.X.2	Nupf_EventExposure Service

##### 5.2.X.2.1	General

Service description: This service provides events related to data collected by NWDAF. The following are the key functionalities of this NF service.

-	Notifying events to the NWDAF.

##### 5.2.X.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: Report related UPF event(s).

Input Required: Event ID, Notification Correlation Information, time stamp.

Input, Optional: Event specific parameter list as described in TS 23.288 [x], UE ID (SUPI and if available GPSI)

Output Required: Result Indication.

Output, Optional: Redirection information.

When the UPF detects the event, it reports the subscribed event together with the Notification Target Address (+ Notification Correlation ID) to the NWDAF.

****** End of Changes ******