SA WG2 Meeting #132 	S2-1903319

# 08-12 April 2019, Xian, PRC														(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1102 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to access network performance measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR solves following editor's notes - Editor's note: How the RTT thresholds per access may be included in ATSSS rules is FFS. RTT measurements are required for the "smallest delay" steering mode and it is up to the UE and to the UPF to decide on criteria like stability of the RTT over the access, data flows 5QI, battery consumption, congestion status, etc. Sending RTT thresholds to the UE would not bring much. Moreover, the N4 rules to UPF would also be impacted for the DL traffic. - Editor's note: Whether the 5QI value used to convey the PMF-Echo Request / PMF-Echo Response messages is determined by Measurement Assistance Information or is pre-configured in the UE is FFS. It shall be able for the UE and the UPF to perform RTT measurements on a per QoS flow level for data flows that are subject to the "smallest delay" steering mode over the same 5QI and queues as the data flow. Hence ATSSS/N4 rules are used for the decision. - Editor's note: The protocol stack in case of Ethernet MA PDU Session is FFS. RTT measurements and Access (Un)Availability Indication should use the same mechanisms for IP traffic and for Ethernet traffic where no IP address is allocated to the UE by the Core Network. PMF protocol should run over Ethernet or UDP/IP. A note should be added in the case stage 3 would prefer another transport protocol. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative text is added for per QoS flow RTT measurements. The editor's note on RTT thresholds is removed. Normative text is added to mandate same RTT measurements mechanisms for IP PDU and Ethernet PDU Session Types. A note is added to allow revisit the transport of PMF protocol by stage 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QoS used for RTT measurements would remain unspecified. Whether RTT thresholds are provided to UE and UPF would remain unspecified. How RTT is measured and how Access (Un)Availability Indication is reported for Ethernet PDU Session Type would not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.1, 5.32.5.2, 5.32.5.3, 5.32.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.32.5	Access Network Performance Measurements

#### 5.32.5.1	General principles

When the UE requests an MA PDU Session Establishment and indicates it is capable to support ATSSS, the UE may receive Measurement Assistance Information, during the establishment of an MA PDU session. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether and when measurement reports need to be sent to the network.

Measurement Assistance Information shall include the Layer 3 address for IP PDU Session Type or the Layer 2 address for Ethernet PDU Session Type of a Performance Measurement Function (PMF) in the UPF, with which the UE can send PMF protocol messages.

The following PMF protocol messages can be exchanged between the UE and the PMF:

-	Messages for Round Trip Time (RTT) measurements, e.g. when the "Smallest Delay" steering mode is used;

-	Messages for reporting Access availability/unavailability by the UE to the UPF.

Editor's note:	Before any PMF protocol messages can be exchanged between the UE and UPF, whether a PMF session between the UE and the UPF should be established is FFS.

When the UE requests a MA PDU session and indicates it is capable to support the MPTCP functionality only, the network should not send Measurement Assistance Information as in this case the UE can use measurements available at the MPTCP layer.

#### 5.32.5.2	Round Trip Time measurement

If required by the ATSSS rules of a MA PDU session, RTT measurement can be conducted by the UE and UPF independently. There is no measurement reporting from one side to the other.

The estimation of the RTT by the UE or by the UPF is based on the following mechanism: one entity (UE or UPF) sends an PMF-Echo Request to the destination address/port of the PMF function in the peer entity, which replies with an PMF-Echo Response to the source address/port of the PMF function in the originating entity. The originating entity derives an estimation of the average RTT over an access type by averaging the RTT measurements obtained over this access.

It shall be able for the UE and the UPF to perform RTT measurements on per QoS flow. The UE and the UPF shall perform RTT measurements for data flows that are subject to the "smallest delay" steering mode over the same 5QI and queues as the data flow.

#### 5.32.5.3	Access Availability/Unavailability Report

If required by the network in the Measurement Assistance Information, the detection of the unavailability and of the availability of an access, as well as and the decision of reporting it, can be performed by the network or the UE, based on implementation. When the UE has decided, based on implementation, to report the unavailability/availability of an access, it shall:

-	build a PMF-Access Report containing the access type and an indication of availability/unavailability of this access;

-	send the PMF-Access Report to the UPF via the user plane.

The UPF shall acknowledge the PMF-Access Report received from the UE.

#### 5.32.5.4	Protocol stack for Access Network Performance Measurements

Editor's note:	The protocol stacks may need to be revised based on stage 3 feedback.

Figure 5.32.5.4-1: Access Network Performance measurements related protocol stack for 3GPP access (IP PDU session Type)

Figure 5.32.5.4-2: Access Network Performance measurements related protocol stack for 3GPP access (Ethernet PDU session Type)

Figure 5.32.5.4-3: Access Network Performance measurements related protocol stack for non-3GPP access (IP PDU session Type)

Figure 5.32.5.4-4: Access Network Performance measurements related protocol stack for non-3GPP access (Ethernet PDU Session Type)

END OF CHANGES