3GPP TSG-SA WG2 Meeting #132	S2-1903366

# Xi’an P.R. China. 8 - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1119 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Granularity of TSN bridge |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-03-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The granularity of the TSN bridge is unclear, DP S2-1903364 “TSN bridge granularity” analyses different options and selects option 2, per UPF based virtual bridge. Also DP S2-1903365 “Principle of 5G bridge port configuration”, provides binding of ports for the TSN bridge configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the granularity of the TSN bridge |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSN bridge functionality is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 5.29.1	5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on the UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the UE/TT side. For each 5GS Bridge of a TSN network, the ports on the UPF/TT side support the connectivity to the TSN network, the ports on the UE/TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the logical TSN bridge is per UPF. The bridge ID of the TSN bridge is bound to the UPF ID. The TSN AF binds the TSN ports with PDU sessions on the UE side, and binds TSN ports with physical UPF/TT ports in UPF side.

All PDU Sessions which connect to TSN via a specific UPF are grouped into a single virtual bridge. The capabilities of each port in the UEs and UPF are integrated as part of the configuration of the 5G virtual bridge and is notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

Figure 5.29.1-x.  Per UPF based virtual bridge

NOTE 1: In case a UE establishes multiple PDU sessions that can go to different UPFs, then the UE is represented by multiple logical TSN bridges. If such a UE is connected to the TSN system on the terminal side over a single interface, then an interim TSN bridge needs to be inserted between the TSN system on the terminal side and the UE to avoid shared Ethernet media where a single interface connects to multiple bridges.  The configuration of such an interim TSN bridge is outside 3GPP scope.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Report the bridge information of 5GS Bridge to TSN network.

-	Map the configuration information obtained from TSN network for 5GS Bridge into 5GS QoS within PDU Session and TSC Assistance Information for efficient time-aware scheduling, as defined at clause 5.29.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes the following:

-	Bridge ID of 5GS Bridge.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay, ingress port ID, egress port ID and traffic class.

-	Propagation delay per port, including transmission propagation delay, egress port ID.

-	Topology of of 5GS Bridge as defined in 802.1AB.

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q.

Editor's note:	It is FFS whether and how the topology of 5GS Bridge as defined in 802.1AB is needed and if additional information is needed.

The AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

Editor's note:	How and when to collect/generate the user plane information of 5GS Bridge within 5GS is FFS.

* * * * End of Change * * * *