3GPP TSG-SA WG2 Meeting #132	S2-1903510

# Xi'an, China, April 8 – 12, 2019												 (revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1215 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Session Modification for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no procedure for PDU Session Modification for MA PDU Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE or network requested MA PDU Session Modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and network cannot perform QoS modification for MA PDU Session and ATSSS rules and N4 rules are not updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.Y	UE or network requested MA PDU Session Modification

4.22.Y.1	General

This procedure is triggered in the following cases:

-	QoS Flow creation / modification (including GBR QoS flow movement)

-	Update of ATSSS rules and/or N4 rules

4.22.Y.2	UE or network requested MA PDU Session Modification (non-roaming and roaming with local breakout)

The signalling flow for a MA PDU Session Modification when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.3.2-1 with the following differences and clarifications:

-	In step 1, if the UPF needs to switch access for the traffics of the GBR QoS flow, the UPF shall send notification to the SMF before switching the access. The notification includes packet detection information and target access. When the SMF received the notification, the SMF triggers this procedure and moves the GBR QoS flow to the target access. The SMF sends N2 SM information to the target AN. After the GBR QoS flow is established over the target access, the SMF releases AN resouce of the GBR QoS flow in the source access. The PDU Session Modification Command message is not sent to the UE.

-	In step 3, if a new QoS Flow is established, the SMF provides QoS profile to the AN as described in TS 23.501 [2] clause 5.32.4.

-	In step 3, when the SMF provides N1 SM container and/or N2 SM information, the SMF includes access type in the Namf_Communication_N1N2MessageTransfer to provide routing information to the AMF.

4.22.Y.3	UE or network requested MA PDU Session Modification (home-routed roaming)

The signalling flow for a MA PDU Session Modification when the UE is roaming and the PDU Session Anchor (PSA) is located in the HPLMN, is based on the signalling flow in Figure 4.3.3.3-1 with the following differences and clarifications:

-	In step 1, if the H-UPF needs to switch access for the traffics of the GBR QoS flow, the H-UPF shall send notification to the H-SMF before switching the access. The notification includes packet detection information and target access. When the H-SMF received the notification, the H-SMF triggers this procedure and moves the GBR QoS flow to the target access. The H-SMF sends updated GBR QoS Flow information contains associated access type to the V-SMF. Based on the information the V-SMF establishes AN resources for the GBR QoS Flow to the target access. After the GBR QoS Flow is established over the target access, the V-SMF releases AN resouce of the GBR QoS Flow in the source access. The PDU Session Modification Command message is not sent to the UE.

-	In step 3, when the H-SMF provides GBR QoS Flow information, the H-SMF includes associated access type in Nsmf_PDUSession_Update.

-	In step 4, if a new QoS Flow is established, the V-SMF provides QoS profile to the AN as described in TS 23.501 [2] clause 5.32.4.

-	In step 4, when the V-SMF provides N1 SM container and/or N2 SM information, the V-SMF includes access type in the Namf_Communication_N1N2MessageTransfer to provide routing information to the V-SMF.

* * * * End of Changes * * * *