3GPP TSG-SA WG2 Meeting #132	S2- 1903571

8 April – 12 April 2019, Xi’an, China

Source:	Samsung

Title:	Dividing expected behavioural parameters and network configuration parameters

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes to separate the current output of clause 6.7 into Expected UE behavioural parameters and Network Configuration parameters

- Discussion

According to the latest version of TS23.502 clause 4.15.6, there are two types of expected parameters are defined, Expected UE behavioural parameters and Network Configuration parameters. Current description of Expected UE behavioural parameter include the parameters of network configuration parameters. To match with the definition of TS 23.502, the parameters of clause 6.7 Expected UE behavioural parameters related network data analytics need to be separated.

2.	Proposal

It is proposed to include the added text into the TS 23.288. This contribution proposes to separate the current output of clause 6.7 into Expected UE behavioural parameters and Network Configuration parameters.

* * * * First Change * * * *

6.7.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the expected UE behaviour parameters as defined in Table 6.7.3-1 together with a UE group ID or a specific UE ID from the NWDAF to the consumer NF (e.g. UDM), indicating

New Analytic IDs named "Expected UE behaviour parameters" and “Network configuration parameters” are defined with one or multiple combinations of the following analytic filters:

-	UE group ID or UE ID.

Table 6.7.3-1: Expected UE behavioural parameters (see TS 23.502[x], clause 4.15.6.3)

| Information | Source | Description |
| --- | --- | --- |
| UE group ID or UE ID | AMF or SMF | Identifies a group of UEs or a specific UE, e.g. internal group ID, SUPI List or SUPI. |
| Stationary indication | AMF | Identifies whether the UE is stationary or mobile, e.g. only on demand. |
| UE Moving Trajectory | AMF | Identifies the UE's expected geographical movement. Example: A planned path of movement |
| Periodic communication indicator | SMF | Identifies whether the UE communicates periodically or not, e.g. only on demand. |
| Communication duration time | SMF | Duration interval time of periodic communication (may be used together with 1). Example: 5 minutes |
| Periodic time | SMF | Interval Time of periodic communication (may be used together with 1). Example: every hour |
| Scheduled communication time | SMF | Time zone and Day of the week when the UE is available for communication. Example: Time: 13:00-20:00, Day: Monday |

Table 6.7.3-1: Network Configuration parameters (see TS 23.502[x], clause 4.15.6.3a)

| Maximum Latency | SMF | Indicating maximum delay acceptable for downlink data transfers. |
| --- | --- | --- |
| Maximum Response Time | SMF | Indicating the time for which the UE stays reachable to allow the AF to reliably deliver the required downlink data. |
| Suggested Number of Downlink Packets | SMF | Indicating the number of packets that the UPF shall buffer in case the UE is not reachable. |

* * * * End of change * * * *