3GPP TSG-SA WG2 Meeting #132	S2- 1903573

8 April – 12 April 2019, Xi’an, China

Source:	Samsung

Title:	Adding input parameters for analysing UPF load analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes to collect the user traffic related data from multiple sources for analysing UPF load information.

- Discussion

According to the conclusion of TR 23.791, NWDAF may collect several parameters to analyse UPF load. The main source of the UPF load is to process user traffic. Therefore, to correctly analyse the UPF load, it is necessary to collect user traffic related data from multiple sources such as AMF, SMF, AF, and OAM. This pCR proposes to collect the user traffic related data for UPF load analysis.

2.	Proposal

It is proposed to include the added text into the TS 23.288. This contribution proposes to collect the user traffic related data from multiple sources for analysing UPF load information.

* * * * First Change * * * *

6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1.

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF resource status | OAM | The status of assigned resources such as CPU and memory |
| NF load | OAM or NRF | The load of a specific NF |

For UPF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-2, in addition with Table 6.5.2-1.

Table 6.5.2-2: Collecting Data for analysing UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Location Info | AMF or SMF | The location information of UE. |
| DNN | SMF or PCF | To identify the DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF or AMF | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| Traffic volume | SMF | The consumed traffic volume for the application identified by the Application Id. |
| UE ID | AMF or SMF | To identify UE |
| Communication and mobility patterns | AF | To characterize the communication and mobility patterns for the service, as specified in TS 23.682 [5] |

Editor's note:	The need for other data to collect is FFS.

* * * * End of change * * * *