SA WG2 Meeting #132	S2-1903598

April 8 – 12, 2019, Xi’an, China	(was S2-19----)

Source:	CATT

Title:	LMF  Service update

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This paper proposes to add a LMF service of context transfer between two LMFs.

1.	Discussion

A new LMF service is introduced in LMF change procedure is section 6.4 to transfer context between LMFs, this pCR is proposed to define the LMF service.

2.	Proposal

It is proposed to update TS 23.273 as follows.

*** First change ***

8.3	LMF Services

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_ Location | DetermineLocation | Request/Response | AMF |
|  | ContextTransfer | Request/Response | Peer LMF |
|  | EventNotify | Notify | GMLC |

8.3.2	Nlmf_Location service

8.3.2.1	General

Service description: This service enables an NF to request location determination for a target UE. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request the current geodetic and optionally civic location of a target UE.

-	Allow the consumer NF to subscribe/unsubscribe the geodetic and optionally civic location of a target UE for some certain events.

-	Allow the consumer NF to get notified about the geodetic and optionally civic location of a target UE when some certain events are detected.

-	UE information management and transfer.

The events to trigger location estimation notification are defined in clause 4.1a.5.1

8.3.2.2	Nlmf_Location_DetermineLocation service operation

Service operation name: Nlmf_Location_DetermineLocation

Description: Provides UE location information to the consumer NF.

NOTE:	For deferred location request, this service operation is used to implicitly subscribe to the notification of the UE location information.

Input, Required: External Client Type, LCS Correlation Identifier.

Input, Optional: serving cell identifier if UE is using 3GPP access, Location QoS, Supported GAD shapes, AMF identity if a UE associated Namf_Communication service is to be invoked by LMF, Notification Target Address, list of Events, Event Reporting Information defined in Table 4.15.1-1 of TS 23.502 [19].

Output, Required: Success/Failure indication

Output, Optional: Geodetic Location, Civic Location, Position Methods Used (in the case of success indication provided), Failure Cause (in the case of failure indication provided).

See clause 6.1 for example of usage of this service operation.

8.3.2.3	Nlmf_Location_EventNotify service operation

Service operation name: Nlmf_Location_EventNotify.

Service operation description: Allow the consumer NF to get notified about the geodetic and optionally civic location of a target UE when some certain events are detected.

Input, Required: LCS Correlation Identifier, UE (SUPI and if available GPSI), Notification Target Address.

Input, Optional: Geodetic Location, Civic Location, Position Methods Used (in the case of success indication provided), Failure Cause (in the case of failure indication provided).

Output, Required: None.

Output, Optional: None.

See clause 6.3 for example of usage of this service operation.

8.3.2.4	Nlmf_Location_ContextTransfer service operation

Service operation name: Nlmf_Location_ContextTransfer.

Service operation description: Provides UE context to the consumer NF.

Input, Required: UE identity, AMF identity.

Input, Optional: current status of event reporting, UE location information, event report message.

Output, Required: Result indication.

Output, Optional: None.

See clause 6.4 for example of usage of this service operation.

*** End of changes ***