3GPP TSG-SA WG2 Meeting #132	S2-1903623

# April 8 - 12, 2019, Xi'an, China

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1202 | rev |  | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PDU Session management procedure for 5G-LAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-03-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) 5G-LAN group configuration is used for SMF to manage PDU Session for 5G-LAN, so the group configuraiton should be stored in the UDE as part Session Management Subscription data. 2) For one 5G-LAN group, the unicast and the multicast shares the same PDU Session. The UPF needs to know if the received data is for unicast or for multicast. For understanding, the destination of the packet should be for multicast, then the UPF will know the data is for multicast within the group based on the PDR configuration. 3) unlike N6 based data forwarding in which the destination address is the server address, in 5G-LAN, the destination address is peer UE address, and for IP type PDU, the address of the peer UE is allocated by the SMF when the PDU Session is established for the peer UE. So it should clarify how the UE get the address for the peer UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Clarify the 5G LAN group configuration is stored in the UDR as part Session Management Subscription data. 2) Clarify in order to support multicast within 5GLAN, the SMF can allocate the destination multicast address for the group and provide the multicast address to the UEs in the group. If the SMF does not allocate the destination multicast address, then a wildcard address can be used for the destination address for multicast in this group. 3) Clarify in order to support unicast within 5GLAN, UE needs to know the destination address of the peer UE within 5GLAN group, this can be implemented by the application layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear for PDU Session management for 5G-LAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

5.29	Support for 5G LAN-type service

5.29.1	General

The service requirements for 5G LAN-type service are specified in TS 22.261 [2].

A 5G-LAN group consists of a a set of UEs using private communication for 5G LAN-type services.

5.29.2	5G LAN group management

5G System supports 5G LAN Group Management can be configured by a network administrator or can be managed dynamically by AF.

The 5G LAN group member is uniquely identifed by GPSI. The group as described in clause 5.2.3.3.1 of TS 23.502 [3] is applicable to 5G LAN-type services.

In order to support dynamic 5GLAN Group Management, the NEF exposes a set of service to manage (e.g. add/delete) 5G LAN group and 5G LAN member.

The 5G LAN group configuration is either provided by OA&M or provided by an AF to the NEF. The 5G LAN group configuration is stored in the UDR as part Session Management Subscription data.

When configuration is provided by an AF, the NEF service operations information flow procedure described in TS 23.502 [3] clause 4.15.6.2 applies. The 5G LAN group configuration in UDR may include the following parameters: GPSI (i.e. UE Identities of the 5G LAN group), PDU session type, DNN, S-NSSAI.

The third party AF may update the UE Identities of the 5G LAN group at any time after the initial provisioning.

Editor's note:	Relation between DNN and 5G LAN group, e.g. whether and how a 1:1 and/or 1:N relation is supported, is FFS.

The PCF delivers 5G LAN group configuration information (DNN, S-NSSAI, PDU session type) to the UE for each GPSI that belongs to a 5G-LAN group. The 5G LAN group configuration information is delivered in the URSP from the PCF to the UE using the UE Configuration Update procedure for transparent UE Policy delivery as described in TS 23.502 [3] clause 4.2.4.3 and TS 23.503 [45] clause 6.1.2.2.

5.29.3	PDU Session management

Session management as defined for 5GS in clause 5.6 is applicable to 5G-LAN-type services with the following clarifications:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G-LAN group

-	A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G-LAN group.

Editor's note:	Whether and how the above bullet needs to be updated due to eSBA is FFS.

Editor's note:	Whether there are impacts to SMF selection to select this dedicated SMF is FFS.

NOTE:	Having a dedicated SMF serving a 5G-LAN group does not contradict that redundancy solutions can be used to achieve high availability.

-	A DNN is associated with a 5G-LAN group

Editor's note:	The above bullet is FFS.

-	The UE provides a DNN associated with the 5G-LAN group to access the 5G LAN-type services for that 5G-LAN group, using the PDU Session Establishment procedure described in TS 23.502 [3], clause 4.3.2

-	During establishment of the PDU Session, secondary authentication as described in clause 5.6.6 and in TS 23.502 [3], clause 4.3.2.3, may be performed in order to authenticate and authorize the UE for accessing the DNN associated with the 5G-LAN group. Authentication and authorization for a DNN using secondary authentication implies authentication and authorization for the associated 5G-LAN group. There is no 5G-LAN group specific authentication or authorization defined.

-	The SM level subscription data for a DNN and S-NSSAI available in UDM, as described in clause 5.6.1, applies to a DNN associated to a 5G-LAN group.

-	Session management related policy control for a DNN as described in TS 23.502 [3], is applicable to a DNN associated to a 5G-LAN group. This includes also usage of URSP, for the UE to determine how to route outgoing traffic to a PDU Session for a DNN associated to a 5G-LAN group.

-	In order to support multicast within 5G-LAN, the SMF can allocate the destination multicast address for the group and provide the multicast address to the UEs in the group. If the SMF does not allocate the destination multicast address, then a wildcard address can be used for the destination address for multicast in this group.

Note X:  In order to support unicast within 5G-LAN, UE needs to know the destination address of the peer UE within 5G-LAN group, this is implemented out of 3GPP scope, e.g. via application layer.

**** End of Changes ****