SA WG2 Meeting #132 	S2-1903650

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain		(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.734 | CR | 0020 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Evaluation for TSN Time Synchronization Solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-04-1 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution proposes to evaluation for TSN Time Sync. Comparison analysis and rationale for 11.2, 28 |  |  |  |  |  |  |  |  |
| Summary of change: |  | -- |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing proper evaluation for TSN Time Sync. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start of changes (all new text) ***

# 7	Evaluation

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 7.x	Evaluation for key issue 3.2

Among the solutions to key issue #3.2 (time synchronization to support TSN):

- solution #17 presents feasibility challenges according to the RAN1 reply LS R1-1901442,

- solution #19 is a special case of solution #28,

- solution #11 option 3 (#11.3) is merged with solution #28, and

- solution #11 option 4 (#11.4) is a special case of solution #11 option 2 (#11.2).

Therefore, this consolidated evaluation focuses on solution #11.2 (pointing out the specifics of #11.4 when needed) and solution #28. The evaluation compares the following aspects of #11.2 and #28: compliance with IEEE 802.1AS-Rev, support for multiple working clock domains, complexity introduced to 5GS nodes, support for efficient TSCAI-based scheduling at gNB, and support for bi-directional transport of time-synchronization information.

IEEE 802.1AS-Rev compliance

For a solution to be IEEE 802.1AS-Rev compliant, the logical TSN bridge should be able to process the incoming gPTP messages and generate outgoing gPTP messages in a way to appear as a time-aware relay on its external interfaces. In particular, the egress Sync messages should preserve the preciseOriginTimestamp of the GM and should include the correctionField calculated based on the residence time and rateRatio, as specified in IEEE 802.1AS-Rev. Solution #11.2 describes a mechanism for the preciseOriginTimestamp preservation and for the correctionField calculation (where this calculation is performed directly in TSN clock domain) and, therefore, it is 802.1AS-Rev compliant. Solution #28 preserves the preciseOriginTimestamp, but doesn’t describe how is the correctionField calculated based on the residence time measurements performed in the 5G clock domain (this residence time shall be converted to the TSN domain since the two may use different timescales, and shall be scaled by the rateRatio). Therefore, amendments to #28 are needed in order to be 802.1AS-Rev compliant.

Support for multiple working clock domains

Support for SIB signaling and RRC signaling allow multiple working clock domains to be supported by solution 11#2. Solution #11.2 can also leverage #11.4 (multiple working clock domains merged into a single 5G clock domain e.g., through normal BMCA operation) when the number of domains exceeds SIB capacity. The number of clock domains that solution 28 supports is limited by the capacity on timestamping and duplication towards UEs at UPF/TT.

Support for admission control and efficient TSCAI-based scheduling at gNB

It should be noted that the TSN clock(s) are assumed to be dynamically and frequently drifting. With the solution #11.2, gNB has the knowledge of TSN clock(s), thus it can generate and estimate expected traffic pattern based on TSCAI. Based on this, efficient scheduling and admission control can be supported. In order for RAN to support admission control based on TSCAI, it needs to know this information before QoS flow establishment (I.e. before the first packet arrives in the NG-RAN).

With the solution #28, gNB is unaware of TSN clock(s) and scheduling is performed in 5G clock domain. This means that scheduler may not be able to exploit TSCAI (e.g. burst arrival time, periodicity) due to frequent and dynamic time drifts between the 5G clock and TSN clock. Therefore, RAN has to overprovision resources or perform scheduling based on the observed traffic pattern. In other words, TSCAI is not usable with solution #28, also supporting efficient scheduling for deterministic application is questionable unless RAN monitors observed traffic pattern (which can be quite complex). Even then the NG-RAN will not be able to exploit this solution for supporting admission control.

Complexity introduced to 5GS nodes

With the solution #11.2, gNB needs to track (multiple) external clocks (one software instance per TSN clock to track the clock w.r.t. local 5G clock - no additional hardware needed). With the solution #28, gNB is not required to track TSN clock(s), but scheduling is more complex in case of drift between 5G and TSN clocks because TSCAI needs to be aligned with the drift. Solution #11.2 has no impact on UPF, while #28 requires timestamping of not only PTP, but potentially all incoming packets that can carry a PTP message. the UE side needs to extract the Time stamp information from all user plane packets.

Support for bi-directional transport of time-synchronization

Solution #11.2 supports unidirectional transfer of Sync messages from UE to UPF. Hence, the port of the logical 5GS time-aware relay that is associated with the UPF-side TT can only be configured as a “Slave” port, and the TSN GM must be located on the UPF-side In the special cases of #11.4, the 5GS acts as a TSN GM, hence all ports are configured as “Master” ports. Solution #28 supports bidirectional Sync transfer, hence TSN GM can be either on UPF- or UE-side.

A summary of the comparison between solutions #11.2 and #28 is provided in the table below:

| Requirement | Synchronization solution |  |
| --- | --- | --- |
|  | #11.2 | #28 |
| IEEE802.1AS compliance | Yes (with preciseOriginTimestamp preservation mechanism) | No (correctionField is based on 5G clock) |
| Support for multiple time-domains | Yes, but limited by SIB capacity (FFS RAN2). Mitigation with time domain merging with #11.4. | Yes |
| Multi-domain scalability (air interface capacity at 32 users distributed over 32 TDs, single TS per UE) | As supported by SIB and RRC signaling messaging, sync messaging can be reduced according to accuracy requirement and environment | Signaling overhead cannot be controlled by 5GS, overhead is default 8 messages per second per UE per external TD DL: ~300kbps / TD UL: ~3kbps / TD / UE |
| Support for efficient TSCAI-based scheduling at gNB (RAN2 requirement) | Yes, instant adjustment of scheduling resources with TSN clock drift (TSCAI includes TD information and TT residence times known to PCF) | No, only slow adjustments to TSN clock drift with core/RAN network guidance (e.g. packet inspection for TSCAI correction) |
| gNB complexity | No hardware impact, one SW clock instance per each external clock being tracked | Complexity due to TSCAI adjustments to compensate for the 5G-TSN clock drift |
| UPF complexity | No impact | Timestamping of potentially all incoming packets that may carry a PTP message |
| UE complexity | ? | ? |
| Synchronization direction support | Downlink only. Uplink support is possible extension via RRC. | Downlink and Uplink |