SA WG2 Meeting #132 	S2-1903670

08 – 12 April, 2019, Xi’an, PRC

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1225 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration transfer to support N26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 3/30/2019 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN3 has already specified the support for configuration transfer procedure that can be sent from NG-RAN to E-UTRAN however this is missing in TS 23.501. Reference to RAN3 LS - R3-191119. CR in CP-190170 on “Enhancements to Configuration Transfer Tunnel over S10 and N26 for EN-DC” (TS 29.274). Furthermore for EPS interworking, since the AMF interfaces with MME like another MME, it should support configuration transfer procedure (in order to be transparent and avoid impacts to MME) over S10. Otherwise, it will be broken for EPS interworking. This is needed for TNL address discovery, SON type of use cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce support for Configuration transfer for 5GS to EPS interworking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration transfer for Intersystem does not work, broken for EPS interworking |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.26.0, 5.26.1, 5.26.2.2, 5.26.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.26	Configuration Transfer Procedure

## 5.26.0	General

The purpose of the Configuration Transfer is to enable the transfer of information between two RAN nodes at any time via NG interface and the Core Network. An example of application is to exchange the RAN node's IP addresses in order to be able to use Xn interface between the NG-RAN node for Self-Optimised Networks (SON), as specified in TS 38.413 [34].

### 5.26.1	Architecture Principles for Configuration Transfer

Configuration Transfer between two RAN node provides a generic mechanism for the exchange of information between applications belonging to the RAN nodes. Such a separate procedure allows avoiding impacts to other RAT access systems when transferred information is added or modified.

In order to make the information transparent for the Core Network, the information is included in a transparent container that includes source and target RAN node addresses, which allows the Core Network nodes to route the messages. The mechanism is depicted in figure 5.26 1.

Figure 5.26-1: inter NG-RAN Configuration Transfer basic network architecture

The NG-RAN transparent containers are transferred from the source NG-RAN node to the destination NG-RAN node by use of Configuration Transfer messages. The NG-RAN transparent containers are also transferred from the source NG-RAN node to the destination E-UTRAN node and vice versa by use of Configuration Transfer messages.

A Configuration Transfer message is used from the NG-RAN node to the AMF over N2 interface, a AMF Configuration Transfer message is used from the AMF to the NG-RAN over N2 interface, and a Configuration Transfer Tunnel message is used to tunnel the transparent container from a source AMF to a target AMF over the N14 interface. In addition, a Configuration Transfer Tunnel message is used to tunnel the transparent container from a source AMF to a target MME and vice versa over the N26 interface (which is based on S10).

Each Configuration Transfer message carrying the transparent container is routed and relayed independently by the core network node(s). Any relation between messages is transparent for the AMF and MME, i.e. a request/response exchange between applications, for example SON applications, is routed and relayed as two independent messages by the AMF and MME. An AMF or MME supporting the Configuration Transfer procedures provides addressing, routing and relaying functions.

### 5.26.2	Addressing, routing and relaying

#### 5.26.2.1	Addressing

All the Configuration Transfer messages contain the addresses of the source and destination RAN nodes. An NG-RAN node is addressed by the Target NG-RAN node identifier.

#### 5.26.2.2	Routing

The following description applies to all the Configuration Transfer messages used for the exchange of the transparent container.

The source RAN node sends a message to its core network node including the source and destination addresses. The AMF uses the destination address to route the message to the correct AMF via the N14 interface or to the correct MME via the N26 interface.

The AMF connected to the destination RAN node decides which RAN node to send the message to, based on the destination address.

#### 5.26.2.3	Relaying

The AMF performs relaying between N2 and N14/N26 messages as described in TS 38.413 [34], TS 29.518 [71] and TS 29.274 [xx].