3GPP TSG-SA WG2 Meeting #132	S2-1903881

8 - 12 Apr 2019, Xi'an, China	merge of 3100 (without 6.7.4), and 3115

Source:	ORANGE, Ericsson

Title:	Expected UE behavioural parameters related network data analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes analytics in order to determine the behaviour of UEs

# Discussion

The TS 23.288 proposes analytics in order to analyse the behaviour of UEs.

The following contribution extends the service on several aspects:

- The service can be triggered at the initiative of NWDAF consumer.

- The storage (e.g. in UDSF) of the resulting information is under the responsibility of the service consumer

- The analytics produced are splitted in two blocks for modularity: mobility and communication.

- Several elements of the analytics produced for communication are removed, because not available to the NWDAF by data collection (on maximum latency, maximum response time)..

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.7 Expected UE behavioural parameters related network data analytics

### 6.7.1	General

The clause 6.7 defines how a service consumer learns from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE.

The service consumer may be an NF (e.g. AMF, UDM), an AF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	AnalyticsId set to "Expected UE behaviour".

-	The Target of Analytic Reporting can be any UE or an Internal Group Identifier. This target refers to the UEs that are presumed to be located in the Area of Interest at the time indicated in the Observation period.

-	An Observation period, which indicates the time when the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.7.3.

### 6.7.2	Input Data

In order to produce "Expected UE behaviour" analytics, the NWDAF collects:

-	UE mobility analytics as defined in clause 6.10.3,

-	UE communication analytics as defined in 6.11.3, and

-	additional UE behavioural information as defined in the Table 6.7.2-1.

Table 6.7.2-1: UE behavioural information collected from 5GC NF(s)

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | Could be e.g. SUPI, which is used by NWDAF to correlate the UE behavioural information from different 5GC NFs. |
| UL or DL Packet Latency |  | Indicating the delay for uplink or downlink packets transfers for the UE. |
| UL or DL data rate |  | Indicating the bit rate for uplink or downlink packets transfers for the UE. |
| Frequent mobility re-registration | AMF | A stationary UE may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple re-registrations if the cells belong to different registration areas. The number of re-registrations may be an indication for abnormal behaviour. |

Editor's Note:	How the NWDAF gets input information (e.g. from OAM) related to UL or DL Packet latency or data rate is FFS.

### 6.7.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are used to expose the expected UE behaviour parameters as defined in Table 6.7.3-1 together with a UE group ID or a specific UE ID from the NWDAF to the consumer NF (e.g. UDM), indicating

The Analytic ID named "Expected UE behaviour " is defined with one or multiple combinations of the following analytic filters:

-	UE group ID or UE ID.

The output data of these analytics is a combination of the data included in Table 6.10.3-1 and Table 6.11.3-1.

* * * Next change * * * *

### 6.10.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytic results provided by the NWDAF could be UE mobility statistics, UE mobility predictions:

-	UE mobility statistics is defined in the Table 6.10.3-1.

Table 6.10.3-1: UE mobility statistics

| Information | Description |  |  |  |
| --- | --- | --- | --- | --- |
| UE group ID or UE ID |  | Identifies a group of UEs or a specific UE, e.g. internal group ID, SUPI List or SUPI. |  |  |
| UE location (1..max) |  |  |  |  |
| >Stationary indication | Identifies whether the UE is stationary or mobile, e.g. only on demand. (TS 23.682 [5], clause 5.10.1). |  |  |  |
| >UE Moving Trajectory | Identifies the UE's expected geographical movement (TS 23.502 [3], clause 4.15.6). Example: A planned path of movement |
| >UE location | TA or cells which the UE stays |  |  |  |
| >Time slot | Time slot that the UE moves to this location |  |  |  |
| >Average Duration | Average time period that the UE stays in this location |  |  |  |
| >Frequency | Number of times that the UE moves to this location |  |  |  |

-	UE mobility predictions is defined in the Table 6.10.3-2.

Table 6.10.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID |
| Predicted UE location (1..max) |  |
| >Stationary indication | Identifies whether the UE is stationary or mobile, e.g. only on demand. (TS 23.682 [5], clause 5.10.1). |
| >UE Moving Trajectory | Identifies the UE's expected geographical movement (TS 23.502 [3], clause 4.15.6). Example: A planned path of movement |
| >UE location | TA or cells where the UE may move into |
| >Time slot | Time slot within which the UE may move to this location |
| >Duration | Time period that the UE may stay in this location |
| >Confidence | Confidence of this prediction |

* * * Next change * * * *

### 6.11.3	Output Analytics

The NWDAF supporting data analytic on UE communication shall be able to provide the analytics to consumer NFs. The analytic results provided by the NWDAF could be the UE communication pattern statistics or predictions as defined in the Table 6.11.3-1;

Table 6.11.3-1: UE communication pattern statistics or predictions

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID |
| UE communications (1..max) |  |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not, e.g. only on demand. (TS 23.682 [5], clause 5.10.1). |
| > Periodic time | Interval Time of periodic communication (may be used together with 1) (TS 23.682 [5], clause 5.10.1). Example: every hour |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication (TS 23.682 [5], clause 5.10.1). Example: Time: 13:00-20:00, Day: Monday |
| > Communication duration time | Duration interval time of periodic communication or the scheduled communication (may be used together with 1) (TS 23.682 [5], clause 5.10.1). Example: 5 minutes |
| > Traffic characterization | Estimated UL/DL traffic volume |
| > Confidence | Confidence of the prediction if this is a communication pattern prediction |

* * * End of changes * * * *