3GPP TSG-SA2 Meeting #132	S2-1903927

# Xi'an, P.R. China, 08-12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1186 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing 5GS UP optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Verizon UK Ltd., Telecom Italia, Orange, Samsung, Huawei, HiSilicon, Convida Wireless, Gemalto, Sierra Wireless, BlackBerry UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-03-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FS_CIoT_5G has concluded to introduce 5GS UP optimization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce 5GS UP optimization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for user-plane optimization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changes >>>

### 5.31.X	User Plane CIoT 5GS Optimization

User Plane CIoT 5GS Optimization enables transfer of user plane data from CM-IDLE without the need for using the Service Request procedure to establish Access Stratum (AS) context in NG-RAN and UE.

If the following preconditions are met:

-	UE and AMF support User Plane CIoT 5GS Optimization (see clause 5.31.2),

-	the UE has established PDU sessions with active UP connection, i.e. AS context is established in NG-RAN and the UE,

then the RRC connection can be suspended by means of the UP Optimization Connection Suspend Procedure (see TS 23.502 [3]).

Based on a trigger from the NAS layer when a UE is in CM-IDLE, the UE shall attempt the UP Optimization Connection Resume procedure (see TS 23.502 [3]). If the UP Optimization Connection Resume procedure fails, the UE initiates the pending NAS procedure. To maintain support for User Plane CIoT 5GS Optimization for UE mobility across different NG-RAN nodes, the AS Context should be transferred between the NG-RAN nodes.

Editor's note:	Details of AS context transfer will be specified by RAN3.

By using the UP Optimization Connection Suspend Procedure:

-	the UE at transition into CM-IDLE stores the AS information;

-	NG-RAN stores the AS information, the NGAP UE association and the PDU session context for that UE;

-	AMF stores the NGAP UE association and a flag indicating that the UE has been suspended, interacts with the SMF to deactivate the user plane resources for the UE's PDU Sessions and enters CM-IDLE.

By using the UP Optimization procedure:

-	the UE resumes the connection with the network using the AS information stored during the UP Optimization Connection Suspend procedure;

-	the, potentially new, NG-RAN node notifies the AMF that the connection with the UE has been resumed and the AMF enters CM-CONNECTED.

Editor's note:	Details of the NGAP messages to be used for this procedure will be specified by RAN3.

Editor's note:	Details of early data support for UP optimization are FFS.

If an AMF has an NGAP UE association stored for a UE, the AMF and the previously involved NG-RAN node shall delete the stored NGAP association using the AN Release procedure TS 23.502 [3] in the following cases:

-	if the AMF receives for the UE a NAS message over another NGAP UE-association;

-	during Registration procedure with AMF change;

-	when the UE performs Initial Registration;

-	when the UE has been deregistered.

Editor's note:	Potential updates to the AN Release procedure are FFS.

## <<< End of changes >>>