3GPP TSG-SA2 Meeting #132	S2-1903930

# Xi'an, China, 8th Apr 2019 - 12th Apr 2019	(revision of S2-1903564)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1232 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NIDD delivery service and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, UANGEL, Nokia, Nokia Shanghai Bell, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incorrect service name is used in the procedure for NEF anchored MO/MT data transport: Nnef_NIDD_Delivery is not an AF/SMF service but an NEF service. Thus, this CR proposes to add new NEF/SMF service operations to handle the unstructured data in SMF and NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4.25.4, modified the step 3-4 in Figure 4.25.4-1 "NEF Anchored Mobile Originated Data Transport procedure" and the corresponding text with the new NEF service operation: Nnef_NIDD_DeliveryNotify. - In clause 4.25.5, modified the step 3,5 in Figure 4.25.5-1 "NEF Anchored Mobile Terminated Data Transport" and the corresponding text with a new SMF service operation - In clause 5.2.6.1, added new NEF service operations: Nnef_NIDD_DeliveryNotify (Subscribe/Notify) for which NEF forwards the MO data to AF in subscribe/notify semantics - In clause 5.2.6.14, corrected the Nnef_NIDD service name - In clause 5.2.6.14.2, corrected the service operation name and modified the description - In clause 5.2.6.14.x, added a new service operation: Nnef_NIDD_DeliveryNotify. - In clause 5.2.8.1, added a new service operation: Nsmf_NIDD_Delivery (Request/Response) for which NF consumer (NEF) submits the MT data to SMF. - In clause 5.2.8.x, 5.2.8.x.1, and 5.2.8.x.2, added a new service operation: Nsmf_NIDD_Delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedure due to incorrect service operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.25.4, 4.25.5, 5.2.6.1, 5.2.6.14, 5.2.6.14.2, 5.2.6.14.x, 5.2.8.1, 5.2.8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

4.25.4	NEF Anchored Mobile Originated Data Transport

Figure 4.25.4-1 illustrates the NEF Anchored Mobile Originated Data Transport procedure.

Figure 4.25.4-1: NEF Anchored Mobile Originated Data Transport procedure

1.	The UE sends a NAS message with unstructured data according to Steps 0-4 of the procedure for UPF anchored Mobile Originated Data Transport in CP CIoT 5GS Optimisation (see clause 4.24.1). The Reliable Data Service header is included if the Reliable Data Service is enabled.

2.	The SMF sends the Nnef_NIDD_Delivery Request (User Identity, NEF ID, unstructured data) message to the NEF.

3.	When the NEF receives the unstructured data and finds an NEF PDU Session context and the related T8 Destination Address, then it sends the unstructured data to the AF that is identified by the T8 Destination address in a Nnef_NIDD_DeliveryNotify Request (GPSI, unstructured data, Reliable Data Service Configuration). If no T8 Destination address is associated with the UE's PDN connection, the data is discarded, the Nnef_NIDD_DeliveryNotify Request is not sent, and the flow continues at step 5. The Reliable Data Service Configuration is used to provide the AF with additional information such as indicate if an acknowledegement was requested and port numbers for originator application and receiver application, when the Reliable Data Service is enabled.

Editor's note:	It is left to Stage 3 whether or not the NEF aggregates Nnef_NIDD_DeliveryNotify Request messages to the AF.

4.	The AF responds to the NEF with a Nnef_NIDD_DeliveryNotify Response (Cause).

5.	The NEF sends Nnef_NIDD_Delivery Response to the SMF. If the NEF cannot deliver the data, e.g. due to missing AF configuration, the NEF sends an appropriate error code to the SMF.

4.25.5	NEF Anchored Mobile Terminated Data Transport

Figure 4.25.5-1 illustrates the procedure using which the AF sends unstructured data to a given user as identified via External Identifier or MSISDN.

Figure 4.25.5-1: NEF Anchored Mobile Terminated Data Transport

1.	If AF has already activated the NIDD service for a given UE, and has downlink unstructured data to send to the UE, the AF sends a Nnef_NIDD_Delivery Request (GPSI, TLTRI, unstructured data, Reliable Data Service Configuration) message to the NEF. Reliable Data Service Configuration is an optional parameter that is used to configure the Reliable Data Service, it may be used to indicate if a Reliable Data Service acknowledgement is requested and port numbers for originator application and receiver application.

2.	The NEF determines the 5GS QoS Flow Context based on the DNN associated with the NIDD configuration and the User Identity. If an NEF 5GS QoS Flow Context corresponding to the GPSI included in Step 1 is found then the NEF checks if the AF is authorised to send data and if it does not exceed its quota or rate. If these checks fail, then steps 3-5 are skipped and an appropriate error code is returned in step 6.

3.	The NEF forwards the unstructured data to the SMF using Nsmf_NIDD_Delivery Request.

4.	The unstructured data is sent as for steps 2 to 9 of the procedure for UPF Anchored Mobile Terminated Data Transport in Control Plane CIoT 5G Optimisation (see clause 4.24.2).

If the Reliable Data Service header indicates that the acknowledgement is requested, then the UE shall respond with an achnowledgement to the DL data that was received.

5.	The SMF sends a Nsmf_NIDD_Delivery Response (cause) message towards the NEF acknowledging the NIDD Submit Request from NEF received in step 3.

If the Reliable Data Service header indicates that the acknowledgement is requested, then the UE shall respond with an achnowledgement to the DL data that was received.

6.	The NEF sends a Nnef_NIDD_Delivery Response (cause) to the AF.

The Reliable Data Service Acknowledgement Indication is used to indicate if an acknowledgement was received from the UE for the MT NIDD. If the Reliable Data Service was requested in step 1, then the Nnef_NIDD_Delivery Response is sent to the AF after the acknowledgement is received from the UE or, if no acknowledgment is received, then the MT NIDD Submit Response is sent to the AF with a cause value indicating that no acknowledgement was received.

2nd CHANGE

5.2.6	NEF Services

5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | SMF |
|  | Trigger | Trigger | AF |
| Nnef_NIDD | Delivery | Request/Response | AF, SMF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request / Response | AF |

3rd CHANGE

5.2.6.14	Nnef_NIDD service

5.2.6.14.1	General

See clauses 4.25.4 and 4.25.5.

5.2.6.14.2	Nnef_NIDD_Delivery service operation

Service operation name: Nnef_ NIDD_Delivery

Description: This service operation is used by the NF consumer to deliver the unstructured data between NF consumer and NEF to support NIDD via NEF.

Inputs (required): User Identity, NEF ID, unstructured data, TLTRI (Optional), Reliable Data Service Configuration (Optional).

Outputs (required): Cause.

5.2.6.14.x	Nnef_NIDD_DeliveryNotify service operation

Service operation name: Nnef_ NIDD_DeliveryNotify

Description: This service operation is used by the NEF to forward the unstructured data to NF consumer to support NIDD via NEF.

Inputs (required): User Identity, unstructured data, TLTRI (Optional), Reliable Data Service Configuration (Optional).

Outputs (required): Cause.

4th CHANGE

5.2.8	SMF Services

5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF/I-SMF |
|  | Update | Request/Response | V-SMF/I-SMF, H-SMF |
|  | Release | Request/Response | V-SMF/I-SMF |
|  | CreateSMContext | Request/Response | AMF |
|  | UpdateSMContext | Request/Response | AMF |
|  | ReleaseSMContext | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF/I-SMF |
|  | Context | Request/Response | AMF, I-SMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF |
|  | Unsubscribe |  | NEF, AMF |
|  | Notify |  | NEF, AMF |
|  | AppRelocationInfo |  | AF |
| Nsmf_NIDD | Delivery | Request/Response | NEF |

5th CHANGE

5.2.8.x	Nsmf_NIDD Service

5.2.8.x.1	General

This service is used for NIDD transfer between SMF and another NF. See clause 4.25.5.

5.2.8.x.2	Nsmf_NIDD_Delivery service operation

Service operation name: Nsmf_NIDD_Delivery

Description: This service operation is used by the NF consumer to deliver the unstructured data between NF consumer and SMF to support NIDD via NEF.

Inputs (required): User Identity, PDU Session ID, unstructured data, Reliable Data Service Configuration (Optional).

Outputs (required): Cause.

END OF CHANGES