3GPP TSG-SA2 Meeting #132	S2-1904013

# Xi'an, P.R. China, 08-12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1186 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing 5GS UP optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Verizon UK Ltd., Telecom Italia, Orange, Samsung, Huawei, HiSilicon, Convida Wireless, Gemalto, Sierra Wireless, BlackBerry UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-03-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FS_CIoT_5G has concluded to introduce 5GS UP optimization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce 5GS UP optimization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for user-plane optimization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changes >>>

### 5.31.X	User Plane CIoT 5GS Optimization

User Plane CIoT 5GS Optimization enables transfer of user plane data from CM-IDLE without the need for using the Service Request procedure to establish Access Stratum (AS) context in NG-RAN and UE.

If the following preconditions are met:

-	UE and AMF support User Plane CIoT 5GS Optimization (see clause 5.31.2),

-	the UE has established PDU sessions with active UP connection, i.e. AS context is established in NG-RAN and the UE,

then the RRC connection can be suspended by means of the UP Optimization Connection Suspend Procedure (see TS 23.502 [3]).

Based on a trigger from the NAS layer when a UE is in CM-IDLE, the UE shall attempt the UP Optimization Connection Resume procedure (see TS 23.502 [3]). If the UP Optimization Connection Resume procedure fails, the UE initiates the pending NAS procedure. To maintain support for User Plane CIoT 5GS Optimization for UE mobility across different NG-RAN nodes, the AS Context should be transferred between the NG-RAN nodes.

Editor's note:	Details of AS context transfer will be specified by RAN3.

By using the UP Optimization Connection Suspend Procedure:

-	the UE at transition into CM-IDLE stores the AS information;

-	NG-RAN stores the AS information, the NGAP UE association and the PDU session context for that UE;

-	AMF stores the NGAP UE association and other information necessary to later resume the UE, interacts with the SMF to deactivate the user plane resources for the UE's PDU Sessions and enters CM-IDLE.

Editor's note:	How to delete the stored NGAP association and UE context and in which scenarios is FFS.

By using the UP Optimization procedure:

-	the UE resumes the connection with the network using the AS information stored during the UP Optimization Connection Suspend procedure;

-	the NG-RAN node notifies the AMF that the connection with the UE has been resumed and the AMF enters CM-CONNECTED and interacts with the SMF to activate the user plane resources for the UE's PDU Sessions.

Editor's note:	Details of early data support for UP optimization are FFS.

## <<< End of changes >>>