3GPP TSG-SA WG2 Meeting #132	S2-1904144

# Xi’an, China, 8 - 12 April 2019 	(revision of S2-1903368)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1120 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Filtering own address for Ethernet PDU Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC, 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Frames with source address equal to the address used in the uplink imply a loop; such frames do not exist when the network is at steady state, but may exist at network topology changes. See discussion in S2-1903367. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SMF should ensure that downlink traffic that has the same source MAC address as the MAC address used by a UE for the UL traffic on the same PDU Session is dropped. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ethernet forwarding loops may occur at network topology changes, which risk the stability of the whole network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 5.8.2.5	Control of User Plane Forwarding

The SMF controls user-plane packet forwarding for traffic detected by a PDR by providing a FAR with instructions to the UPF, including:

-	Forwarding operation information;

-	Forwarding target information.

The details of the forwarding target and operation will depend on the scenario and is described below. The following forwarding functionality is required by the UPF:

-	Apply N3 /N9 tunnel related handling, i.e. encapsulation.

-	Forward the traffic to/from the SMF, e.g. as described in Table 5.8.2.5-1.

-	Forward the SM PDU DN Request Container from SMF to DN-AAA server

-	Forward the traffic according to locally configured policy for traffic steering.

Data forwarding between the SMF and UPF is transmitted on the user plane tunnel established on N4 interface, defined in TS 29.244 [65].

Scenarios for data forwarding between the SMF and UPF are defined as below:

Table 5.8.2.5-1: Scenarios for data forwarding between the SMF and UPF

|  | Scenario description | Data forwarding direction |
| --- | --- | --- |
| 1 | Forwarding of user-plane packets between the UE and the SMF e.g. DHCP signalling. | UPF to SMF SMF to UPF |
| 2 | Forwarding of packets between the SMF and the external DN e.g. with DN-AAA server | UPF to SMF SMF to UPF |
| 3 | Forwarding of packets subject to buffering in the SMF. | UPF to SMF SMF to UPF |
| 4 | Forwarding of End Marker Packets constructed by the SMF to a downstream node. | SMF to UPF |
| 5 | Forwarding of user data using Control Plane CIoT 5GS Optimisation | UPF to SMF SMF to UPF |

When configuring an UPF acting as PSA for an Ethernet PDU Session Type, the SMF may instruct the UPF to route the DL traffic based on the MAC address(es) used by the UE for the UL traffic.

In that case:

-	broadcast and/or multicast DL traffic on the N6 Network Instance targets every DL PDU Session (corresponding to any N4 Session) associated with this Network Instance.

-	for uplink broadcast and/or multicast traffic received by the UPF over a PDU session on a N3/N9 interface, the UPF should forward the traffic to the N6 interface and downlink to every PDU session (except the one of the incoming traffic) associated with the same N6 Network Instance.

The SMF/UPF should ensure that downlink traffic that has the same source MAC address as the MAC address(es) used for UL traffic on the same PDU Session is dropped. When the Ethernet PDU Session Information is set in the PDR, the UPF drops such DL traffic based on the MAC address(es) used for the UL traffic. Alternatively, in case Ethernet PDU Session Information is not set in the PDR, the SMF may installs explicit downlink filters in UPF for this purpose.

NOTE 1:	This is done in order to avoid the formation of loops in case of Ethernet topology changes, e.g., due to topology changes in the Data Network and/or topology changes in local networks behind the UE and/or topology changes due to UPF relocation.

NOTE 2:	In order to handle scenarios where a device behind a UE is moved from one UE to another UE, the SMF/UPF can remove a MAC address from the dropping rules in case that MAC address has not been detected in UL traffic for a period of time.

For ARP/IPv6 Neighbour Solicitation traffic, a SMF's request to respond to ARP/IPv6 Neighbour Solicitation based on local cache information or to redirect such traffic from the UPF to the SMF overrules the traffic forwarding rules described above.

NOTE 3:	Local policies in UPF associated with the Network Instance can prevent local traffic switching in the UPF between PDU Sessions either for unicast traffic only or for any traffic. In the case where UPF policies prevent local traffic switching for any traffic (thus for broadcast/multicast traffic) some mechanism such as responding to ARP/ND based on local cache information or local multicast group handling is needed to ensure that upper layer protocol can run on the Ethernet PDU sessions.

If allowed by local UPF (acting as PSA) policies, for Ethernet traffic with unknown unicast destination MAC addresses, the UPF should forward the traffic in the same manner as described above for the broadcast and/or multicast traffic handling.

The SMF may ask to get notified with the source MAC addresses used by the UE and provide UPF with corresponding forwarding rules related with these MAC addresses.

* * * * End of Change * * * *