3GPP TSG-SA WG2 Meeting #132	S2-1904202

# Xi'an, China, April 8 – 12, 2019												 (revision of S2-1904087)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1164 | rev | 2 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on GBR QoS Flow establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In MA PDU Session, GBR QoS Flow is supported by establishing user plane resources over single access. However, it is not clear how the traffic switching can be supported for GBR QoS Flow. For downlink traffic, the UPF should notify the SMF of traffic switching so that the SMF establishes GBR QoS Flow over the target access network. For uplink, in order to avoid any signalling exchange between the UE and network, it is proposed that the SMF provides the ATSSS rule so that uplink traffic for GBR traffic is send over a single access. Based on this, the SMF knows in which access the GBR QoS Flow shall be established. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UPF notifies the SMF of traffic swithcing decision. Based on the notification, the SMF moves GBR QoS Flow to the other access. Clarify that the ATSSS rules provided to the UE only allows the UE to steer traffic over a single access for GBR traffic so that network knows in which access the UE sends GBR traffic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For traffic using GBR QoS Flow, QoS is not supported if traffic switching happens. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.32.4	QoS Support

The 5G QoS model for the Single-Access PDU Session is also applied to the Multi-Access PDU Session, i.e. the QoS Flow is the finest granularity of QoS differentiation in the Multi-Access PDU Session. One difference compared to the Single-Access PDU Session, is that in a MA PDU Session there can be separate user-plane tunnels between the AN and the PSA, each one associated with a different access. However, the QoS Flow is not associated with specific access, i.e. it is access agnostic, so the same QoS is supported when the traffic is distributed over 3GPP and non-3GPP accesses. The SMF shall provide the same QFI in 3GPP and non-3GPP accesses so that the same QoS is supported in both accesses.

A QoS Flow of the MA PDU Session may be either Non-GBR or GBR depending on its QoS profile. For a Non-GBR QoS Flow, the SMF provides a QoS profile to both access networks, if the UE is in CM-CONNECTED state in both accesses. For a GBR QoS Flow, the SMF shall provide a QoS profile to a single access network as follows:

-	If the PCC rule allows a GBR QoS Flow in a single access, the SMF provides the QoS profile for the GBR QoS Flow to the access network allowed by the PCC rule.

-	If the PCC rule allows a GBR QoS Flow in both accesses, the SMF decides to which access network to provide the QoS profile for the GBR QoS Flow based on its local policy (e.g. the access where the traffic is ongoing according to the Multi Access Routing rules).

For a GBR QoS Flow, traffic splitting is not supported because the QoS profile is provided to a single access network at a given time. If the UPF determines that it cannot send GBR traffic over the current ongoing access e.g. based on the N4 rules and access availability and unavailability report from the UE as described in clasuse 5.32.5.3, the UPF shall notify the SMF. Based on the notification, the SMF decides whether to move GBR QoS Flows to the other access. If the SMF decides to move GBR QoS Flow, the SMF triggers PDU Session Modification procedure and, afterwards moves GBR QoS Flow to the other access.

NOTE:	The ATSSS rule for GBR QoS Flow only allows the UE to steer traffic over a single access so that network knows in which access the UE sends GBR traffic. If the network wants to move GBR QoS Flow to the other access, the network needs to update ATSSS rule of the UE.

When the MA PDU Session is established, the SMF provides QoS rule(s) to the UE, which are applied by the UE as specified in clause 5.7.1.4. The QoS rule(s) are commonly used for both 3GPP access and non-3GPP access, so the QoS classification is independent of ATSSS rules.

The derived QoS rule generated by Reflective QoS is applied independently of the access on which the RQI was received.

* * * * End of Changes * * * *