3GPP TSG-SA WG2 Meeting #132	S2-1904295

Xi’an - China, 8 – 12 April 2019	(revision of S2-1903118)

Source:	Ericsson

Title:	EPC/5GC interworking procedures

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This pCR propose procedures for interworking between EPC and 5GC GMLCs.

# 1. Discussion

To support EPC interworking scenarios it is in clause 4.1 stated that a “common interface shall be used for the location request from LCS client or AF”. As an LCS Client or external AF normally is unaware of is target UE is served by EPS or 5GS there are need for procedures where LCS Client or AF and UE is served by different system types.

# 2. Proposal

It is proposed to include below text in TS 23.273

***** BEGIN 1st CHANGE *****
All new text

## 6.x	Procedures with interaction between 5GC and EPC

### 6.x.1	MT-LR Procedure

Figure 6.x.1-1 illustrates network positioning requested by the LCS clients or the AF external to the PLMN for the case where the LCS Client or AF is associated with a GMLC in either 5GC or EPC domain and the UE is served by the opposite domain. Both non-roaming and roaming scenarios are described. For non-roaming scenario both the case of separate H-GMLC and V-GMLC connected with NL3 reference point and the case with combined H-GMLC and V-GMLC are described.

Figure 6.x.1-1: MT-LR procedure with 5GC and EPC interaction

1a.	If the procedure is initiated towards an EPS R/H-GMLC, steps 1-7 in TS 23.271 [4] clause 9.1.1 are performed.

1b.	If the procedure is initiated towards an 5GS H-GMLC, steps 1-3 in clause 6.1.2 are performed.

2.	For non-roaming case and if H_GMLC and V-GMLC are combined, this step is skipped. Otherwise the H-GMLC may receive an address of a V-GMLC (together with the network address of the current serving node) from the UDM/HSS in step 1a/1b. The H-GMLC then sends the location request to the V-GMLC by invoking the Ngmlc_ProvideLocation service operation towards the V-GMLC. If the result of privacy check indicates that the notification (and verification) based on current location is needed, the H-GMLC shall send a location request to the V-GMLC indicating "positioning allowed without notification".

3a.	If the UE is served by 5GS, steps 5-14 in clause 6.1.2 are performed.

3b.	If the UE is served by EPS, steps 2-9 or 9b in TS 23.271 [4] clause 9.1.15 are performed.

4.	For non-roaming case and if H_GMLC and V-GMLC are combined, this step is skipped. Otherwise the V-GMLC forwards the location estimation of the target UE, its age, its accuracy and optionally the information about the positioning method to the H-GMLC.

5.	If the privacy check in step 1a/1b indicates that further privacy checks are needed, the (H)-GMLC shall perform an additional privacy check in order to decide whether the (H-)GMLC can forward the location information to the LCS client or AF or send a notification if the result of the privacy check requires the notification and verification based on current location. One example when this additional privacy check is needed is when the target UE user has defined different privacy settings for different geographical locations. When an additional privacy check is not needed, the (H-)GMLC skips steps 6-8.

6.	For non-roaming case and if H_GMLC and V-GMLC are combined, this step is skipped. Otherwise if the result of privacy checks in step 5 indicates that the notification (and verification) based on current location is needed, and in the case of roaming, the (H-)GMLC shall send a location request to the V-GMLC indicating "notification only".

7a.	If the UE is served by 5GS, steps 18-22 in clause 6.1.2 are performed.

7b.	If the UE is served by EPS, the EPC post positioning notification and verification procedure in TS 23.271 [4] clause 9.1.18 is performed.

8.	For non-roaming case and if H_GMLC and V-GMLC are combined, this step is skipped. Otherwise the V-GMLC forwards an indication of the result of notification and verification procedure to the H-GMLC.

9a.	If the procedure was initiated in EPS, steps 15-16 in TS 23.271 [4] clause 9.1.1 are performed.

9b.	If the procedure was initiated in 5GS, steps 24 in clause 6.1.2 are performed.

NOTE:	As LCS Client/AF and UE are in different domains either steps 1a, 3a, 7a and 9a or steps 1b, 3b, 7b and 9b are performed.

***** Next CHANGE *****
All new text

### 6.x.2	MO-LR Transfer to a Third Party Procedure

Figure 6.x.2-1 illustrates the UE requesting its location to be sent to a Third Party (i.e. an external LCS Client) for the case where the LCS Client is associated with a GMLC in either 5GC or EPC domain and the UE is served by the opposite domain. Both non-roaming and roaming scenarios are described. For non-roaming scenario both the case of separate H-GMLC and V-GMLC connected with NL3 reference point and the case with co-located H-GMLC and V-GMLC are described

Figure 6.x.2-1: MO-LR procedure with 5GC and EPC interaction

1a.	If the procedure is initiated in EPS, steps 1-6 in TS 23.271 [4] clause 9.2.6 are performed.

1b.	If the procedure is initiated in 5GS, steps 1-7 in clause 6.2 are performed.

2.	For non-roaming case and if H_GMLC and V-GMLC is combined, this step is skipped. Otherwise the V-GMLC invokes the Ngmlc_LocationUpdateNotify service operation towards to the H-GMLC including the information received in step 1a/1b.

3a.	If the LCS Client is associated to a 5GS H-GMLC, steps 9-10 in clause 6.2 are performed.

3b.	If the LCS Client is associated to a EPS R/H-GMLC, steps 8-11 in TS 23.271 [4] clause 9.2.6 are performed.

4.	For non-roaming case and if H_GMLC and V-GMLC is combined, this step is skipped. Otherwise if the identified LCS Client is not accessible, the V-GMLC sends a Ngmlc_LocationUpdateNotify service response to AMF with an appropriate error cause. Otherwise, the response shall include an acknowledgement. The message shall specify whether the location estimate of the UE has been handled successfully by the identified LCS Client, and if not, the corresponding error cause obtained in step 3a/3b.

5a.	If the procedure was initiated in EPS, steps 13-14 in TS 23.271 [4] clause 9.2.6 are performed.

5b.	If the procedure was initiated in 5GS, steps 12-13 in clause 6.2 are performed.

NOTE:	As LCS Client and UE are in different domains either steps 1a, 3a and 5a or steps 1b, 3b, and 5b are performed.

***** End of CHANGE *****