SA WG2 Meeting #132 	S2-1904308

# Xi’an, China, 8th Apr – 12th Apr, 2019							                  (revision of S2-1903555)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1179 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Traffic offload by UPF controlled by the I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - See paper S2-1903554 on how the information is transferred at N16a interface to support traffic offload by UPF controlled by the I-SMF. - Some other editorial change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify which N4 information is sent from SMF to I-SMF, and how I-SMF generate N4 rules based on N4 information. - Editorial change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is not clear how the I-SMF generate N4 rules based on this information. - Some unclear description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.6, 5.34.6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.34.6	Interaction between I-SMF and SMF for the support of traffic offload by UPF controlled by the I-SMF

5.34.6.1 General

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/Branching Point in VPLMN is not supported in HR case. It applies for the architectures described in clauses 5.34.4 and 5.34.5

When the I-SMF is inserted into a PDU Session, e.g. during PDU Session establishment or due to UE mobility, the I-SMF provides the DNAI list it supports to the SMF. Based on the DNAI list information received from I-SMF, the SMF provides the DNAI(s) of interest for this PDU Session for local traffic steering to the I-SMF e.g. immediately or when a new or updated or removed PCC rule(s) is/are received. The DNAI(s) of interest is derived from PCC rules.

The I-SMF is responsible for the insertion, modification and removal of UPF(s) to ensure local traffic steering. The SMF does not need to have access to local configuration or NRF output related with UPF(s) controlled by I-SMF. Based on the DNAI(s) of interest for this PDU Session for local traffic steering and UE location the I-SMF determines which DNAI(s) are to be selected, selects UPF(s) acting as UL CL/BP and/or PDU Session Anchor based on selected DNAI, and insert these UPF(s) into the data path of the PDU Session.

When a UL CL/BP has been inserted, changed or removed, the I-SMF indicates to the SMF that traffic offload have been inserted, updated or removed for a DNAI, providing also the IPv6 prefix that has been allocated in case a new IPv6 prefix has been allocated for the PDU Session.

From now on the SMF and I-SMF interactions entail:

-	Notifying the SMF with the new Prefix (multi-Homing case): the SMF is responsible of Issuing Router advertisement to the UE including the priorities provided to the UE to select among the prefixes related with the PDU Session.

-	N4 interactions related with traffic offloading. The SMF provide N4 information to the I-SMF for how the traffic shall be detected, enforced, monitored in UPF(s) controlled by the I-SMF: the SMF issues requests to the I-SMF containing N4 information to be used for creating / updating /removing PDR, FAR, QER, URR, etc. The N4 information provided by the SMF to the I-SMF are described in clause 5.34.6.x.

-	Receiving N4 notifications related with traffic usage reporting: the I-SMF forwards to the SMF N4 information corresponding to UPF notifications related with traffic usage reporting; the SMF aggregates and constructs usage reports towards PCF/CHF.

-	Sending Trace Requirements to the I-SMF; upon reception of Trace Requirements from the SMF the I-SMF shall provide Trace Requirements on the N4 interface towards the UPF(s) it is controlling.

NOTE:	How the SMF decides what traffic steering and enforcement actions are enforced in UPF(s) controlled by I-SMF is left for implementation.

The I-SMF is responsible of the N4 interface towards the local UPF(s) including:

  - the usage of AN Tunnel Info received from the 5G AN via the AMF in order to build PDR and FAR,

  - the choice (by I-SMF, by UPF) of the CN Tunnel Info at the UPF controlled by the I-SMF,

  - to the of control UPF actions when the UP of the PDU Session becomes INACTIVE,

  - the sending of end markers at Hand-Over not involving a change of the UPF interfacing an UPF not controlled by the I-SMF.

Next CHANGE

5.34.6.x	N4 information sent from SMF to I-SMF

The SMF generates N4 information based on the DNAI(s) indicated by the I-SMF as being used and PCC rules associated with these DNAI(s). This N4 information is sent from the SMF to the I-SMF after UL CL/Branching Point insertion/update/removal, and the I-SMF uses this N4 information to derive rules installed in the UPFs controlled by the I-SMF.

The N4 information is based on the rules and parameters defined in clause 5.8.2.11, i.e. PDR, FAR, URR and QER. The following parameters are not included in the N4 information sent by the SMF and are generated by the I-SMF:

-	CN tunnel info.

-	Network instance (if needed).

The N4 information exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with an N16a association ID allowing the SMF to modify or delete the N4 information at a later stage.

The I-SMF generates an N4 Session ID and for each rule a Rule ID (unless the ones received from the SMF can be used) and maintains a mapping between the locally generated identifiers and the ones received from the SMF. The I-SMF replaces those IDs in the PDR(s), QER(s), URR(s) and FAR(s) received from the SMF. When the I-SMF receives the N4 information, the Network instance (if needed) and CN tunnel info included in the rules sent to the UPF is generated by I-SMF.

The I-SMF shall determine the appropriate setting based on the received Forwarding policy and local configuration.

END OF CHANGES