SA WG2 Meeting #132	S2-1904348

April 8 – 12, 2019, Xi’an, China	(revision of S2-1903125)

Source:	Ericsson

Title:	QoS model for W-5GAN

Document for:	Approval of figure for QoS model

Agenda Item:	6.7

Work Item / Release:	5WWC/Rel-16

Abstract of the contribution: This contribution proposes a figure for the QoS Model.

# Introduction

This contribution proposes a figure depicting the principle for classification and user plane marking for QoS flows and mapping to the access network resources for TS 23.316. It uses the QoS model in TS 23.501 clause 5.7.

# Proposal

It is proposed to update TS 23.316 as follows:

**** First Change ****

## 4.5	QoS model

Editor's note:	This clause includes delta related to QoS model defined in TS 23.501 [2] clause 5.7. Clauses can be added if needed.

Editor's note:	QoS model should be verified with BBF/CableLabs.

The QoS model of TS 23.501 clause 5.7 is applicable to the W-5GAN scenario, with the difference that the W-AGF acts as an Access Network (AN).

The principle for classification and marking of User Plane traffic and mapping of QoS flows to W-UP resources is illustrated in Figure 4.5-1.

Figure 4.5-1: The principle for classification and User Plane marking for QoS Flows and mapping to W-UP resources

On the user-plane:

During the setup of W-UP resources in the W-5GAN it should be supported to bind W-UP resources to individual QFI(s). One W-UP resource can be indicated as the default W-UP resource. There shall be one and only one Default W-UP resource per PDU session. The 5G-RG shall send all QoS Flows to this W-UP resource for which there is no mapping information to a specific W-UP resource.

Handling of UL traffic by the 5G-RG:

When the 5G-RG transmits an UL PDU, the 5G-RG should determine the QFI associated with the UL PDU (by using the QoS rules of the PDU Session), it shall encapsulate the UL PDU inside an access layer dependent W-UP packet and shall forward the W-UP packet to W-AGF via the W-UP resource associated with this QFI. The header of the W-UP packet carries the QFI associated with the UL PDU.

Handling of DL traffic by W-AGF:

When the W-AGF receives a DL PDU via N3, the W-AGF the identity of the PDU Session and optionally uses QFI in order to determine the W-UP resource to use for sending the DL PDU to the 5G-RG. The W-AGF encapsulates the DL PDU inside an access layer dependent W-UP packet. The header of the W-UP packet carries the QFI associated with the DL PDU. The W-AGF may include also in the W-UP header a Reflective QoS Indicator (RQI), which shall be used by the 5G-RG to enable reflective QoS.

The W-AGF will map 5QI received from the 5GC into access-specific QoS parameters relevant to the wireline access network.

Editor’s note: The carrying of QFI in the W-UP header is to be verified with BBF/Cablelabs.

NOTE: The mapping of 5QI to W-5GBAN QoS parameters is documented by the BBF for W-5GBAN in [ref]. The mapping of 5QI to W-5GCAN QoS parameters is documented by CableLabs for W-5GCAN in [ref].

**** End of First Change ****