3GPP TSG-SA WG2 Meeting #132	S2-1904377

# 8 - 12 April 2019. Xi’an P.R. China.

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0233 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Background data transfer controlled in application layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | xBDT |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The xBDT WID was approved in previous SA2 meeting, where some of companies has the requirement to send the Background Data at a give location and/or given time. Actually this BDT service can be well performed by Application layer usng existing URSP as defined in subclause 6.6.2. To elaborate how to use URSP to realize BDT service, we propose to add the related description as a informative text in TS 23.503. Note: This CR does not mean to necessarily exlcude a 3GPP layer based solution i.e. provisioning BDT rule to UE, but it offers an alternative to fulfill the requirement of BDT without any normative change. The objectives of the xBDT WID state that the following aspects are intended to be specified - The existing URSP framework will be used for delivery of policy information related to background data transfer from 5G core network to the UE. For this, the paper reuses URSP as well as UCU procedure. - The policy information content will define time window and location criteria that need to be met for background data transfer. For this, application layer can control the BDT transmisting per time and location; - How and when the PCF activates/disbributes the policies related with background data transfer to the UEs will be defined. For this, clause 6.1.2.4 Negotiation for future background data transfer can be reused to trigger UE policy provisioning; - There is a single dedicated PDU Session that is used for background data transfer which is established and released based on the background data transfer policies. For this, the UE will establish PDU Sessions per URSP rule for BDT while release it by implementation. (NOTE: releasing PDU Session or not belongs to UE’s own behavior which does not affect any of BDT services) - The policy should consider to avoid huge number of UE sending the data and or signalling to the network at the same time. For this, URSP can be reused to reduce any additional policy overhead as much as possible to save control-plane. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description to elaborate how to use URSP to realize BDT service as an alternative. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Nor clear how to ralize BDT using URSP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | To be a new Annex in 23.503 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

Annex X (Informative): using Rel-15 URSP for Background Data transfer (BDT) service

The Application in mobile phone collects the background data until the certain condition is met e.g. at given time and/or location, then sending the Background Data Transfer (BDT) to UEwhen the certain condition is met. And UE asscoaites the Background Data to a PDU Session based on URSP evaluation, as defined in subclause 6.6.2.

NOTE X: The PDU Session specifically used for BDT can be defined and provisioned to UE based on SLA between MNO and the third party.

When data transferring is over, UE moves out of specific area, or the time window for BDT has passed, the Application stops the Background data transferring, and UE may remove the PDU session used for BDT based on UE implementation.

* * * * End of Change * * * *