3GPP TSG-SA WG2 Meeting #132	S2-1904612

# 8th –12th April, 2019, Xi’an, China	(revision of S2-1903585)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1190 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving editor's note on eSBA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 5.29.3, we have below ENs on outcome of eSBA study: Editor’s note: Whether and how the above bullet needs to be updated due to eSBA is FFS Editor's note: Whether there are impacts to SMF selection to select this dedicated SMF is FFS. From eSBA CR (1010), the NF set concept has been introduced for an interchangeable NF instances of the same type, supporting the same services and the same Network Slice(s). Based on the eSBA works, it is propose to introduce the “dedicated SMF set” can also be responsible for managing all the PDU Sessions of a 5G-LAN group. However, it is not logic to refer everytime Set concept, as anyway eSBA is more generic and applicable for NFs/NF Services in the network. Thus, it is propose to remove the EN and assume that redundancy option is applicable as noted in the NOTE. In addition, to conclude on the SMF selection, first we need to address the EN on “DNN association with a 5G-LAN group”, it is proposing to refer clause 6.3.2 for SMF selection, based on the outcome, clause 6.3.2 need to be updated if necessary. Thus, it is remove with SMF selection clause reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on eSBA normative work, availability concept (e.g. NFs/NF Service set) is more generic and it application for whole TS, thus it is propose to remove eSBA EN. In addition, for the SMF selection referred the clause 6.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Addressing the editor’s note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.29.3	PDU Session management

Session management as defined for 5GS in clause 5.6 is applicable to 5G-LAN-type services with the following clarifications:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G-LAN group

-	A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G-LAN group and SMF selection is described in clause 6.3.2.

NOTE:	Having a dedicated SMF serving a 5G-LAN group does not contradict that redundancy solutions can be used to achieve high availability.

-	A DNN is associated with a 5G-LAN group

Editor's note:	The above bullet is FFS.

-	The UE provides a DNN associated with the 5G-LAN group to access the 5G LAN-type services for that 5G-LAN group, using the PDU Session Establishment procedure described in TS 23.502 [3], clause 4.3.2

-	During establishment of the PDU Session, secondary authentication as described in clause 5.6.6 and in TS 23.502 [3], clause 4.3.2.3, may be performed in order to authenticate and authorize the UE for accessing the DNN associated with the 5G-LAN group. Authentication and authorization for a DNN using secondary authentication implies authentication and authorization for the associated 5G-LAN group. There is no 5G-LAN group specific authentication or authorization defined.

-	The SM level subscription data for a DNN and S-NSSAI available in UDM, as described in clause 5.6.1, applies to a DNN associated to a 5G-LAN group.

-	Session management related policy control for a DNN as described in TS 23.502 [3], is applicable to a DNN associated to a 5G-LAN group. This includes also usage of URSP, for the UE to determine how to route outgoing traffic to a PDU Session for a DNN associated to a 5G-LAN group.

**** End of Changes ****