3GPP TSG-SA2 Meeting #132	S2-1904630

# Xi'an, China, 8th Apr 2019 - 12th Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1120 | rev | 1 | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the EN in Bindings between the consumer and producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-03-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align the binding procedures with the NF Set and NF Service Set concepts, and resolve the related Editor’s Note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the supported binding addresses, aligned with the NF Set and NF Service Set concepts. Add a new clause for update of the bindings between the consumer and producer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specification. Unclear how the bindings are updated and released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.11, 4.17.11.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ****************

*** BEGIN CHANGES ***

### 4.17.11 Binding between NF service consumer and NF service producer

#### 4.17.11.1 General

This clause describes the procedures to establish binding between the NF service consumer and producer.

Binding can be used to indicate suitable target producers for NF service instance selection and reselection.

A short-lived binding between the NF service consumer and producer can be used when the next service operation is expected to come quickly after the previous operation is completed, in order to inform the NF service consumer to re-use the same NF service instance of the provider for the next service operation for the concerned data context. The service operations are faster by sticking to the same NF service instance of the provider and using a local cache, as storing and retrieving the data context between the two service operations is not necessary. When the sequence of service invocations has been completed, the binding can be updated from the NF service instance to the NF Set or NF instance for subsequential service operations.

The short-lived binding can be released, and data context stored, once the sequence of service invocations that use a local cache for the concerned data context has been completed.

A long-lived binding between the NF service consumer and producer is used to avoid service discovery and selection for the subsequential service operations concering the data context.

When the binding is established, it is set either with the NF instance or the NF Set; the NF service consumer or SCP should select all concerned NF service instances from this NF instance or NF Set, respectively. The binding may additionally be set either with NF service instance or NF Service Set, as described in this clause.

It is up to the NF service producer to establish and update the binding. The binding is released implicity when the concerned context is deleted, e.g. when the UE context is deleted as part of PDU session release procedure.

Direct Communication or Indirect Communication procedures may be used between the Consumer and Producer. In case of Indirect Communication, an SCP is located between the Consumer and Producer. The NF service consumer is able to maintain the binding to the NF service producer by using the resource address and binding indicator included in the service request or response, so the SCP does not need to store the binding indicator.

#### 4.17.11.2 Binding created as part of service response

When the NF service consumer communicates with the NF service producer, the producer may return a binding indicator to the consumer. The consumer stores the received binding indicator and uses it for the subsequential requests concerning the data context.

Figure 4.17.11.2-1: Binding created as part of service response

1. 	If Direct Communication is used, the NF service consumer selects the NF service producer and sends the request to the selected NF service producer. If Indirect Communication mode is used the NF service consumer sends the request to the SCP and provides within the service request to the SCP the discovery and selection parameters necessary to discover and select a NF cervice producer.

2. 	The NF service producer sends a response to the NF service consumer. In the response the NF cervice producer may include a a binding indicator. If the NF service consumer receives a binding indicator as specified in Table 6.3.1.0-1 in TS 23.501 [2], it uses it for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

3.	The NF Service Consumer uses the binding indicator received in the previous step for subsequent requests regarding the concerned resource.

4. 	The NF service producer sends a response to the consumer. The NF service producer may respond with an updated resource address and updated binding indicator different to the one received in the previous response.

#### 4.17.11.3 Binding created as part of service request

Binding can be created as part of the implicit or explicit subscription request. In this case, the subscription request or the service request that creates the implicit subscription may include a binding indicator. In addition, if the NF service consumer can also be as a NF service producer for later communication from the contacted producer, a service request sent to  the producer may include binding indicator

Figure 4.17.11.3-1: Binding created as part of service request

1. 	Instance A, as an NF service consumer sends a service request using either Direct Communication or Indirect communication via SCP and Instance B is selected as NF service producer. If the request creates a subscription, or if Instance A, can also be NF service producer for later communication for the concerned data context, it may include binding  indicator as specified in Table 6.3.1.0-1 in TS 23.501 [2] in the request sent to the NF service producer.

2. 	Instance B as the NF service producer sends a response to the NF service consumer.

3.	When the producer sends a notification or when Instance B as NF service consumer needs to invoke the service provided by Instance A, Instance B sends a request with the binding indicator received in step 1 as described in Steps 3-4 in Figure 4.17.11.2-1.

#### 4.17.11.4 Binding update

The binding indicator can be updated by the NF service producer in-band as part of the service operation request or response, or out-of-band via notification operation.

Figure 4.17.11.4-1: Binding update

Case A)	NF service producer updates the binding indicator in the response to the service request.

Case B)	NF service producer updates the binding indicator out-of-band by notifying the NF service consumer.

Case C)	NF service consumer that can also take a role of NF service producer updates the binding indicator as part of the service request.

Case D)	NF service consumer that can also take a role of NF service producer updates the binding indicator by notifying the NF service consumer.

*** END CHANGES ***